# 3GPP TSG RAN WG4 Meeting#112	R4-2411680

# Maastricht, NL, Aug 19 – Aug 23, 2024

Source: 	Qualcomm Incorporated

Title: 	Discussion on the demodulation performance requirements for 6Rx devices

Agenda item:	8.1.1.3.2

Document for:	Discussion

# 1. Introduction

During the last meeting, RAN4 discussed related issues pertaining to performance requirements for 6Rx devices [1]. In this paper, we share our views on the open issues related to MIMO layer evaluation for the 6Rx devices.

2. General Considerations

6-Layer Performance Evaluation Assumptions

Way Forward: RAN4 to further discuss the 6-Layer MIMO performance evaluation assumptions considering realistic antenna correlation assumptions and deployment scenarios.

2.1 Assumptions for Performance Evaluations

During RAN#111, assumptions for performance evaluation, mainly the nature of correlation between receive antennas were discussed. The way forward is copied above.

2.1.1 Correlation Assumptions

Figure 1: Test phones with up to eight receive antennas

One of the important considerations for 6-layer performance is the nature of antenna correlation that the device may experience in real deployment scenarios. Therefore, we think it is reasonable to assume some realistic correlation model for the purpose of performance requirements. To this end, we have conducted a field experiment as shown in Fig. 1, where the test device is a fully equipped with camera, sensors, and mmW frontend facing the same space limitation and technical challenges as a commercial device. From this setup, we have measured the envelope correlation between the pair of receive antenna for a handheld device. It should be noted that envelope correlation is a useful measure to show the correlation between the radiation patterns of two different antennas [2].

Based on the measured envelope correlation values from the field tests, it is found that between any pair of antennas for a test device that has been optimized for antenna placing and decoupling, the envelope correlation coefficients have been upper bounded by 0.3. It is also observed that most of the correlation coefficients between any pair of antennas are below 0.1. Such observations suggests that low correlation values in frequency bands above 3.5 GHz can be achieved even for non-foldable handheld devices.

Observation 1: For a handheld test device that has been optimized for antenna placing and decoupling, the correlation coefficients between pair of antennas is upper bounded by 0.3.

Observation 2: Correlation coefficients between any pair of receive antennas are mostly below 0.1 for a handheld device.

Furthermore, it is reasonable to expect that adding 6Rx support in a new commercial UE platform could be easier than modifying an existing test device. Therefore, the correlation values measured in the test device can be seen as conservative.

Observation 3: Commercial device may see even lower correlation values between pairs of receive antennas.

Proposal 1: Consider correlation values with correlation coefficient between pairs of antennas to be less than 0.1 for a handheld device.

3. 6-layer Support

Agreement:

-	Decision on support of 6 MIMO layers for handheld is deferred until performance level differences in comparison with 4 MIMO layers are known based on evaluations considering realistic antenna correlation assumptions and deployment scenarios

During the last meeting, RAN4 discussed the feasibility of introducing 6-layer performance requirements for 6Rx devices [1]. The agreement is presented below. In this section, we discuss field and simulation results for 6Rx handheld device showing the feasibility of 6-layer support. To this end, we provide results from field tests obtained using test devices. Having access to the measured correlation values from the test devices, we then provide several sets of physical layer simulation results considering different DMRS and antenna configurations. More details are provided below.

3.1 Field Tests

3.1.1 Test Set up

To understand the achievable gains for a 6Rx/6-layer configuration in field deployment scenarios, we conduct an experiment with test devices having 8 receive antennas. As it is shown in Fig. 1, the test device has a regular form factor of a non-foldable handheld device and has been optimized considering antenna placing and decoupling for operation in 3.5GHz band. The target bandwidth of the test device is 3.3 – 3.8 GHz. Beside comparing the performance across different layers of configurations, a secondary goal is to understand the achievable correlation values between the pairs of receive antennas.

3.1.2 Field Test Results

The measurement campaign was carried under a commercial network in several regions of a cell. Typical inter-site distances in the field test were between 200 and 500 ms. CSI-RS was configured with 8 Tx ports, hence also PDSCH transmission happened with 8 Tx ports. Since 6 layers are not supported in the commercial network, CSI-RS measurements were extracted from the log files and spectral efficiency was analysed offline. For the offline analysis of the CSI-RS based spectral efficiencies a linear MMSE-IRC receiver has been applied. Hence, the results can be seen as conservative compared to a non-linear reduced ML receiver that commercial devices may implement.

Figure 2: Projected throughput gains by six receive antennas

Figure 3: Average number of layers across SSB RSRP.

For the field tests, the spectral efficiencies based on CSI-RS measurements have been evaluated in those three categories for a test device equipped with four receive antennas and a test device equipped with six receive antennas. Clearly, the device with 4Rx antennas is limited to four MIMO layers. Fig. 2 shows projected throughput gains from field measurements which uses a sub-optimal MMSE receiver and compared with that of an advanced receiver, such as R-ML performance.

Observation 4: Six receive antennas provide very large throughput gain for SSB RSRP > -80 dBm and -80 dBm<SSB RSRP<-100 dBm in the order of 35% and 45%, respectively.

Observation 5: An advanced receiver, such as R-ML receiver can offer noticeable performance improvement over an MMSE receiver for 6Rx devices.

Fig. 3 shows the achievable number of layers scheduled for both 6Rx and 4Rx devices across measured SSB RSRP values. In particular, the observed rank for SSB RSRP > -80 dBm and -80 dBm<SSB RSRP<-100 dBm has been increased from four to five even with MMSE receiver and can achieve six-layer transmission with a R-ML receiver.

Observation 6: The following observations can be made for achievable average number of layers:

- Six receive antennas can achieve 5-layer transmission for SSB RSRP > -80 dBm and -80 dBm<SSB RSRP<-100 dBm with sub-optimal MMSE processing.

- Six receive antennas can achieve 6-layer transmission with a R-ML receiver when SSB RSRP > -80 dBm.

Th above observations suggest that more than four MIMO layers can be achieved in a handheld device, even without needing the form factor of a foldable device. For a foldable device, it is expected that larger gains will be observed since it may be possible to further reduce correlations between receive antenna pairs.

3.2 Physical Layer Simulations

In this section, we discuss the physical layer simulation results considering the impact of DMRS overhead, receive antenna correlation, number of transmit antennas.

3.2.1 Correlation Assumptions

We assume that the correlation within the primary four receive antennas (in a 6Rx device) are assigned the least pairwise correlation from the measured values. We name this group as primary group. It is also assumed that a device with four receive antenna will have the same correlation as the primary group of a 6Rx device. We further consider a secondary group, which only contains the remaining two receive antennas. Following this, we construct a reference correlation matrix considering lowest correlation within the secondary group (i.e., two remaining receive antennas) and across primary and secondary groups. Additionally, we consider a pessimistic correlation matrix, where the maximum correlation (i.e., worst-case) correlation among the 5th and 6th receive antennas to other antenna pairs are considered based on the measured correlation values from the field tests. To this end, the non-diagonal elements of 5th and 6th columns and rows of the 6x6 correlation matrix are all assumed to be same with a correlation coefficient of 0.25. We use these pessimistic correlation matrices in our physical layer simulations.

3.2.2 Simulation Assumptions

We consider the following assumptions in our physical layer simulations. We consider a 4-layer configuration with six receive antennas and eight transmit antennas as a reference scenario. As a performance metric, we consider relative throughput for the 5-layer and 6-layer scenarios with respect to the reference scenario at each SNR value, where a positive relative throughput means that 5-layer and 6-layer scenarios provide a gain over that of the 4-layer scenario.

Table 1: Physical Layer Simulation Assumptions

| SCS/BW | 30KHz, 100MHz |
| --- | --- |
| No. of layers | Fixed {4L, 5L, 6L} |
| MCS | Adaptive MCS |
| Channel model | TDLA 30-10 |
| DMRS configurations | {1+1, 2+2, 2+0} |
| Number of transmit antennas | {8, 16} |
| Number of receive antennas | 6 |

3.2.3 Physical Layer Simulation Results

First, we study the impact of DMRS overhead on the throughput performance via link-level simulations. For this purpose, we consider three different Type 1 DMRS configurations (Rel-15). The reference scenario (configuration 1) for this study is one frontloaded DMRS and one additional DMRS (addPos1). Such a configuration cannot support more than 4 layers. Therefore, we consider a second configuration (configuration 2) with two frontloaded DMRS and two additional consecutive DMRS symbols (addPos1) for more than 4-layer scenario. We also consider a third configuration (configuration 3) for more than 4-layer scenario with only two frontloaded DMRS symbols, i.e., without any additional DMRS symbols (i.e., addPos0). It should be noted that unlike configuration 2, configuration 3 offers the same DMRS overhead as in configuration 1. We assumed the same reference correlation matrix as described in the previous section.

Table 2: Throughput performance relative to 6Rx/4L 1+1 DMRS (addPos1)

| RX/Layer | DMRS Configuration | SNR (dB) | Relative Throughput Gains (%) |
| --- | --- | --- | --- |
| 6RX/5L | 2+0 | 20 | 0.6 |
|  |  | 32 | 8.1 |
|  |  | 36 | 22.5 |
|  |  | 42 | 24.7 |
|  | 2+2 | 20 | -17.2 |
|  |  | 32 | -8.4 |
|  |  | 36 | -1.8 |
|  |  | 42 | 1.8 |
| 6RX/6L | 2+0 | 20 | -15.3 |
|  |  | 32 | -2.2 |
|  |  | 36 | 9.6 |
|  |  | 42 | 25.1 |
|  | 2+2 | 20 | -30.1 |
|  |  | 32 | -19.3 |
|  |  | 36 | -3.9 |
|  |  | 42 | 15.8 |

As shown in Table 2, we see that configuring four DMRS symbols per slot (2+2 DMRS, configuration 2) has large overhead and yields to a performance degradation for 6-layer scenario.

Observation 7: Configuration four DMRS symbols per slot (2+2 DMRS case) yields to a performance degradation and should be avoided for 6-layer scenario.

Hence, the reference scenario with 6Rx/4-layer (configuration 1) should be compared with configuration 3 as these two configurations have same DMRS overhead. From Table 2 with configuration 3, it can be observed that 6Rx/5-layer achieves consistently higher throughput (indicated by a positive relative throughput) than the reference scenario (i.e., with 6Rx/4-layer) at around SNR = 30 dB with the reference correlation matrix. On the other hand, 6Rx/6-layer configuration outperforms that of 6Rx/4-layer at around SNR = 33 dB and that of 6Rx/5-layer at around SNR = 42 dB. These observations are in line with the results obtained from the field measurements.

Observation 8: Comparing between configuration 1 (i.e., 6Rx/4layer with 1+1 DMRS) and configuration 3, 6Rx/6-layer outperforms 6Rx/4-layer at around SNR = 33 dB and 6Rx/5-layer at around SNR = 42 dB.

Observation 9: Comparing between configurations 1 and 3, 6Rx/5-layer starts to consistently outperform 6Rx/4-layer at around SNR = 20 dB.

We have further simulated with a pessimistic correlation matrix, where the non-diagonal elements of 5th and 6th columns and rows of the 6x6 correlation matrix are all assumed to be same, i.e., with a correlation coefficient of β=0.25 as shown below:

Pessimistic correlation matrix:11ββββ11ββββββββββββ1ββ1

Table 3: Throughput performance relative to 6Rx/4L 1+1 DMRS (addPos1) with a pessimistic correlation matrix

| RX/Layer | β | SNR (dB) | Relative Throughput Gains (%) |
| --- | --- | --- | --- |
| 6RX/5L | 0.25 | 20 | -3.8 |
|  |  | 32 | 4.8 |
|  |  | 36 | 20.8 |
|  |  | 42 | 24.7 |
|  | Reference correlation matrix | 20 | 0.6 |
|  |  | 32 | 8.1 |
|  |  | 36 | 22.5 |
|  |  | 42 | 24.7 |
| 6RX/6L | 0.25 | 20 | -18.1 |
|  |  | 32 | -4.5 |
|  |  | 36 | 6.6 |
|  |  | 42 | 23.5 |
|  | Reference correlation matrix | 20 | -15.3 |
|  |  | 32 | -2.2 |
|  |  | 36 | 9.6 |
|  |  | 42 | 25.1 |

The corresponding results are presented and compared with the reference correlation matrix in Table 3. The following observations can be made:

Observation 10: The performance impact is less than 1 dB by having all elements of both 5th and 6th columns and rows of the 6x6 correlation matrix to be 0.25.

Observation 11: The performance of 5-layer and 6-layer configurations with six receive antennas is somewhat less sensitive to β value up to 0.25.

Table 4: Throughput performance relative to 8Tx 6Rx/4L 1+1 DMRS (addPos1) with 16 transmit antennas

| RX/Layer | Number of transmit antennas | SNR (dB) | Relative Throughput Gains (%) |
| --- | --- | --- | --- |
| 6RX/5L | 8 | 20 | -3.8 |
|  |  | 32 | 4.8 |
|  |  | 36 | 20.8 |
|  |  | 42 | 24.7 |
|  | 16 | 20 | 9.9 |
|  |  | 32 | 14.6 |
|  |  | 36 | 24.0 |
|  |  | 42 | 24.7 |
| 6RX/6L | 8 | 20 | -18.1 |
|  |  | 32 | -4.5 |
|  |  | 36 | 6.6 |
|  |  | 42 | 23.5 |
|  | 16 | 20 | -5.4 |
|  |  | 32 | 3.0 |
|  |  | 36 | 19.7 |
|  |  | 42 | 35.5 |

Additionally, we studied the impact of number of transmit antennas on the performance of 6Rx device. Based on the results provided in Table 4 with the pessimistic correlation matrix, it is observed that there is a significant performance improvement across the entire SNR range compared to same configurations with 8Tx antenna ports, i.e., reference scenario with 4Rx/4-layer. This shows that additional transmit antenna ports can significantly increase the benefits of 6Rx, and particularly, more gains are observed for 6Rx/6-layer case at high SNR regime.

Observation 12: Compared to 8 transmit antennas, a configuration of 16 transmit antennas can offer significant gain (up of 40% relative throughput improvement) for 6Rx/6-layer. It is expected that configuring 32 transmit antennas will avail even more gain.

To study the impact of correlation values further, additional simulations have been carried out for a correlation matrix that has the same correlation coefficient in all non-diagonal entries. For a 6Rx device, the 6x6 correlation matrix with correlation coefficients ρ∈0.05, 0.2, 0.4, 0.5 across all non-diagonal entries is provided below

Symmetric correlation matrix: 1ρρ1ρρρρρρρρρρρρ1ρρ1ρρρρρρρρρρρρ1ρρ1, ρ∈0.05, 0.2, 0.4, 0.5

Table 5: Performance results with symmetric correlation matrices

| RX/L | ρ | SNR (dB) | Relative Throughput Gains (%) |
| --- | --- | --- | --- |
| 6RX/5L | 0.1 | 20 | -1.1 |
|  |  | 32 | -0.3 |
|  |  | 36 | -0.4 |
|  |  | 42 | 0.0 |
|  | 0.2 | 20 | -3.3 |
|  |  | 32 | -2.0 |
|  |  | 36 | -0.5 |
|  |  | 42 | 0.0 |
|  | 0.4 | 20 | -10.8 |
|  |  | 32 | -6.9 |
|  |  | 36 | -3.3 |
|  |  | 42 | 0.0 |
|  | 0.5 | 20 | -15.0 |
|  |  | 32 | -9.6 |
|  |  | 36 | -5.3 |
|  |  | 42 | -0.1 |
| 6RX/6L | 0.1 | 20 | -0.3 |
|  |  | 32 | -0.4 |
|  |  | 36 | -0.5 |
|  |  | 42 | -0.4 |
|  | 0.2 | 20 | -2.3 |
|  |  | 32 | -1.5 |
|  |  | 36 | -1.7 |
|  |  | 42 | -0.4 |
|  | 0.4 | 20 | -8.3 |
|  |  | 32 | -4.8 |
|  |  | 36 | -4.6 |
|  |  | 42 | -2.6 |
|  | 0.5 | 20 | -13.0 |
|  |  | 32 | -7.3 |
|  |  | 36 | -6.2 |
|  |  | 42 | -4.4 |

Table 5 shows the relative throughput with respect to a reference scenario (6Rx/4layer, 8Tx antenna ports and symmetric correlation matrix with ρ=0.05). It is seen from Table 5 that for ρ=0.2 the losses are negligible, and a graceful degradation is observed for ρ=0.4 and ρ=0.5.

Observation 13: Negligible losses are observed for a symmetric correlation matrix having correlation coefficient up to 0.2 across all non-diagonal elements.

3.2.3 Summary of Simulation Results

In this section, we have shared results from field tests, where a test device, which has been optimized for antenna placement and decoupling, is used. It is observed that a handheld device with six receive antennas can offer a large diversity gain compared to device with four receive antennas. Even such a device can achieve 5-layer transmission in cell center with a good multiplexing gain.

Additionally, we have carried out physical layer simulations, where we have used the measured correlation values from the field tests. In particular, we have considered DMRS overhead, different correlation types as well as the increased number of transmit antennas in our studies. It is observed that for the same DMRS overhead 6Rx/6-layer configuration outperforms that of 6Rx/4-layer at around SNR = 33 dB and 6Rx/5layer at around SNR = 42 dB. Furthermore, we have evaluated performance with a pessimistic correlation matrix with 5th and 6th columns and rows of 6x6 correlation matrix having correlation coefficient of 0.25. The results show minimal performance impact (e.g., ~1dB performance loss) with respect to a more benign correlation matrix. This observation suggests that 5-layer and 6-layer performances may not be very sensitive to the correlation values.

Besides, we have examined performance with 16 transmit antennas and compared with the baseline configuration of 8 transmit antennas. It has been noticed that a configuration of 16 transmit antennas can offer significant gain (up of 40% relative throughput improvement) over 8 transmits antennas for a 6Rx/6-layer scenario, assuming all other simulation assumptions remain unchanged.

Based on the observations from the field tests and considering the physical layer simulation results with different DMRS configurations, antenna and correlation assumptions, we think that it is feasible to define 6-layer performance requirements for 6Rx devices.

Proposal 2: RAN4 to introduce 6-layer performance requirements for handheld and FWA devices.

- FFS whether to use same requirements for handheld and FWA devices.

3.3 6-layer Feature

6-layer Support as optional feature

Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature.

It was discussed during RAN4#111 whether to introduce 6-layer support as an optional feature. The way forward for this issue is presented below.

We note that 8-layer support for 8Rx devices has been introduced as an optional feature [3]. Therefore, we think that it is also feasible to introduce 6-layer support for 6Rx devices as an optional feature.

Proposal 3: RAN4 to introduce 6-layer support for 6Rx devices as an optional feature.

5. Conclusions

In this paper, we share our views on the performance requirements for 6Rx devices. The following have been proposed.

Observation 1: For a handheld test device that has been optimized for antenna placing and decoupling, the correlation coefficients between pair of antennas is upper bounded by 0.3.

Observation 2: Correlation coefficients between any pair of receive antennas are mostly below 0.1 for a handheld device.

Observation 3: Commercial device may see even lower correlation values between pairs of receive antennas.

Proposal 1: Consider correlation values with correlation coefficient between pairs of antennas to be less than 0.1 for a handheld device.

Observation 4: Six receive antennas provide very large throughput gain for SSB RSRP > -80 dBm and -80 dBm<SSB RSRP<-100 dBm in the order of 35% and 45%, respectively.

Observation 5: An advanced receiver, such as R-ML receiver can offer noticeable performance improvement over an MMSE receiver for 6Rx devices.

Observation 6: The following observations can be made for achievable average number of layers:

- Six receive antennas can achieve 5-layer transmission for SSB RSRP > -80 dBm and -80 dBm<SSB RSRP<-100 dBm with sub-optimal MMSE processing.

- Six receive antennas can achieve 6-layer transmission with a R-ML receiver when SSB RSRP > -80 dBm.

Observation 7: Configuration four DMRS symbols per slot (2+2 DMRS case) yields to a performance degradation and should be avoided for 6-layer scenario.

Observation 8: Comparing between configuration 1 (i.e., 6Rx/4layer with 1+1 DMRS) and configuration 3, 6Rx/6-layer outperforms 6Rx/4-layer at around SNR = 33 dB and 6Rx/5-layer at around SNR = 42 dB.

Observation 9: Comparing between configurations 1 and 3, 6Rx/5-layer starts to consistently outperform 6Rx/4-layer at around SNR = 20 dB.

Observation 10: The performance impact is less than 1 dB by having all elements of both 5th and 6th columns and rows of the 6x6 correlation matrix to be 0.25.

Observation 11: The performance of 5-layer and 6-layer configurations with six receive antennas is somewhat less sensitive to β value up to 0.25.

Observation 12: Compared to 8 transmit antennas, a configuration of 16 transmit antennas can offer significant gain (up of 40% relative throughput improvement) for 6Rx/6-layer. It is expected that configuring 32 transmit antennas will avail even more gain.

Observation 13: Negligible losses are observed for a symmetric correlation matrix having correlation coefficient up to 0.2 across all non-diagonal elements.

Proposal 2: RAN4 to introduce 6-layer performance requirements for handheld and FWA devices.

- FFS whether to use same requirements for handheld and FWA devices.

Proposal 3: RAN4 to introduce 6-layer support for 6Rx devices as an optional feature.

6. References

[1] R4-2410751, “WF on requirements for 6Rx,” 3GPP TSG RAN WG Meeting #111, Fukuoka, JP May 20 – 24, 2024.

[2] X. Mei and K.-L. Wu, “Envelope correlation coefficient for multiple MIMO antennas of mobile terminals,” IEEE Int. Symp. on Antennas and Propagation, Montreal, CAN, July 5-10, 2020

[3] R4-2406030, “Draft CR on 8Rx PDSCH demodulation requirements,” 3GPP TSG RAN WG Meeting #110-bis, Changsha, CN Apr 15 – 19, 2024.