3GPP TSG-RAN WG4 Meeting #112	R4-2414270

# Maastricht, Netherlands, 19th – 23rd August 2024

Agenda item:	7.13.1

Source: 	Nokia

Title: 	WF on introduction of the 1.4 GHz Band

Document for:	Approval

# WF on introduction of the 1.4 GHz Band

The following is proposed for approval:

- Band number: use band number n110.

- System parameters:

  - Channel bandwidths:

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n110 | 15 | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

  - Channel raster:

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n110 | 100 | 278000 – <20> – 279000 | 286400 – <20> – 287000 |

  - Sync raster:

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n110 | 15 kHz | Case A | 33802 – <1> – 33804 |

- UE REFSENS: [-102.2] dBm

- UE Tx-Rx frequency separation: 40 and 42 MHz

- CR responsibility:

| Specification | Proposed company for formal CR |
| --- | --- |
| 38.101-1 | Nokia |
| 38.133 | ZTE |
| 38.106 | Nokia |
| 38.115-1 | Nokia |
| 38.174 | Nokia |
| 38.176-1 | Nokia |
| 38.176-2 | Nokia |
| 36.104 | Nokia |
| 36.141 | Nokia |
| 37.104 | Huawei |
| 37.141 | Huawei |
| 38.104 | Nokia |
| 38.141-1 | Huawei |
| 38.141-2 | Huawei |
| 37.105 | ZTE |
| 37.145-1 | Ericsson |
| 37.145-2 | Ericsson |
| 38.101-5 | ZTE |
| 38.307 | CATT |