# 3GPP TSG-RAN WG4 Meeting #112bis		        R4-2415464

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	5.20.2

Source: 	Huawei, HiSilicon

Title:	On UE RF requirements for band n28

Document for:	Approval

# 1	Introduction

Considerable progress on introducing PC2 and 40MHz CBW for band n28 [1] has been made and captured in the WF [2]. This paper continues to discuss the remaining open issues, namely:

- PC2 A-MPR for NS_17

- BW=40MHz related

- PC3 REFSENS and PC2 RSD

- PC3 and PC2 A-MPR for NS_18

- ∆MPR

- UE coexistence requirements

- Channel location

- Channel raster

- Release independence

# 2	Discussion

Issue 2-2-2  NS_17 A-MPR for PC2 for BW<=30MHz

This issue has been discussed intensively in Rel-18. And it has been agreed [3] that no A-MPR is needed for dual-duplexer implementation.

Observation 1: For NS_17, it has been agreed that no PC2 A-MPR is needed for dual-duplexer implementation.

For full-band duplexer implementation, some minor adjustments to the starting point proposals in [2] are needed based on our evaluation.

Proposal 1: For full-band duplexer implementation, consider the following PC2 A-MPR requirements for NS_17:

Table 1: A-MPR regions for NS_17 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 723 ≤ Fc ≤ 728 | ≤ 0.18 | ≤ 1.44 | A1 |
|  |  | ≥ 0 | >= 5.4 | A2 |

Table 2: A-MPR for NS_17 for PC2

| Modulation/Waveform |  | A1 | A2 |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ [3] | ≤ [3.5] |
|  | QPSK | ≤ [3] | ≤ [4] |
|  | 16 QAM | ≤ [3.5] | ≤ [5] |
|  | 64 QAM | ≤ [4] | ≤ [5] |
|  | 256 QAM |  | ≤ [5.5] |
| CP-OFDM | QPSK | ≤ [5] | ≤ [5.5] |
|  | 16 QAM | ≤ [5] | ≤ [5.5] |
|  | 64 QAM | ≤ [5] | ≤ [5.5] |
|  | 256 QAM |  |  |

As seen from above, UEs with dual-duplexers could transmit higher power than UEs with full-band duplexers. Furthermore, dual-duplexer implementations could provide extra attenuation to UL in adjacent bands, which is crucial for L-L CA/DC such as n18/18+n28/28, n5/5+n28/28.

Proposal 2: Subject to operators’ demand, whether single set or two sets of requirements should be defined can be further discussed by considering both A-MPR and L-L (such as n18+n28) band combination performances.

Issue 2-3-2 n28 REFSENS for PC3

- FFS on PC3 REFSENS based on more companies’ input.

- Following values are proposed in this meeting as starting point.

| Source | 40 MHz (dBm) |
| --- | --- |
| Skyworks(R4-2413062) | -66.3 |
| Qualcomm (R4-2413149) | -65.9 |
| Muruta (R4-2411476) | -67.1 for 15KHz -67.2 for 30KHz |
| Average | -66.4 |

We have performed measurements on the PA output noise on the Rx frequencies. Based on our evaluation on the potential impact on the Rx sensitivity, we think the proposed average value of -66.4 dBm is reasonable.

Proposal 3: Take [-66.4] dBm as the REFSENS for 40MHz.

Issue 2-3-3 PC2 RSD for 1Tx and 2Tx for 40MHz

- FFS on PC2 RSD for 40MHz

In the last meeting, the following values were proposed:

| Source | RSD 1Tx (dB) | RSD 2Tx (dB) |
| --- | --- | --- |
| Skyworks (R4-2413062) | 4 | 8.5 |
| Muruta (R4-2411476) | 2.8 | 8.8 |

Based on our evaluation, we propose the following values to be considered.

Proposal 4: Consider the following values as PC2 RSD: [3.0] dB for 1Tx and [8.2] dB for 2Tx.

Issue 2-3-6 NS_18 for 40MHz

Simulations for both PC3 and PC2 are conducted, and the results are recorded in the appendix.

Proposal 5: For PC3 and PC2 A-MPR for BW=40MHz NS_18, consider the simulation results provided in the appendix.

Issue 2-3-8 channel location

- FFS on how to capture the UL channel location for UE supporting maximum bandwidth of 40MHz in n28.

Since 40MHz BW is optional and the applicable frequency range becomes larger, a new note on the channel location is needed. To simplify the description, we suggest to use “channel bandwidth” instead of “carrier frequencies” or “transmission bandwidth configuration”. Additionally, an extra 40kHz should be included if the exceptional channel raster point is added for the UEs.

Proposal 6: To simply the description, modify the existing NOTE 7 for band n28 as follows:

NOTE 7:	For UEs supporting 30MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to either 703-733MHz or 718-748MHz for the 20, 25 and 30MHz bandwidth.

Proposal 7: For UEs supporting 40MHz, add a new note as follows:

NOTE x:	For UEs supporting 40MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to either 703-743MHz or 718-748MHz for the 30MHz bandwidth. And for the 40MHz bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to either 703-743.04MHz.

Issue 2-3-9 channel raster

- The following options can be considered for n28 40MHz

  - Add the exceptional channel raster point of n28 (UL: 723.04MHz, DL: 778.04MHz) to UE RF specification TS 38.101-1 for UE CBW 40MHz.

  - UE supporting 40 MHz channel bandwidth in band n28 shall support Enhanced channel raster.

The exceptional channel raster point has been made available for the network from Rel-16. It enables a 30MHz UE to access a 40MHz network. For a 40MHz UE to access such a network, the UE needs to support the same channel raster point.

For the time being, it seems that there’re only two valid channel raster points (i.e. UL: 723MHz or 723.04MHz) for deploying a 40MHz network on band n28, given the status of world-wide spectrum allocations. Hence, it’s unnecessary to mandatorily require the UE to support enhanced channel raster for the sake of 40MHz.

Proposal 8: Add the exceptional channel raster point of n28 (UL: 723.04MHz, DL: 778.04MHz) to UE RF specification TS 38.101-1 for UE CBW 40MHz.

Issue 2-3-10 Release independence

- FFS on the release independence of 40MHz for n28

  - Option 1: Rel-15

  - Option 2: Rel-16

The product development for Rel-15 is already stable. If there’s no urgent deployment need, we prefer the earliest release to be Rel-16. Subject to further discussions, we also need to check if new signalling is needed or not.

Proposal 9: Prefer to release independence from Rel-16 if no new signalling is introduced.

# 3	Conclusion

In this paper, we continue to discuss the remaining open issues for the Rel-19 WI on band n28, and make the following proposals:

Observation 1: For NS_17, it has been agreed that no PC2 A-MPR is needed for dual-duplexer implementation.

Proposal 1: For full-band duplexer implementation, consider the following PC2 A-MPR requirements for NS_17:

Table 1: A-MPR regions for NS_17 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 723 ≤ Fc ≤ 728 | ≤ 0.18 | ≤ 1.44 | A1 |
|  |  | ≥ 0 | >= 5.4 | A2 |

Table 2: A-MPR for NS_17 for PC2

| Modulation/Waveform |  | A1 | A2 |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ [3] | ≤ [3.5] |
|  | QPSK | ≤ [3] | ≤ [4] |
|  | 16 QAM | ≤ [3.5] | ≤ [5] |
|  | 64 QAM | ≤ [4] | ≤ [5] |
|  | 256 QAM |  | ≤ [5.5] |
| CP-OFDM | QPSK | ≤ [5] | ≤ [5.5] |
|  | 16 QAM | ≤ [5] | ≤ [5.5] |
|  | 64 QAM | ≤ [5] | ≤ [5.5] |
|  | 256 QAM |  |  |

Proposal 2: Subject to operators’ demand, whether single set or two sets of requirements should be defined can be further discussed by considering both A-MPR and L-L (such as n18+n28) band combination performances.

Proposal 3: Take [-66.4] dBm as the REFSENS for 40MHz.

Proposal 4: Consider the following values as PC2 RSD: [3.0] dB for 1Tx and [8.4] dB for 2Tx.

Proposal 5: For PC3 and PC2 A-MPR for BW=40MHz NS_18, consider the simulation results provided in the appendix.

Proposal 6: To simply the description, modify the existing NOTE 7 for band n28 as follows:

NOTE 7:	For UEs supporting 30MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to either 703-733MHz or 718-748MHz for the 20, 25 and 30MHz bandwidth.

Proposal 7: For UEs supporting 40MHz, add a new note as follows:

NOTE x:	For UEs supporting 40MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to either 703-743MHz or 718-748MHz for the 30MHz bandwidth. And for the 40MHz bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to either 703-743.04MHz.

Proposal 8: Add the exceptional channel raster point of n28 (UL: 723.04MHz, DL: 778.04MHz) to UE RF specification TS 38.101-1 for UE CBW 40MHz.

Proposal 9: Prefer to release independence from Rel-16 if no new signalling is introduced.

# References

- RP-241660 New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, CMCC, CBN

- R4-2414274 WF on introduction of PC2 and 40MHz CBW in NR band n28, CMCC, CBN, China Broadnet, RAN4#112

- R4-2321715 WF on HPUE for FDD bands, China Unicom, RAN4#109

- R4-2413027 On Rel-19 work for band n28, Huawei, HiSilicon, RAN4#112

# Appendix: Simulation Results for NS_18 40MHz

Table 1 below shows the A-MPR simulation results for NS_18 for both PC3 and PC2 based on 1Tx. Note that in the figures, 0dB is assigned if A-MPR ≤ MPR.

Table 1: A-MPR simulation results for NS_18

|  | PC3 A-MPR | PC2 A-MPR |
| --- | --- | --- |
| QPSK |  |  |
| 16QAM |  |  |