3GPP TSG-RAN WG4 Meeting #114	R4-2500038

Athens, Greece, 17th Feb 2025 – 21st Feb 2025

Source:	AT&T, FirstNet, Skyworks Solutions, Inc., Murata Manufacturing Co Ltd., Qualcomm

Title:	TP for TR 38.750 Addition of n14 PC2 UL for CA_n14-n77

Agenda item:			6.8.4

Document for:	Approval

# 1. Introduction

A text proposal for TR 38.850 to add n14 PC2 UL for CA_n14-n77.

# 2.	Discussion

For n14 PC2 UL for CA_n14-n77, uplink harmonic MSD analyses were provided in [1], [2], and [3].

The following is a summary of the proposals from the companies.

Table 2-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD (NOTE 7) | MSD (NOTE 8) | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n14 | n77 | 5 | 15 | 5 | 10 | 12.9 [1] 13.4 [2] 13.0 [3] | 17.3 [1] 16.4 [2] 15.6 [3] | NOTE 5 | UL5/DL1 direct-hit |
| n14 | n77 | 5 | 15 | 5 | 100 | 4.3 [1] 4.5 [2] 4.4 [3] | 5.9 [1] 5.6 [2] 5.6 [3] | NOTE 5 | UL5/DL1 direct-hit |
| NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 7: Applicable to UEs supporting PC2 with 1Tx. NOTE 8: Applicable to UEs supporting PC2 with 2Tx. |  |  |  |  |  |  |  |  |  |

# 3. Text Proposal

---Start of changes---

## 5.X	CA_n14-n77

### 5.X.1	UE maximum output power

Table 5.X.1-1: NR CA configurations and bandwidth combinations
sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n14A-n77A | n148 n778, 9 CA_n14A-n77A8 | n14 | 5, 10 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

NOTE 3:   For each channel bandwidth of each component carrier, refer to Table 5.3.5-1 for the applicable SCSs. For a given band, not all UE channel bandwidths support the same SCSs.

NOTE 8:   Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

NOTE 10: 	Only single uplink carriers with power class other than PC3 are listed.

### 5.X.2	Reference sensitivity requirements

#### 5.X.2.1	General

For PC3, CA_n14-n77 has harmonic MSD for UL n14. This section will examine the existing PC3 MSD and propose MSD for PC2 FDD based on the average MSD values proposed in the analyses provided by companies.

#### 5.X.2.2	Reference sensitivity requirements with PC2 on n14

For CA_n14-n77, this is the configuration and MSD for UL n14 with PC3.

Table 5.X.2.2-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n14 | n77 | 5 | 15 | 5 | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n14 | n77 | 5 | 15 | 5 | 100 | 2.9 | NOTE 5 | UL5/DL1 direct-hit |
| NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Using the averaging approach with the MSD values proposed by companies, the following is proposed for the MSD for UL n14 with PC2:

Table 5.X.2.2-2: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD (NOTE 7) | MSD (NOTE 8) | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n14 | n77 | 5 | 15 | 5 | 10 | 13.1 | 16.5 | NOTE 5 | UL5/DL1 direct-hit |
| n14 | n77 | 5 | 15 | 5 | 100 | 4.4 | 5.7 | NOTE 5 | UL5/DL1 direct-hit |
| NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 7: Applicable to UEs supporting PC2 with 1Tx. NOTE 8: Applicable to UEs supporting PC2 with 2Tx. |  |  |  |  |  |  |  |  |  |

### 5.X.3	∆TIB and ∆RIB values (Void)

---End of changes---

# 4. References

[1] R4-2500222: CA_n14A-n77A UL n14A PC2; Murata Manufacturing Co Ltd.

[2] R4-2501989: HPUE MSD for CA_n2A-n77A and CA_n14A-n77A; Skyworks Solutions Inc.

[3] R4-2501575: CA_n2A-n77A and CA_n14A-n77A PC2 MSD; Qualcomm France