3GPP TSG-RAN WG4 Meeting #114							    	     		R4-2500090

Athens, GR, 17th – 21st February 2025

Title: 	Candidate TN PA for NR-NTN HPUE in smartphones

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.10.2.2.1 Tx requirements	 [NR_IoT_NTN_req_test_enh-Core]

Document for:	Approval

# 1	Background

In RAN#113, we discussed [1] the options for NTN HPUE in smartphones and how to enable the optimum reuse of the smartphone TN hardware. In the meeting, the first coexistence results showed that relaxed ACLR might be feasible, and some options were captured as potential way forwards [2]. In this contribution, we further discuss which target TN PA should be used to develop the NR-NTN HPUE requirement for smartphones and explore options for increased power, a similar contribution [3] focuses on IoT-NTN HPUE.

# Discussion

## 2.1 WF inputs

The way forward [2] provides a number of inputs on relaxed ACLR and power class targets for NR-NTN HPUE.

Issue 1-8 to Issue 1-10: Required ACIR/ACLR for from co-ex study

Agreement: Based on the results collected till RAN4#113, observations as following for the required ACLR from co-existence study evaluation perspective:

For NR-NTN PC2:

- Less than 22 dB with 1.5km isolation distance assumed (baseline assumption)

- [26.5 dB] with 0km isolation distance assumed.

  - Interested companies are encouraged to provide results on this case

For NR-NTN PC1.5:

- Less than 22.9 dB with 1.5km isolation distance assumed (baseline assumption)

- [30.2] dB with 0km isolation distance assumed.

  - Interested companies are encouraged to provide results on this case

For NR-NTN PC1:

- Less than 23.7 dB with 1.5km isolation distance assumed (baseline assumption)

- [28.3/32.7] dB with 0km isolation distance assumed.

  - Interested companies are encouraged to provide results on this case

Issue 2-1: MOP: Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance for PC2 for NR-NTN Non-Redcap HPUE

Issue 2-2: MPR/A-MPR; MPR for NTN HPUE should be further evaluated according to the agreed ACLR value, PA model and other limiting factor for MPR evaluation.

Issue 2-3: SEM

Way forward:

- Follow previous agreement that re-use existing SEM from PC3 to other PCs as starting point.

- Further discuss if SEM can be relaxed for the best power gains for NTN with co-ex study outcomes.

Issue 2-5: NR-NTN PC2 UE

Proposals

  - Option 1: In Rel-19 RAN WG4 can focus on 26dBm 1TX (PC2) for handheld devices. (P1a/R4-2417747)

  - Option 2: For handheld UE, 1Tx and 2Tx are covered with existing TN 26dBm and 23dBm PAs respectively. (P1/R4-2419521)

Issue 2-6: NR-NTN PC1.5 UE

- Proposals

| PC1.5 UE | 1Tx | 2Tx |
| --- | --- | --- |
| Handheld | Not to specify: P1b/R4-2417747, P2/r4-2419072, P1/R4-2419015, P1/R4-2419521 | To specify: P1/R4-2417814, P5/R4-2418200, P1/R4-2419521, P1/R4-2419532 Not to specify: P1b/R4-2417747, P2/r4-2419072, P1/R4-2419015 |
| Non-handheld | Not to specify: P1/R4-2419668, P3/R4-2419072 FFS: P5/R4-2418200, P1/R4-2419521 | To specify: P1/R4-2417814, P5/R4-2418200, P1/R4-2419521, P1/R4-2419532, P2/R4-2419521 Not to specify: P1/R4-2419668, P3/R4-2419072, P1b/R4-2417747 |

Issue 2-7: NR-NTN PC1 UE

- Proposals

| PC1 | 1Tx | 2Tx | 3Tx | 4Tx |
| --- | --- | --- | --- | --- |
| Handheld | Not to specify: P1/R4-2419521 | Not to specify: P5/R4-2418200, P1/R4-2419521 | To specify: P1/R4-2419521 Not to specify: P2/R4-2419668 FFS: P5/R4-2418200 |  |
| Only non-handheld: P2/R4-2419015, P1/R4-2419072 | To specify: P1a/R4-2417747, P2/R4-2419521 | Not to specify: P5/R4-2418200 | To specify: P2/R4-2419521 Not to specify: P2/R4-2419668 FFS: P5/R4-2418200 | To specify: P4/R4-2419072 |

Issue 3-1-1: General considerations

- Proposals

  - Option 1: For power class 2, there is no differentiation of handheld and non-handheld UE. (P2b/R4-2418201)

  - Option 2: The uplink configuration for reference sensitivity normally doesn’t depend on the power class, so for PC2 NR NTN UE, the REFSENs test can adopt the same UL configuration as for PC3 NR NTN UE. (P2/R4-2419717)

## 2.2 Observation on ACLR, SEM and Occupied BW

Looking at the first coexistence simulation results, and assuming the 1.5km isolation distance, significant ACLR relaxation seem achievable versus TN:

- 22/22.9/23.7 for PC2/1.5/1 respectively for NR-NTN

- 21.1/21.9dB for PC2/PC1 respectively for IoT-NTN

These ACLR are significantly lower than for TN with >6dB relaxation. However, it is unclear whether such low ACLR are useful to define the linearity requirement for the PA as other requirements may anyhow limit the power boosting possibility compared to TN. Especially SEM and occupied BW may be limiting the power compared to ACLR.

Occupied BW:

- NR has an occupied BW requirement of 99% in 38.101-1 for all channel BW which is reused in 38.101-5. The same applies to LTE.

- For Cat NB, occupied bandwidth is 99% for 200KHz and applies to IoT-NTN.

For a fully allocated channel this occupied bandwidth sets a maximum for the ACLR as the summed power of both adjacent channels should be less than 1%, thus an ACLR of 23dB if the power is the same in each adjacent channel. However, there will still be additional power to be accounted for in alternate channels and in the gaps between the channel and the adjacent channels measurement bandwidth. A safe approach is to target a 24dB ACLR which results in 0.4% energy in each adjacent channel and thus leaves 0.2% to any other out of channel energy. Obviously, this will require further verification with simulations/measurements. The Table 1 below gives some possible ACLR values for different residual power outside the channel and its adjacent channels.

Table 1: occupied bandwidth budget versus ACLR

| ACLR [dB] | 20 | 22.5 |  | 23 | 23.5 | 24 | 25 | 30 | 31 | 37 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| % power in Adjx2 | 2.0% | 1.1% |  | 1.0% | 0.9% | 0.8% | 0.6% | 0.2% | 0.2% | 0.0% |
| rest to 99% | -1.0% | -0.1% |  | 0.0% | 0.1% | 0.2% | 0.4% | 0.8% | 0.8% | 1.0% |
| occupied BW | Fails for full allocation |  |  | min ACLR no margin |  | reasonable values |  | TN/NTN PC3 | TN PC2/1.5 | TN PC1 |

Observation: The 99% occupied bandwidth requirement may limit the achievable ACLR to ~24dB for large allocations.

SEM: If the occupied bandwidth may dominate over ACLR for full allocation, with the current assumption to reuse the NR SEM, the narrow allocations that are favoured for NTN may fail the SEM when located towards the channel edge:

- IMD3 of the allocation and its image at 28dBc may fail the -13dBm/MHz SEM in the adjacent channel region. 1RB at 26dBm and its image at -2dBm may generate IMD3 > to -13dBm especially at higher compression level than TN.

- IMD5 of the allocation and its image at 28dBc may fail the -30dBm/MHz SEM in the second adjacent channel region.

Observation: For narrow RB allocations that are critical for NTN UL, the limitation will come from SEM rather than ACLR.

Narrow allocations (a few RBs) are key to NTN UL given the large transmission distance, it is thus essential to make sure that beyond ACLR relaxation, the power is not limited by SEM by such allocations. There are multiple ways to avoid such limitation and they may be cumulated:

- Relax the SEM requirement (if not mandated by regulation)

- Assume an improved image rejection (improves IMD3 dB/dB, IMD5 2dB/dB).

- Define a new inner region like for FR2 (inner third region) which only has IMD5 falling into the ACLR/SEM region while IMD3 fall in-band as shown in Figure1.

Figure 1: IMD3 and IMD5 of wanted RBs and its image for inner third allocation.

This inner third approach is quite effective as now the energy in the ACLR measurement BW is only related to IMDs >3 and the SEM limit of -13dBm/MHz only applies to IMD5. It should be noted that such allocations have improved A-MPR or provide better band protection as only the IMD5 falls out of band. Such allocations can further benefit from SEM relaxation and/or image rejection improvements.

This means that the largest inner third allocation is: 8/17/26/35RBs for 5/10/15/20MHz CBW respectively. This seems large enough for typical UL configuration for NR-NTN and it may even be possible to target smaller allocations.

Observation: Inner third allocation is of interest for NR-NTN HPUE as their ACLR and SEM is dominated by IMD5 only. Narrower allocations may even be of interest for NR-NTN UL.

Note that this concept can be used for RedCap UEs as their allocation inside the CBW may already meet the inner third criteria.

## 2.3 Applicability of multi-Tx for NR-NTN HPUE in Rel-19

The main goal for NTN-HPUE is to enable allocations of a few RBs to reach the satellite and allow extended services and/or better reach of GSO satellites. In that sense, UL MIMO is of little interest. In the same way TxD may not be effective because the satellite link is mostly LOS and the satellite will not have more antennas than the UE. This means that the only interest of multi-Tx for NTN is beamforming or at least some directivity gain.

Even with integrated antennas of smartphones, phasing of the UL on multiple antennas can modify the radiation pattern and provide gain in some directions. However, it would require feedback from the satellite to optimally tune each antenna to best point at the satellite and track it as it moves, or the UE is manipulated. Currently, the only available code book is the single stream x*90degre phase shift between antennas which may not be good enough and given the latency, it is unclear that proper tuning feedback via the satellite is feasible.

Observation: While UL MIMO and TxD have little to no interest in NR-NTN, multi-Tx may still be of interest to generate some directivity gain by distorting the overall radiation pattern, but it is unlikely that optimum code books and associated feedback are in scope of Release 19.

## 2.4 Target TN PA for smartphone and potential power increase and limitations

Based on the above discussions, we propose the NR-NTN HPUE work for smartphones to focus on evaluating the TN 1Tx PC2 PAs. These PAs are already in production for a while in TDD mid and high bands and are now targeting FDD mid bands. It should be noted that the same PA is used for all mid bands from 1.6 to 2GHz. For 1Tx PC2 FDD the critical aspect has never been the PA capability, but rather the duplexers requiring higher power handling capability which is solved by slightly larger filters and benefits from improvements in the technology. This slight additional cost must be compared with the need for a second duplexer and PA for a 2Tx approach.

Current NTN bands with their relatively large duplex gaps compared to TN mid-bands are even less of a concern here as their insertion losses are reduced, see the comparison to FDD mid-bands in Table 2.

Table 2: duplex gap comparison between NTN and TN mid-bands

| Band | n1 | n2 | n3 | n25 | n254 | n255 | n256 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Duplex gap [MHz] | 130 | 20 | 20 | 15 | 874 | -101.5 | 190 |

A 1Tx PC2 PA meets 26dBm at the antenna for inner allocations with the following assumptions:

- 1dB MPR for 100RB (outer) 20MHz QPSK DFT-s-OFDM waveform with 31dB ACLR and NR SEM

- 4dB post PA loss

- 28dBc image and carrier rejections

However, for NR-NTN the same PA may reach closer to 29dBm based on the following aspects:

- Around 2dB higher power for large allocations thanks to an ACLR relaxed to 22-24dB

- Potentially lower post PA loss for NR-NTN bands that have less critical duplex gaps

- Improved MPR for inner third allocations (IMD5 limited).

Beyond the power increases related to relaxed linearity and post PA losses, power boosting options already studied for TN using Pi/2 BPSK and QPSK waveforms may be added on top. However, they would need to be re-evaluated based on the set agreed in-band and out of band emissions.

One other possibility for boosting the narrow allocations of interest for NR-NTN is to look at the reduced PAPR versus RBs in Figure 2. For example, for a DFT-s-OFDM QPSK waveform, a 1RB allocation shows about 1dB lower PAPR. However, a 4RB allocation shows similar PAPR than a fully allocated 20MHz channel.

Figure 2: QPSK DFT-s-OFDM PAPR for 1RB (left), 4RB (middle) and 100RBs (right)

As seen above, re-using a PC2 1Tx TN PA may allow to reach closer to PC1.5 output power in multiple ways. One further limitation to be considered though is the DC current capability of the UE for a single PA. This aspect requires further consideration as it may limit the achievable maximum power and if PC1.5 is already specified in TN, it is achieved with two PAs, thus the per PA DC biasing is limited to a PC2 current. The UEs that still support 2G, will allow a slightly higher DC current per PA as the 2G maximum power current is about 30% higher than the PC2 maximum current.

Observations: With relaxed linearity requirements, potentially lower post PA losses and power boosting options linked to waveforms or allocation types, it may be feasible to achieve close to PC1.5 output power with a 1Tx PC2 TN PA. To further evaluate this a complete set of in-band and out of band emissions requirement is needed but also the maximum DC current supported by smartphone UEs needs to be assessed.

## 2.5 Proposals for NR-NTN HPUE

With all the aspects discussed above, we have not tried yet to perform a detailed evaluation as we believe that a complete set of criteria is needed to avoid multiple iterations. With this in mind, we make the following proposal to evaluate 1Tx PC2 FDD PAs for NR-NTN HPUE in smartphone UEs.

Proposal on PCmax and MPR/A-MPR studies for NR-NTN HPUE handheld:

- For smartphones, focus the evaluations on 1Tx PC2 FDD PA and develop associated requirements.

- Evaluate output power capability with following parameters:

  - Occupied BW of 99%, 22-24dB ACLR and NR SEM and spurious emissions

  - 3-4dB post PA losses

- Waveforms: priority for Pi/2 BSPK and QPSK w/wo shaping, 10MHz and 20MHz CBW as a start.

- Evaluate the following allocation types: Current FR1 inner (inner half)/outer and FR2 inner third and 1RB

- Potential limitations due to DC current capability should be considered.

# Conclusion

In this contribution, we discuss the linearity requirement, waveforms and allocation types of interest for the evaluation of 1Tx PC2 TN PA for NR-NTN HPUE. This allowed us the following observations.

Observations:

- The 99% occupied bandwidth requirement may limit the achievable ACLR to ~24dB for large allocations.

- For narrow RB allocations that are critical for NTN UL, the limitation will come from SEM rather than ACLR.

- Inner third allocation is of interest for NR-NTN HPUE as their ACLR and SEM is dominated by IMD5 only. Narrower allocations may even be of interest for NR-NTN UL.

- While UL MIMO and TxD have little to no interest for NR-NTN, multi-Tx may still be of interest to generate some directivity gain by distorting the overall radiation pattern, but it is unlikely that optimum code books and associated feedback are in scope of Release 19.

- With relaxed linearity requirements, potentially lower post PA losses and power boosting options linked to waveforms or allocation types, it may be feasible to achieve close to PC1.5 output power with a 1Tx PC2 TN PA. To further evaluate this a complete set of in-band and out of band emissions requirement is needed, but also the maximum DC current supported by smartphone UEs needs to be assessed.

Based on these observations, the following evaluation is proposed.

Proposal on PCmax and MPR/A-MPR studies for NR-NTN HPUE handheld:

- For smartphones, focus the evaluations on 1Tx PC2 FDD PA and develop associated requirements.

- Evaluate output power capability with following parameters:

  - Occupied BW of 99%, 22-24dB ACLR and NR SEM and spurious emissions

  - 3-4dB post PA losses

- Waveforms: priority for Pi/2 BSPK and QPSK w/wo shaping, 10MHz and 20MHz CBW as a start.

- Evaluate the following allocation types: Current FR1 inner (inner-half)/outer and FR2 inner third and 1RB

- Potential limitations due to DC current capability should be considered.

# References

[1] R4-2419521 PA architectures for handheld NTN HPUE, Skyworks Solutions Inc., RAN4#113

[2] R4-2420403 WF on NR NTN HPUE, Samsung, RAN4#113

[3] R4-250xxx Candidate TN PA reuse for NTN-IoT HPUE in smartphones, Skyworks Solutions Inc., RAN4#114