3GPP TSG-RAN WG4 Meeting #114	R4-2500133

Athens, GR, 17th – 21st February, 2025

Agenda item:	5.9

Source:	MediaTek Inc.

Title:	Discussion on applicability rules for IoT-NTN requirements

Document for:	Discussion

# Introduction

At the RAN4#113 meeting, multiple Tdocs initiated discussions regarding the applicability rules for IoT-NTN requirements, as referenced in [1] and [2]. In this contribution, we share our viewpoints on this matter and propose three alternatives for consideration. Additionally, we have submitted a CR based on alternative 1, as detailed in [2]. The ultimate revision to TS36.102 will be determined by the consensus reached in RAN4.

# Discussion

The applicability rules for IoT-NTN UE are introduced in TS36.102 based on the following table 8.2.1.2-1.

Table 8.2.1.2-1: Requirements applicability for optional UE features

| UE feature/capability | Test list | Applicability notes |
| --- | --- | --- |
| NTN access (ntn-Connectivity-EPC-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1 |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2 |
| NTN scenario support (ntn-ScenarioSupport-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
| Operation in coverage enhancement mode A (ce-ModeA-r13) | Clause 8.2.1.1 (Test 1, Test 2) | The requirements apply only for UE Category M1 |
| Operation in coverage enhancement mode B (ce-ModeB-r13) | Clause 8.2.1.1 (Test 3) | The requirements apply only for UE Category M1 |
| Note: For UE supports NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also applies to UE according to the UE category and capability |  |  |

Take NB1 and NB2 as examples: if ntn-ScenarioSupport-r17 is set to “ngso” or “not included”, Tests 1 and 2 in Clause 8.3.1.1 are applicable. Additionally, according to the Note, the requirements in TS36.101 Clauses 8 and 9 also apply to UEs based on their category and capability, regardless of the NTN scenario support specified in ntn-ScenarioSupport-r17.

Observation 1: Based on the current applicability rules:

- The requirements in TS36.101 and TS36.102 apply to ntn-ScenarioSupport-r17 is “ngso” or “not included”

- The requirements in TS36.101 apply to ntn-ScenarioSupport-r17 is “gso” or “ngso” or “not included”

However, it is possible that an IoT-NTN UE does not support TN bands and cannot be tested with TS36.101. In this case, no requirements apply if ntn-ScenarioSupport-r17 is set to “gso”. Only the requirements in TS36.102 apply if ntn-ScenarioSupport-r17 is set to “ngso” or “not included”.

Table 1. The specification applies to different NTN scenario supports according to the current applicability rules outlined in TS36.102.

| ntn-ScenarioSupport-r17 | gso | ngso | not included |
| --- | --- | --- | --- |
| If UE supports both NTN and TN bands | TS36.102 and TS36.101 | TS36.102 and TS36.101 | TS36.102 and TS36.101 |
| If UE supports only NTN bands | No requirements | TS36.102 | TS36.102 |

Observation 2: It is possible that IoT-NTN UE does not support TN bands

Observation 3: If IoT-NTN UE does not support TN bands:

- No requirements apply to ntn-ScenarioSupport-r17 is “gso”

- Only requirements in TS36.102 apply to ntn-ScenarioSupport-r17 is “ngso” or “not included”

To resolve the situation where no requirements are applicable when ntn-ScenarioSupport-r17 is set to "gso", we suggest three potential alternatives for amending TS36.102.

Proposal 1: To address the issue where no requirements apply when ntn-ScenarioSupport-r17 is set to "gso" and the UE does not support TN bands, we propose three alternatives for modifying TS36.102. RAN4 can review these alternatives and choose one to implement in the revision of TS36.102.

Alternative 1

- Revise the Note to: For UE supports NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also applies to UE under NTN configuration according to the UE category and capability

| UE feature/capability | Test list | Applicability notes |
| --- | --- | --- |
| NTN access (ntn-Connectivity-EPC-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1 |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2 |
| NTN scenario support (ntn-ScenarioSupport-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
| Operation in coverage enhancement mode A (ce-ModeA-r13) | Clause 8.2.1.1 (Test 1, Test 2) | The requirements apply only for UE Category M1 |
| Operation in coverage enhancement mode B (ce-ModeB-r13) | Clause 8.2.1.1 (Test 3) | The requirements apply only for UE Category M1 |
| Note: For UE supports NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also applies to UE under NTN configuration according to the UE category and capability |  |  |

For alternative 1, the requirements in TS36.101 Clauses 8 and 9 can be applied to UEs under NTN configuration (e.g., Ephemeris, NTN bands). This ensures that there is no test coverage gap for ntn-ScenarioSupport-r17 set to “gso” if the UE cannot support TN bands. We have summarized which specifications apply to different NTN scenario supports in Table 2.

Table 2. Specification applies to different ntn scenario support with Alternative 1 changes

| ntn-ScenarioSupport-r17 | gso | ngso | not included |
| --- | --- | --- | --- |
|  | TS36.101 under NTN configuration | TS36.102 and TS36.101 under NTN configuration | TS36.102 and TS36.101 under NTN configuration |

Alternative 2

- Remove the restriction that tests in TS36.102 only apply to ntn-ScenarioSupport-r17 is “ngso” or “not included”.

- Revise the Note to: For UE supports NTN access (ntn-Connectivity-EPC-r17) and TN bands, the requirements in TS36.101 Clause 8 and Clause 9 also applies to UE according to the UE category and capability

| UE feature/capability | Test list | Applicability notes |
| --- | --- | --- |
| NTN access (ntn-Connectivity-EPC-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1 |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2 |
| NTN scenario support (ntn-ScenarioSupport-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
| Operation in coverage enhancement mode A (ce-ModeA-r13) | Clause 8.2.1.1 (Test 1, Test 2) | The requirements apply only for UE Category M1 |
| Operation in coverage enhancement mode B (ce-ModeB-r13) | Clause 8.2.1.1 (Test 3) | The requirements apply only for UE Category M1 |
| Note: For UE supports NTN access (ntn-Connectivity-EPC-r17) and TN bands specified in TS36.101 5.5F, the requirements in TS36.101 Clause 8 and Clause 9 also applies to UE according to the UE category and capability |  |  |

In alternative 2, the requirements outlined in Clauses 8 and 9 of TS36.101 can be applied to IoT-NTN UEs that support TN bands. Furthermore, the requirements specified in TS36.102 are applicable to UEs irrespective of the NTN scenario support indicated by ntn-ScenarioSupport-r17. We have summarized the applicable specifications for different NTN scenario supports in Table 3.

Table 3. Specification applies to different ntn scenario support with Alternative 2 changes

| ntn-ScenarioSupport-r17 | gso | ngso | not included |
| --- | --- | --- | --- |
| If UE supports both NTN and TN bands | TS36.102 and TS36.101 | TS36.102 and TS36.101 | TS36.102 and TS36.101 |
| If UE supports only NTN bands | TS36.102 | TS36.102 | TS36.102 |

Alternative 3

- Remove the restriction that tests in TS36.102 Clause 8 only apply to ntn-ScenarioSupport-r17 is “ngso” or “not included”.

- Remove the Note.

| UE feature/capability | Test list | Applicability notes |
| --- | --- | --- |
| NTN access (ntn-Connectivity-EPC-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1 |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2 |
| NTN scenario support (ntn-ScenarioSupport-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
| Operation in coverage enhancement mode A (ce-ModeA-r13) | Clause 8.2.1.1 (Test 1, Test 2) | The requirements apply only for UE Category M1 |
| Operation in coverage enhancement mode B (ce-ModeB-r13) | Clause 8.2.1.1 (Test 3) | The requirements apply only for UE Category M1 |
| Note: For UE supports NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also applies to UE according to the UE category and capability |  |  |

In alternative 3, NTN UEs are only required to comply with the requirements specified in TS36.102. When ntn-ScenarioSupport-r17 is set to "gso", there will be no test coverage gap, even if the UE does not support TN bands. However, the downside of this alternative is the absence of requirements for PDCCH/PBCH demodulation and CSI reporting. We have summarized the applicable specifications for different NTN scenario supports in Table 4.

Table 4. Specification applies to different ntn scenario support with Alternative 3 changes

| ntn-ScenarioSupport-r17 | gso | ngso | not included |
| --- | --- | --- | --- |
|  | TS36.102 | TS36.102 | TS36.102 |

# Conclusion

In this contribution, we provide our views on applicability rules for IoT-NTN requirements. Observations and proposals are summarized as below.

Observation 1: Based on the current applicability rules:

- The requirements in TS36.101 and TS36.102 apply to ntn-ScenarioSupport-r17 is “ngso” or “not included”

- The requirements in TS36.101 apply to ntn-ScenarioSupport-r17 is “gso” or “ngso” or “not included”

Observation 2: It is possible that IoT-NTN UE does not support TN bands

Observation 3: If IoT-NTN UE does not support TN bands:

- No requirements apply to ntn-ScenarioSupport-r17 is “gso”

- Only requirements in TS36.102 apply to ntn-ScenarioSupport-r17 is “ngso” or “not included”

Proposal 1: To address the issue where no requirements apply when ntn-ScenarioSupport-r17 is set to "gso" and the UE does not support TN bands, we propose three alternatives for modifying TS36.102. RAN4 can review these alternatives and choose one to implement in the revision of TS36.102.

- Alternative 1

- Revise the Note to: For UE supports NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also applies to UE under NTN configuration according to the UE category and capability

| UE feature/capability | Test list | Applicability notes |
| --- | --- | --- |
| NTN access (ntn-Connectivity-EPC-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1 |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2 |
| NTN scenario support (ntn-ScenarioSupport-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
| Operation in coverage enhancement mode A (ce-ModeA-r13) | Clause 8.2.1.1 (Test 1, Test 2) | The requirements apply only for UE Category M1 |
| Operation in coverage enhancement mode B (ce-ModeB-r13) | Clause 8.2.1.1 (Test 3) | The requirements apply only for UE Category M1 |
| Note: For UE supports NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also applies to UE under NTN configuration according to the UE category and capability |  |  |

- Alternative 2

- Remove the restriction that tests in TS36.102 Clause 8 only apply to ntn-ScenarioSupport-r17 is “ngso” or “not included”.

- Revise the Note to: For UE supports NTN access (ntn-Connectivity-EPC-r17) and TN bands, the requirements in TS36.101 Clause 8 and Clause 9 also applies to UE according to the UE category and capability

| UE feature/capability | Test list | Applicability notes |
| --- | --- | --- |
| NTN access (ntn-Connectivity-EPC-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1 |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2 |
| NTN scenario support (ntn-ScenarioSupport-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
| Operation in coverage enhancement mode A (ce-ModeA-r13) | Clause 8.2.1.1 (Test 1, Test 2) | The requirements apply only for UE Category M1 |
| Operation in coverage enhancement mode B (ce-ModeB-r13) | Clause 8.2.1.1 (Test 3) | The requirements apply only for UE Category M1 |
| Note: For UE supports NTN access (ntn-Connectivity-EPC-r17) and TN bands specified in TS36.101 5.5F, the requirements in TS36.101 Clause 8 and Clause 9 also applies to UE according to the UE category and capability |  |  |

- Alternative 3

- Remove the restriction that tests in TS36.102 only apply to ntn-ScenarioSupport-r17 is “ngso” or “not included”.

- Apply the same requirements of “NGSO only” or “both GSO and NGSO” to “GSO only”

- Remove the Note.

| UE feature/capability | Test list | Applicability notes |
| --- | --- | --- |
| NTN access (ntn-Connectivity-EPC-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1 |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2 |
| NTN scenario support (ntn-ScenarioSupport-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
| Operation in coverage enhancement mode A (ce-ModeA-r13) | Clause 8.2.1.1 (Test 1, Test 2) | The requirements apply only for UE Category M1 |
| Operation in coverage enhancement mode B (ce-ModeB-r13) | Clause 8.2.1.1 (Test 3) | The requirements apply only for UE Category M1 |
| Note: For UE supports NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also applies to UE according to the UE category and capability |  |  |

# Reference

- R4-2417645, (LTE_NBIoT_eMTC_NTN_req-Perf) CR to 36.102 Rel-18 Cat-F for the Note in Table 8.2.1.2-1, CAICT

- R4-2417644, (LTE_NBIoT_eMTC_NTN_req-Perf) Discussion on the understanding of the Note in Table 8.2.1.2-1 of TS 36.102, CAICT

- R4-2500134, CR for TS36.102, applicability rules for IoT-NTN requirements, MediaTek