3GPP TSG-RAN WG4 Meeting #114                                           R4-2500160

Athens, GR, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 	Further consideration on UE RF testability issues for Rel-19 LP-WUS

Agenda Item:	7.26.2.4

Document for:	Approval

Introduction

Some progress was made in last RAN4 meeting on testability issues for LP-WUS [1].

Issue 4-1-1: alignment of common understanding on LP-WUS requirements and corresponding testing

Agreements:

  - The same LP-WUS RF requirements for all three RRC states are assumed. But only one RRC state needs to be tested.

Issue 4-1-2: General framework of interface between the MR and the LR to verify RF performance

Agreements:

  - Group discuss and conclude the general framework for LP-WUS performance verification, consider the following as starting point:

    - Option 1: Counter the detection rate without waking up the MR would be enough to verify LP-WUS RF performance.

    - FFS if the MR should only be woken up when data transfer the system simulator is needed.

    - Other options are not precluded.

The agreement on general framework is a good direction to simplify the test procedure, though the details are far from clear. The following discussion provides our consideration on the testability issues, and most of the contents were presented before.

Discussion

Test under different RRC states

From the study in RAN1, LP-WUS can operate in both RRC_IDLE/INACTIVE and RRC_CONNECTED modes. It is noted that the process for LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode are different. In addition, supporting IDLE/INACTIVE mode and CONNECTED mode is based on UE capabilities, which means a UE supporting LP-WUS operation in IDLE/INACTIVE mode does not necessarily to support operation in CONNECTED mode, and vice versa.

Since the RF test is not the functionality test, ideally the test under one mode is sufficiently to verify the RF capability, and most likely UE would support IDLE/INACTIVE mode for better utilizing the WUS mechanism in terms of power saving. However, we are not so sure that RAN1 could provide confirmable feedback that all UE supporting LP-WUS should support the feature under IDLE/INACTIVE mode. Therefore, to cover the completed operation scenarios, test cases should be designed for both IDLE/INACTIVE mode and CONNECTED mode.

Observation 1: LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode depends on UE capability, which means UE may not support both modes.

Proposal 1: Test cases should be designed separately LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode since the procedures are different and the LP-WUS signals could be different.

As for false alarm rate, we think it should be considered in the conformance test. For the LP-WUS signals, the ones without sub-group ID/UE_ID should not be detected by the specific UEs. Regarding the details on how to design the test cases, e.g. whether sending the LP-WUS with random sub-group ID/UE_ID or different batch with specific ID or without ID can be left to RAN5.

It is noted that during the SI study, RAN1 uses both 0.1% and 1% false alarm rate while just 1% for the detection rate in the evaluation. We think that too stringent false alarm rate may not be necessary. The consequence of false detection just results more power consumption by waking up the MR, but it may not be as serious as the detection rate, since missing the detection means the UE may miss the paging information. Thus, loose false alarm rate could also be considered, e.g. 10%. At least, 1% could be considered for both detection rate and false alarm rate.

Test mode

As preliminarily discussed in [4], we think test mode should be introduced for the LP-WUS test. Normally, the test mode is set under connected mode. However, test mode can also be used for non-connected mode, e.g. V2X UE in out-of-coverage state, and the scenario of beam correspondence in initial access.

After setting up the test mode in connected mode, when the UE enters into the IDLE/INACTIVE mode, with analysis above in section 2.1, the UE can record the detection rate and false alarm rate then report the rates to TE when the UE enters back to the connected mode. In our view, the test mode as well as the details of the test mode can also be left to RAN5.

Proposal 2: Test mode as well as the details of test mode for LP-WUS verification can be left to RAN5.

Configurations of LP-WUS signal for Rx requirements

Currently, for all NR Rx requirements - except spurious emissions, which are tested with the maximum CBW - all other requirements are tested with the lowest, middle, and highest CBWs supported by the UE. However, LP-WUS is fixed to 11 PRBs regardless of the SCS, so the choice of NR carrier CBW for LP-WUR testing has minimal impact. Additionally, some requirements, such as ACS, ASCS, and blocking, are tested in the presence of an interferer, while others are tested without an interferer but with an NR signal padded in the same carrier, such as SE; REFSENS, by contrast, is tested without an NR signal. Thus, the configurations of the LP-WUS signal for Rx requirements should be carefully considered. The following is the preliminary thinking of the configurations.

Table 1: Test configurations for LP-WUR

| Requirements | Test Configurations |
| --- | --- |
| REFSENS | - 5MHz, 15kHz SCS - LP-WUS is configured at center of the carrier - without NR signals in the same carrier |
| Maximum input level | - 5MHz, 15kHz/30kHz SCS - LP-WUS is configured at center of the carrier - without NR signals in the same carrier |
| ACS | - 10MHz, 15kHz/30kHz SCS - LP-WUS is configured at edge of the carrier - without NR signals in the same carrier |
| ASCS | - L/M/H CBWs, 15kHz/30kHz SCS - LP-WUS is configured at center of the carrier - with adjacent NR signals padded in the same carrier |
| In-band blocking | - 10MHz, 15kHz/30kHz SCS - LP-WUS is configured at edge of the carrier - without adjacent NR signals padded in the same carrier |
| Out-of-band blocking | - LP-WUS is configured at center of the carrier - without adjacent NR signals padded in the same carrier |
| Narrow band blocking | - 10MHz, 15kHz/30kHz SCS - LP-WUS is configured at edge of the carrier - without adjacent NR signals padded in the same carrier |
| Spurious response | - 10MHz, 15kHz/30kHz SCS - LP-WUS is configured at edge of the carrier - without adjacent NR signals padded in the same carrier |
| Wide band Intermodulation | - 10MHz, 15kHz/30kHz SCS - LP-WUS is configured at center of the carrier - without adjacent NR signals padded in the same carrier |
| Spurious emissions | - Maximum CBW, 15kHz/30kHz SCS - LP-WUS is configured at center of the carrier - with adjacent NR signals padded in the same carrier |

Proposal 3: Test configurations in Table 1 could be considered as starting point for further discussion.

Conclusion

In this paper we provide our consideration on the framework of LP-WUS with following observations and proposals.

Observation 1: LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode depends on UE capability, which means UE may not support both modes.

Proposal 1: Test cases should be designed separately LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode since the procedures are different and the LP-WUS signals could be different.

Observation 2: Test with complete procedure of waking up MR and report ACK in connected mode is not only time consuming but also incurring fake detection with MR is always on.

Proposal 2: Test mode as well as the details of test mode for LP-WUS verification can be left to RAN5.

Proposal 3: Test configurations in Table 1 could be considered as starting point for further discussion.

- References

[1] R4-2420528, WF on NR LP-WUS UE requirements, vivo

[2] TS 38.509, Special conformance testing functions for User Equipment (UE)

[3] TS 36.509

[4] R4-2405487, Consideration on UE RF aspects for Rel-19 LP-WUS, Huawei, HiSilicon