3GPP TSG-RAN4 #114	R4-2500166

Athens, Greece, 17th Feb. – 21st Feb. 2025

Title:	Discussion on the Note in Table 8.2.1.2-1 of TS 36.102

Source:			Anritsu Corporation

Agenda Item:			5.9.4

Document for:			Approval

# 1.	Introduction

At the RAN4 #113 meeting in Orlando, there was a discussion paper and a CR to propose changing a note in TS 36.102 [1, 2], which was postponed there to discuss further on an interpretation of associated capabilities in TS 36.331 or TS 36.306. [4,5]

In this contribution we share our view on the interpretation of those capabilities related to NTN support.

# 2.	Discussion

2.1 Interpretation of relationships among IEs

There are multiple IEs related to the capabilities of NB-IoT NTN devices as follows.

ue-Category-NB

ntn-Connectivity-EPC-r17

ntn-ScenarioSupport-r17

supportedBandList-xxx

And when we see those relationship in TS 36.311 and TS 36.306 [4,5], though it appears that the IE ue-Category-NB and other IEs are hierarchical and a device which supports NTN shall also support TN features, our current understanding on a capability of the NB-IoT device is that it depends on a combination of the IEs ue-Category-NB, ntn-Connectivity-EPC-r17 and supportedBandList-xxx. (Although we have questions regarding the use case of NB-IoT NTN devices that do not support TN features.) Thus, from the message / IE point of view, we assume it is possible that NB-IoT NTN device may support only NTN bands, which contradicts the previous agreement in 2022 [3].

Observation 1: A capability of the NB-IoT device depends on a combination of the IEs ue-Category-NB, ntn-Connectivity-EPC-r17 and supportedBandList-xxx.

Observation 2: It is possible that NB-IoT NTN device supports only NTN bands.

A following Table 2.1-1 shows the cases based on the combination of these IEs. Cases which do not make sense are greyed out. Note that we omit listing the case that the IE ue-Category-NB is not supported.

Table 2.1-1: NB-IoT device capabilities based on a combination of IEs

| ntn-Connectivity-EPC-r17 | supportedBandList-xxx |  | Remarks |
| --- | --- | --- | --- |
|  | TN band | NTN band |  |
| Not supported | Supported | Supported | This combination cannot exist. |
|  |  | Not supported | Legacy NB-IoT cases |
|  | Not supported | Supported | This combination cannot exist. |
|  |  | Not supported | This combination has no meaning as the device. |
| Supported | Supported | Supported | Supporting both TN and NTN capabilities. |
|  |  | Not supported | This combination cannot exist. |
|  | Not supported | Supported | Only NTN supported NB-IoT devices. (Is this scenario useful when this device cannot access to a TN network?) |
|  |  | Not supported | This combination cannot exist. |

Therefore, though we previously agreed by the WF [3] that the UE needs to pass TS36.101 [6] Cat-NB1/NB2 requirements regardless of the NTN scenario (GSO and/or NGSO), we need to consider the case that the NB-IoT NTN UE does not support TN bands.

Observation 3: Though the current NB-IoT NTN UE capability contradicts the previous agreement in RAN4, we need to consider the case that the NB-IoT UE with NTN capability does not support TN bands.

2.2 Solutions to cover the case when NB-IoT NTN device does not support TN bands

From the conformance test perspective, we prefer that we decide the applicable test requirements based on the combination of IEs ue-Category-NB, ntn-Connectivity-EPC-r17 and supportedBandList-xxx.

Table 2.2-1 shows the combination of IEs and applicable demodulation requirements from the viewpoint of maintaining TS 36.102 specification [7]. For simplicity, we omit listing cases that the IE ue-Category-NB is not supported.

And it should be noted that TN features shall be tested based on TS 36.101 [6] when TN band is supported even if the IE ntn-Connectivity-EPC-r17 is supported.

Table 2.2-1: Combination of IEs and proposal of applicable demodulation requirements to NB-IoT NTN devices from 36.102 maintenance viewpoint

| ue-Category-NB | ntn-Connectivity-EPC-r17 | supportedBandList-xxx |  | Applicable demodulation specifications |
| --- | --- | --- | --- | --- |
|  |  | TN band | NTN band |  |
| Supported | Supported | Supported | Supported | 36.102 only for NTN bands regardless of GSO/NGSO capability |
|  |  | Not supported | Supported |  |

And if we consider applicability of requirements from both TN and NTN device viewpoints, the table above could be expressed as follows.

Table 2.2-2: Combination of IEs and proposed applicable demodulation requirements to NB-IoT devices from both 36.101 and 36.102 viewpoint

| ue-Category-NB | ntn-Connectivity-EPC-r17 | supportedBandList-xxx |  | Applicable demodulation specifications |
| --- | --- | --- | --- | --- |
|  |  | TN band | NTN band |  |
| Supported | Not supported | Supported | Not supported | 36.101 |
|  | Supported | Supported | Supported | 36.101 only with TN bands. 36.102 only with NTN bands regardless of GSO/NGSO capability |
|  |  | Not supported | Supported | 36.102 only for NTN bands regardless of GSO/NGSO capability |

Proposal 1: Correct the applicable test requirements in TS 36.102 that requirements apply to the NB-IoT UE with NTN support regardless of GSO and/or NGSO support.

The actual changes in TS 36.102 would be as follows.

Table 8.2.1.2-1: Requirements applicability for optional UE features

| UE feature/capability | Test list | Applicability notes |
| --- | --- | --- |
| NTN access (ntn-Connectivity-EPC-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1 |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2 |
| NTN scenario support (ntn-ScenarioSupport-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
| Operation in coverage enhancement mode A (ce-ModeA-r13) | Clause 8.2.1.1 (Test 1, Test 2) | The requirements apply only for UE Category M1 |
| Operation in coverage enhancement mode B (ce-ModeB-r13) | Clause 8.2.1.1 (Test 3) | The requirements apply only for UE Category M1 |
| Note 1: Void For UE supports NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also applies to UE according to the UE category and capability |  |  |

- Conclusion

In this contribution we share our view on the interpretation of those capabilities related to NTN support.

Observation 1: A capability of the NB-IoT device depends on a combination of the IEs ue-Category-NB, ntn-Connectivity-EPC-r17 and supportedBandList-xxx.

Observation 2: It is possible that NB-IoT NTN device supports only NTN bands.

Observation 3: Though the current NB-IoT NTN UE capability contradicts the previous agreement in RAN4, we need to consider the case that the NB-IoT UE with NTN capability does not support TN bands.

Proposal 1: Correct the applicable test requirements in TS 36.102 that requirements apply to the NB-IoT UE with NTN support regardless of GSO and/or NGSO support.

Table 8.2.1.2-1: Requirements applicability for optional UE features

| UE feature/capability | Test list | Applicability notes |
| --- | --- | --- |
| NTN access (ntn-Connectivity-EPC-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1 |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2 |
| NTN scenario support (ntn-ScenarioSupport-r17) | Clause 8.2.1.1 (Test 1, Test 2, Test 3) | The requirements apply only for UE Category M1, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
|  | Clause 8.3.1.1 (Test 1, Test 2) | The requirements apply only for UE Category NB1, NB2, and only when ntn-ScenarioSupport-r17 is “ngso” or is not included |
| Operation in coverage enhancement mode A (ce-ModeA-r13) | Clause 8.2.1.1 (Test 1, Test 2) | The requirements apply only for UE Category M1 |
| Operation in coverage enhancement mode B (ce-ModeB-r13) | Clause 8.2.1.1 (Test 3) | The requirements apply only for UE Category M1 |
| Note 1: Void For UE supports NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also applies to UE according to the UE category and capability |  |  |

# 4.	References

[1] R4-2417644, “Discussion on the understanding of the Note in Table 8.2.1.2-1 of TS 36.102”, CAICT, RAN4 #113, Orlando, USA

[2] R4-2417645, “CR to 36.102 Rel-18 Cat-F for the Note in Table 8.2.1.2-1”, CAICT, RAN4 #113, Orlando, USA

[3] R4-2220278, “WF on IoT-NTN UE demodulation and CQI reporting requirements”, MediaTek, RAN4 #105, Toulouse, France

[4] TS 36.331, “Radio Resource Control (RRC); Protocol specification”, v18.4.0, 2024-12

[5] TS 36.306, “User Equipment (UE) radio access capabilities”, v18.4.0, 2024-12

[6] TS 36.101, “User Equipment (UE) radio transmission and reception”, V18.8.0, 2024-12

[7] TS 36.102, “User Equipment (UE) radio transmission and reception for satellite access”, v18.7.0, 2024-12