# 3GPP TSG-RAN WG4 Meeting #114	R4-2500234

# Athens, GR, 17th – 21st February, 2025

Agenda item:	5.27.1

Source:	Apple

Title:	On RRM core requirements maintenance for NES

Document for:	Discussion

# Introduction

In last RAN4#113 meeting, the WF for NES has been agreed in [1] and moderator has captured all open issues in the topic summary [2]. There are still some opens issues for further discussion in the maintenance stage, and in this contribution, we continue the discussion on remaining RRM core requirements for NES.

# Discussion

Issue 1-1-1: Neighbour cells on carrier of SSB-less SCell

The remaining issues from last meeting are as followings:

Issue 1-1-1: Neighbour cells on carrier of SSB-less SCell Sub-issue 1: Whether servingCellMO can be present for inter-band SSB-less SCell. Agreement: RAN4 provide analysis on the categorization of intra-frequency/inter-frequency, whether MG is needed and any other aspects (if identified) for following cases: - When servingCellMO is configured - When servingCellMO is not configured RAN4 provide the above analysis to RAN2 (if consensus can be reached) and wait for RAN2 decision on whether servingCellMO is present for carriers of SSB-less SCell.

When we assume SSB-less SCell operation, we never touch the neighbor cell measurement on the carrier of SSB-less SCell. The SSB-less SCell is because network would like to save the energy of this cell when the traffic load of this cell is low, but it does not necessarily mean all the other cells on this carrier are in the NES mode (without SSB transmission). If some of the neighbor cells on this carrier have SSB transmission and network configure such MO to UE, it’s unclear to us if it shall be treated as intra-frequency or inter-frequency. In R16 we have a feature for inter-frequency measurement without MG, and the condition to support that feature is also “the SSB is completely contained in the active BWP of the UE”, as following,

Figure 1. example of neighbor cell SSBs located in the active BWP of SSB-less SCell

Similarly, in NES case, if there are multiple SSBs from neighbor cells located in the active BWP of the SSB-less SCell as figure 1, these neighbor cell measurements cannot be treated as intra-frequency anyway. Thus, we think if neighbor cells on carrier of SSB-less SCell have SSB transmission, the measurement for those neighbor cells shall be treated as inter-frequency measurement without MG as long as the SSBs from those neighbor cells can be contained in the active BWP of SSB-less SCell.

In RAN4#112 meeting, some companies commented that if network configures serving cell MO for the SSB-less SCell, the intra-frequency and inter-frequency shall be decided according to the serving cell MO and other MOs of neighbor cells. However, in last RAN2#127bis meeting, RAN2 had one agreement that,

R2-2409392 Report of [AT127bis][008][NES] SSB-less Scell (Huawei) Huawei, HiSilicon discussion Rel-18 Netw_Energy_NR-Core - Noted Agreements 1 From RAN2 perspective, no benefit is identified for configuring servingCellMO for inter-band SSB-less SCell. 2 No LS is sent to R4. 3 Discuss in the next meeting on whether to capture “For inter-band SSB-less SCell(s), this field is not present” in the conditional presence of servingCellMO, depending on R4 progress. 4 Postpone the discussion on intra-band SSB-less SCell.

In our view it’s very clear there is no benefit to configure such servingCellMO for inter-band SSB-less SCell and it won’t break anything in the system without such servingCellMO configuration for SSB-less SCell. We recommend RAN4 to consider as RAN2 mentioned in their agreement that, “For inter-band SSB-less SCell(s), this field is not present” in the conditional presence of servingCellMO. If the target SSB from neighbor cell containing in the active BWP can be treated as gap-less measurement, then we didn’t see much difference between calling it intra-frequency measurement or inter-frequency measurement, especially from UE implementation perspective.

Since we have already completed the core part of this WI, if companies have concern to open new discussion in the maintenance feature, we are also fine to limit the scenario in R18 to that all neighbor cells on the CC of SSB-less SCell shall be SSB-less cells as well.

Proposal 1: to agree with RAN2 that “For inter-band SSB-less SCell(s), this field is not present” in the conditional presence of servingCellMO.

Proposal 2: if neighbor cells on carrier of SSB-less SCell have SSB transmission, the measurement for those neighbor cells shall be treated as inter-frequency measurement without MG as long as the SSBs from those neighbor cells can be contained in the active BWP of SSB-less SCell.

If needed, we are also open to clarify this NES case in the current condition for definition of inter-frequency measurement without MG.

Proposal 3: open to clarify the NES case in the current condition for definition of inter-frequency measurement without MG, as following:

A measurement is defined as an inter-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) in active BWP, for UE capable of interFrequencyMeas-NoGap provided that

- the UE supports interFrequencyMeas-Nogap-r16 [15], and

- the SSB is completely contained in the active BWP of the UE, or

- the SSB of neighbor cells is completely contained in the active BWP of SSB-less SCell of the UE

- For inter-frequency SSB based measurements without measurement gaps, UE may cause scheduling

restriction as specified in clause 9.3.9.3.

- Note: Non-CA capable UE is not expected to indicate support of interFrequencyMeas-Nogap-r16 [15].

# Conclusion

In this contribution, we continue the discussion on remaining RRM core requirements for NES.

Proposal 1: to agree with RAN2 that “For inter-band SSB-less SCell(s), this field is not present” in the conditional presence of servingCellMO.

Proposal 2: if neighbor cells on carrier of SSB-less SCell have SSB transmission, the measurement for those neighbor cells shall be treated as inter-frequency measurement without MG as long as the SSBs from those neighbor cells can be contained in the active BWP of SSB-less SCell.

Proposal 3: open to clarify the NES case in the current condition for definition of inter-frequency measurement without MG, as following:

A measurement is defined as an inter-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) in active BWP, for UE capable of interFrequencyMeas-NoGap provided that

- the UE supports interFrequencyMeas-Nogap-r16 [15], and

- the SSB is completely contained in the active BWP of the UE, or

- the SSB of neighbor cells is completely contained in the active BWP of SSB-less SCell of the UE

- For inter-frequency SSB based measurements without measurement gaps, UE may cause scheduling

restriction as specified in clause 9.3.9.3.

- Note: Non-CA capable UE is not expected to indicate support of interFrequencyMeas-Nogap-r16 [15].

# References

[1] R4-2420121, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon, RAN4 #113

[2] R4-2418268, Topic summary for [113][210] Netw_Energy_NR, Moderator (Huawei), RAN4#113