# 3GPP TSG-RAN WG4 Meeting #114	R4-2500239

# Athens, GR, 17th – 21st February, 2025

Agenda item:	7.29.4.1

Source:	Apple

Title:	On R19 NTN (e)RedCap RRM requirements

Document for:	Discussion

# Introduction

The WF on RRM requirements for Rel-19 NR NTN phase3 [1] has been agreed in RAN4#113, and the open issues have been summarized in the topic summary for [113][230] NR_NTN_Ph3_Part1 [2].

In this contribution, we continue the discussion on the RRM requirement for R19 NTN (e)RedCap.

# RRM for support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN

Issue 1-1-1: Whether to define measurement requirement for inter-RAT (NR NTN to E-UTRAN TN) in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN?

<Way Forward>: - Proposal 1 (CATT, Xiaomi, LG, Samsung CMCC, Apple, Ericsson, HW, ZTE): - RAN4 to define measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN. - Proposal 2 (vivo): - RAN4 not to define the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

In Rel-17 NTN, RAN4 has cell reselection requirement for NTN to LTE TN (inter-RAT E-UTRAN cells with TN carrier). That means: from the network deployment perspective, it’s a feasible scenario to consider for mobility. For RedCap requirement in R17, RAN4 also has requirement for measurements of inter-RAT E-UTRAN cells for RedCap UE. Thus, we think it’s necessary to have the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement for (e)RedCap UE, and the existing cell reselection requirements in RRC_IDLE/RRC_INACTIVE state defined for Rel-17 and Rel-18 (e)NTN can be the baseline for (e)RedCap 2Rx UE with FR1-NTN bands.

Proposal 1: RAN4 to define measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement for (e)RedCap UE. The existing requirements for measurements of inter-RAT E-UTRAN cells with TN carrier can be used as baseline.

Issue 1-2-2: eDRX configuration

< Agreement >: - For earth-moving LEO scenario: - All eDRX cycle cannot be applied for RedCap/eRedCap UEs in NTN and not to define requirements for eDRX in earth-moving beam LEO scenario. - For quasi-earth fixed LEO scenario: - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN. - Requirements are applicable for 2.56s and 5.12s eDRX cycle - FFS for 10.24s eDRX cycle and larger than 10.24s eDRX cycle

RAN4 needs more discussion on whether new requirement shall be introduced for RRM relaxation and eDRX. For example, in legacy NTN requirement, the 2.56s DRX is not assumed to be well utilized in the scenario where earth-moving LEO is deployed as the target satellite or serving satellite for measurement. The main reason is: if earth-moving LEO is used and the footprint on earth is not that large, e.g., couple kilometers, then the longer measurement delay due to using 2.56s will fail the mobility. The remaining issue is whether to apply the eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN if the eDRX cycle ≥10.24s in quasi-earth fixed LEO scenario. The definition of Quasi-Earth-fixed cell is: An NTN cell fixed with respect to a certain geographic area on Earth during a certain time duration. It can be provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period (e.g., the case of NGSO satellites generating steerable beams). In TS38.331, t-Service is used to decide the service time for both service link switches in NTN quasi-Earth fixed cell and feeder link switches for both NTN quasi-Earth fixed and Earth-moving cell. However, there is no any limitation in the spec for the total service length of a quasi-Earth fixed cell. Thus, we propose: for quasi-earth fixed LEO scenario, to define and apply eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN for all eDRX cycles.

Proposal 2: for quasi-earth fixed LEO scenario, to define and apply eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN for all eDRX cycles.

Issue 1-2-4: Multiple SMTCs

<Agreements>: - RAN4 to define RRM requirements for multiple SMTC-s for a single frequency carrier for (e)RedCap UEs. - For whether to support parallel measurements on multiple NGSO satellites within a SMTC (Feature 25-5) for (e)RedCap UEs? - Option 1 (MTK, Xiaomi, Samsung, ZTE, Ericsson, LG, QC): Not support - Option 2 (HW, Nokia): FFS - Further discuss the supported number of SMTCs, the number of cells and SSB, the number of supported satellites, the proximity distance between two SMTCs. - Further discuss the specific measurement requirements, i.e, whether and how to extend measurement period. - Further discuss other R17 NTN UE capabilities (i.e, Feature 25-2/25-3) related to 'in parallel'.

Regarding the multiple NGSO satellites within a SMTC (Feature 25-5), it will require UE to process the multiple signals from different NGSO satellites with different Doppler compensations. Compared with the legacy NTN UE, power saving and complexity would be more important to the (e)RedCap UE, and therefore we prefer to not consider parallel measurements on multiple NGSO satellites within a SMTC (Feature 25-5) for (e)RedCap UEs.

Proposal 3: not consider parallel measurements on multiple NGSO satellites within a SMTC (Feature 25-5) for (e)RedCap UEs.

With the multiple SMTC configured on the same carrier, in legacy NTN requirement, RAN4 defined the proximity for the SMTCs as following,

9.1C.9.3 Collision between multiple SMTCs on a SAN carrier Collision between occasions of multiple SMTCs may occur as specified in this clause. When UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to 4 ms. The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion.

The proximity distance between the SMTCs reflects the UE capability of how fast it can complete the post-processing for the data buffered from prior SMTC. If network configures 2 SMTCs on a NTN carriers and the proximity distance between these two SMTCs are less than 4ms, UE is required to measurement only one of them in legacy NTN. Now, considering the RedCap UE has reduced UE processing capability than legacy non-RedCap UE, the proximity distance might need to be revisited in the spec. We think the relaxation for proximity could be considered for (e)RedCap UE in NTN, e.g.,

    - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

      - the two SMTC occasions are fully or partially overlapping in time domain, or

      - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to (4+x) ms. (x is the relaxation for (eRedCap UE)), x can be FFS. We propose 1ms as starting point.

    - The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion.

Proposal 4: to consider relaxation for proximity when (e)RedCap UE is configured with more than one SMTC on a SAN carrier, i.e.,

    - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

      - the two SMTC occasions are fully or partially overlapping in time domain, or

      - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to (4+x) ms. (x is the relaxation for (eRedCap UE)), x can be FFS. (We propose 1ms as starting point)

    - The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion.

# Conclusion

In this contribution, we discuss the RRM requirement scope for WI of Non-Terrestrial Networks (NTN) for NR Phase 3.

Proposal 1: RAN4 to define measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement for (e)RedCap UE. The existing requirements for measurements of inter-RAT E-UTRAN cells with TN carrier can be used as baseline.

Proposal 2: for quasi-earth fixed LEO scenario, to define and apply eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN for all eDRX cycles.

Proposal 3: not consider parallel measurements on multiple NGSO satellites within a SMTC (Feature 25-5) for (e)RedCap UEs.

Proposal 4: to consider relaxation for proximity when (e)RedCap UE is configured with more than one SMTC on a SAN carrier, i.e.,

    - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

      - the two SMTC occasions are fully or partially overlapping in time domain, or

      - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to (4+x) ms. (x is the relaxation for (eRedCap UE)), x can be FFS. (We propose 1ms as starting point)

    - The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion.

# References

[1] R4-2420104, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT, RAN4#113

[2] R4-2418288, Topic summary for [113][230] NR_NTN_Ph3_Part1, CATT, RAN4#113