3GPP TSG-RAN WG4 Meeting #114		R4-2500245

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	7.23.3.1

Source:	Apple

Title:   	On UE RRM requirements for UE-to-UE CLI handling

Document for:	Discussion

# 1	Introduction

In the latest R19 SBFD WI [1], there is RAN4 objective on UE CLI measurement and reporting, as shown below:

- L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

At the last RAN4 meeting, a WF was agreed [2]. In this contribution, we continue to share our views on the open issues.

# 2	Discussion

Support of Measurement methods

Regarding the measurement methods, the agreements were captured in the WF as below.

#### Issue 1-1-2: Measurement methods - Agreements (from Thu AH session) - RAN4 shall specify L1 CLI measurement requirements at least for below methods: - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Additional methods can be considered based on further RAN1 agreements. - Differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions.

RAN1 made the following agreement in RAN1#119,

Agreement For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported - Method #3: UE measures RSSI within UL subband - A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig.

Therefore, we propose that RAN4 specifies the measurement requirement for Method#3. Regarding the fact that a UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol, RAN1 agreement that measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig should be sufficient. If needed, we can also consider adding a statement in the RAN4 specification that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol.

Proposal 1: It is proposed to specify L1 CLI measurement requirement for Method 3, UE measures RSSI within UL subband, per RAN1 agreement, in addition to Method#1 and Method#2. FFS if it should be clarified in the RAN4 specification that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol.

Number of measurement samples

From the current simulation results provided so far, the measurement accuracy between 1 sample and 3 samples is around 0.5-1dB. Since L1 measurement is specified to reduce measurement delay to allow timely network scheduling or coordination to avoid CLI, we think it is better to prioritize short measurement delay to slightly degraded measurement accuracy. Measurement accuracy can be specified in light of the measurement accuracy.

Proposal 2: It is proposed to consider specifying requirement with 1 sample measurement considering the reduced measurement delay expected of L1 measurement and the difference in measurement accuracy between 1 sample and 3 samples. The measurement accuracy requirement can be further discussed.

Time offset between DL timing and SRS arrival timing

In response to RAN4’s inquiry, RAN1 provided their views in [3] as copied below:

# 1 Overall description When discussing L1 UE-to-UE CLI measurement reporting, RAN1 has not made a distinction between the two application scenarios mentioned in the RAN4 LS (intra-cell vs. inter-cell UE-to-UE L1 CLI measurement). L1 UE-to-UE CLI measurement and reporting specified by RAN1 can be applied for both application scenarios. For the case of intra-cell L1 SRS-RSRP measurement and reporting, a victim UE may be configured with a set of SRS-RSRP measurement resources that overlap with the resources used by the aggressor UE(s) to transmit SRS since the gNB has knowledge the aggressor UE’s SRS configurations. However, for inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs. But specification impact of this in Xn/F1 has been excluded according the following note from the WID - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

Given the following points from the RAN1 reply LS:

- For intra-cell L1 SRS-RSRP measurement and reporting, a victim UE may be configured with a set of SRS-RSRP measurement resources that overlap with the resources used by the aggressor UE(s) to transmit SRS since the gNB has knowledge the aggressor UE’s SRS configurations. However, for inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs. But specification impact of this in Xn/F1 has been excluded.

- From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

We think it is reasonable that RAN4 specify requirements for intra-cell case only. As a result, the cell phase synchronization error of 3us is not considered in the time offset between DL reference timing in the serving cell and SRS arrival time. More specifically, the time offset is no larger than Terror_SRS_RSRP, where Terror_SRS_RSRP is TC × NTA_offset + 1.67us for FR1 or TC × NTA_offset + 0.67us for FR2.

Proposal 3: Per RAN1 LS that from RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement, it is proposed that RAN4 focuses on requirements for Intra-cell UE-to-UE L1 CLI measurement scenario.

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: It is proposed to specify L1 CLI measurement requirement for Method 3, UE measures RSSI within UL subband, per RAN1 agreement, in addition to Method#1 and Method#2. FFS if it should be clarified in the RAN4 specification that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol.

Proposal 2: It is proposed to consider specifying requirement with 1 sample measurement considering the reduced measurement delay expected of L1 measurement and the difference in measurement accuracy between 1 sample and 3 samples. The measurement accuracy requirement can be further discussed.

Proposal 3: Per RAN1 LS that from RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement, it is proposed that RAN4 focuses on requirements for Intra-cell UE-to-UE L1 CLI measurement scenario.

# 4	References

RP-241614, "Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)," Huawei

RP-242420097, "WF on RRM requirements for NR_duplex_evo," Huawei, HiSilicon

R1-2410890, "Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation," RAN1