3GPP TSG-RAN WG4 Meeting #114														R4-2500246

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.17.2.1

Source:	Apple

Title:	On FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

# Introduction

The WF of NR Radio Resource Management (RRM) Phase 5 [1] has been agreed in RAN4#113, which includes open issues for further discussion. In this contribution, we continue the discussion for FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor.

# Discussion

Conditions to apply L3 measurement delay reduction by optimizing Rx BSF

As captured in the WF, there was agreement on the triggering conditions to activate L3 measurement delay reduction and no agreement yet on the triggering conditions to deactivation.

### Issue 1-1: Conditions to apply L3 measurement delay reduction by optimizing Rx BSF Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF (including activation condition and de-activation conditions): - Option 1(CATT, LGE, CMCC, Apple, HW, vivo, OPPO, ZTE, MTK, QC, Samsung): RAN4 to introduce signal quality (e.g., cell edge) based triggering condition to activate/deactivate multi-Rx for L3 measurement - If option 1 is agreed, discuss whether option 1a is agreeable - Option 2(CATT, Apple, OPPO, MTK, QC, Samsung): RAN4 to introduce UE mobility status based triggering condition to activate/deactivate multi-Rx for L3 measurement - Opponent to option 2 (CMCC, Intel): it is proposed not to consider UE mobility as triggering condition for activation/deactivation of FBS - If option 2 is agreed, discuss whether option 2a is agreeable. - Option 3(Nokia, Ericsson): Use L3 event-triggered reporting conditions for triggering FBS - Discuss whether option 3 is agreeable. - Discuss Option 3b and option 10 to decide if the UE FBS status report to network is needed. - Other options can be discussed online based on companies’ comments. Agreement: - RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement. Agreement: - Not support UE mobility status based triggering condition in Rel-19. Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” : Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2418276.

As raised in our previous contribution [2], to save power at the UE, conditional activation and deactivation of L3 measurement delay reduction based on signal quality is needed. Besides signal quality, there are other factors at both the network and UE that may require overriding the configured conditional activation and deactivation.

From the network perspective, there are two motivations:

- Load balancing: The network wants to switch the UE to another serving cell to balance the load in the network.

- QoS handling: The network monitors the QoS performance and decides that the current serving cell does not have enough resources to meet the QoS requirements and hence handing over to another cell is preferred.

From the UE perspective, UE may choose to override the configured conditional activation and de-activation of fast L3 measurements based on

- Power consumption

- Overheating

- UE QoS monitoring

For the conditional activation/deactivation based on signal quality, say RSRP, some RSRP threshold is expected to be configured by the network. To override it, the network can send a RRCReconfiguration message to provide extremely high or extremely low RSRP threshold and UE will compare the measured RSRP with the threshold to activate or deactivate L3 measurement delay reduction.

For the UE to initial overriding, the UE could use UAI or MAC-CE. If UAI is considered, either the existing UAI IE “multiRx-PreferenceFR2-r18” can be re-purposed to indicate not only L1 multi-RX operation, but also L3 measurement delay reduction, or a new IE such as “FastL3Measurement-r19” can be designed.

Proposal 1: To consider other factors, such as load balancing and QoS handling at the network or power consumption, overheating, and UE QoS monitoring at the UE, it is proposed to allow both the UE and network to override the configured conditional activation and de-activation of L3 measurement delay reduction (or fast L3 measurements). Details are FFS.

SSB processing delay/time

Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC Agreement: - For scenario 1/2/3/4 - additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement. - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms - where TSSB_processing = 2 ms Agreement: - For scenario 7 - to consider an additional processing delay (2ms) after the last SMTC periodicity [Wayforward] - For scenario 5 - FFS: - Option 1: to consider an additional processing delay (2ms) in the handover delay equation - Option 2: In the handover delay equation, SSB processing time (2ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - Option 3: no need to add additional processing time in the delay requirement.

When both panels are activated to make measurements, the measurement samples from each panel can be processed sequentially if only one searcher is assumed. Since the SMTC periodicity is 5ms or longer, it is possible for the UE to process both samples (one from each panel) within one SMTC periodicity. Therefore, it is OK to only consider some additional processing time after the last SMTC periodicity. To that end, Option 1 is reasonable.

Proposal 2: It is possible for the UE to process both samples (one from each panel) within one SMTC periodicity. Therefore, it is OK to only consider some additional processing time after the last SMTC periodicity for Scenario 5, i.e., Option 1.

Requirement at transition between the normal L3 measurement and FBS L3 measurement

Issue 1-4-1: FFS: requirement at transition between the normal L3 measurement and FBS L3 measurement - Proposal (ZTE): - The requirement at transition between the normal L3 measurement and FBS L3 measurement should be identified. There are two options: - Option 1: Reuse similar solution as the requirements at transition between measurement within gap and measurement outside gap, i.e. the longer delay is the period when transition happens. - Option 2: Once the transition happens, apply the requirements after the transition, no matter it is the normal L3 measurement or the FBS L3 measurement. - Recommended WF - TBA

Similar issues were discussed in R18 multi-RX WI. In the end, such a requirement were not pursued due to the associated complexity such as the needed conditions to define the start and end of transition. For L3 measurement, similar complexity exists and thus such a requirement should not be specified.

Proposal 3: Similar to the handling of R18 L1 measurements, no requirement is specified at transition between the normal L3 measurement and FBS L3 measurement.

# Conclusion

In this contribution, we continue the discussion for FR2-1 SSB based L3 measurement delay reduction for connected mode.

Proposal 1: To consider other factors, such as load balancing and QoS handling at the network or power consumption, overheating, and UE QoS monitoring at the UE, it is proposed to allow both the UE and network to override the configured conditional activation and de-activation of L3 measurement delay reduction (or fast L3 measurements). Details are FFS.

Proposal 2: It is possible for the UE to process both samples (one from each panel) within one SMTC periodicity. Therefore, it is OK to only consider some additional processing time after the last SMTC periodicity for Scenario 5, i.e., Option 1.

Proposal 3: Similar to the handling of R18 L1 measurements, no requirement is specified at transition between the normal L3 measurement and FBS L3 measurement.

# References

- R4-2420093, WF for NR_RRM_Ph5_Part1, Apple

- R4-2418536, On FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor, Apple