3GPP TSG-RAN WG4 Meeting #114		R4-2500262

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.16.2

Source:	Apple

Title:	On Spatial Channel Modeling

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#113 spatial channel model for demodulation requirements was discussed and way forward [1] was approved. In this contribution we share our views on scope of study for spatial channel models for defining demodulation and CSI reporting requirements in RAN4.

# 2. Discussion

## General Aspects

In RAN4#113 the following agreements were made:

#### Cases for SU-MIMO Way Forward: Random PMI as a baseline, FFS Fixed. Interested companies invited to choose based on most common PMI in PMI histogram. #### Duplex Mode, CBW and SCS Agreement: Carrier Frequency: 3.5 GHz Duplex Mode: TDD CBW: 40 MHz SCS: 30 kHz #### Receiver Type Agreement: IRC Baseline for SU-MIMO; Interested Companies to use IRC and E-IRC for MU-MIMO and LMMSE for SU-MIMO

#### Cases for MU-MIMO

Agreement:

Interested companies to bring to RAN4#114 views on the following cases:

PDSCH

2+2 layers with IRC (type I orthogonal and random precoding, to distinguish target and co-scheduled UE)

Interested companies are encouraged to assess 2+2 layers with IRC and E-IRC (type I orthogonal and random precoding, to distinguish target and co-scheduled UE)

For MU-MIMO RAN4 has defined requirements in Rel-17, Rel-18 with TDL channel model. The purpose of introducing requirements with MU-MIMO is to determine if the UE can reject or suppress the inter user interference. Employing TDL channels doesn’t limit testing of UE processing to cancel inter-user interference in case of MU-MIMO. Further justification is needed for studying impact of MU-MIMO performance with spatial channel model.

- UE processing is tested sufficiently for cancelling inter-user interference in MU-MIMO scenarios with TDL channel model.

Further justification is needed for considering MU-MIMO for the study.

In case MU-MIMO use cases are agreed for the study, the test setup should be what RAN4 has used so far – with random PMI for target UE, orthogonal/random PMI for co-scheduled UEs. Scenarios with multiple UEs, PMI feedback should be out of scope. There is no use is the test setup in the study cannot be used for the actual test setup when requirements are defined.

In case MU-MIMO is agreed use case for the study the test setup is limited with what RAN4 has used so far – single UE with random PMI for target UE.

In case MU-MIMO is agreed use case for the study the test setup shall not include PMI feedback and/or multiple UEs.

#### Metrics for comparison

Way forward from RAN4#113:

Contributions welcome at RAN4#114.

So far we have agreed on some initial test cases for SU-MIMO, but we have not discussed the metric we would employ to compare the methodologies to rationalize the need for a special channel model for demodulation requirements.

One of the objectives of the SI is to identify the limitations of current channel models and how MIMO performance is impacted:

The necessity of introducing spatial channel model should be well justified. We need to further discuss the metric used to compare methodologies that help identify limitations in the current methodology that impacts UE/BS demod/ MIMO performance. Without a concrete idea on how to this is done, it would be difficult to move ahead with the agreed comparison.

RAN4 to discuss test metrics to compare the methodologies that help identify limitations in the current methodology that impacts UE MIMO performance.

As a first step, we should also compare the performance with existing TDL channel model to newly proposed CDL and TDL based methodologies and identify limitations with existing channel models.

Compare performance with existing TDL channel models and newly proposed spatial channel models to identify limitation with existing channel models.

We also need to evaluate if results are well aligned among companies with new methodologies including the test complexity and result convergence time.

RAN4 to consider the following aspects in determining the feasibility of spatial channel model methodology:
- Test convergence time
- Alignment of results
- Test complexity

## CDL based methodologies

For CDL based methodologies the following agreements were made in RAN4#113:

#### Antenna Array Virtualisation Notation Agreement: - Use 5-argument AAV notation of (M,N,P,Ms,Ns) for all configurations - (M,N,P,Ms,Ns) with sub-array - (M,N,P,1,1) for direct passthrough - (M,N,P,M,N) for ‘fully connected’ #### GCS Antenna Coordinates for gNB Agreement: - Height = 25 m - Azimuth = 0 (placement on the x axis) - X Coordinate = 0 m #### LCS Antenna Coordinates for gNB Agreement: - α = 0° - β = 10° - γ = 0° #### GCS Antenna Coordinates for UE Agreement - Height = 1.5 m; - Azimuth = 0; (placement on the x axis); - X Coordinate = 100 m; (Distance from gNB = 100 m) Note: This set up is for the macro scenario #### LCS Antenna Coordinates for UE Agreement: - α = 180° - β=0 - γ = 0° #### BS Antenna Polarisation Agreement - BS Polarisation slant (+45, -45) #### BS Radiation Pattern Agreement: - BS antenna radiation pattern as defined in TR 38.901 Table 7.3-1.

#### Angles of Departure and Angles of Arrival

Way Forward:

Option 1: Replace the mean angle   in the last position of departure and arrival angles generated equation by the desired angle as  .

Set mf,desired based on the LOS direction between the TX and RX

Option 2: Following 38.827 (i.e., mf,desired = mf,model).

In TR 38.827 mf,desired is set as mf,model. We compared the performance between between the 2 options and don’t see any significant performance difference. Hence , we propose to use the approach in 38.827 to  set mf,desired = mf,model to align with other channel model generation parameters from the TR.

Use mf,desired = mf,model in deriving angles of departure and arrival based on 38.827.

#### BS Antenna Virtualiser

Way Forward:

Broadside for BS AAV options except fully connected.

Max receive power for standard fully connected AAV.

Interested companies to study with AAV to use max receive power of single strongest direction.

On the BS virtualizer we have the following views:

Broadside direction. This is only appliable to AAV with sub-array configuration. This is not preferred as there could be an issue to obtain > 2 layers if the sub-arrays are pointing in the same direction.

Max Receive Power. This is the method from TS38.827 direction is the most straight forward and convenient to adopt. This is applicable to both AAV without sub-array and with sub-array configuration. It is easy to specify this for different TX port configurations based on the channel model.

For BS antenna configurations with AAV we propose to use the max receive power for BS antenna virtualizer.

For BS antenna configurations with AAV use max receive power based BS antenna virtualizer.

#### Frequency Bands

Proposals:

- Option 1: RAN4 to study if the same CDL channel model can be defined for all carrier frequencies/ bands in given frequency range

The current models in the TR 38.827 are defined for specific scenarios, carrier frequency, with some assumptions on RMS delay spread, UE velocity, etc. RAN4 defines band agnostic demodulation and CSI reporting requirements.

- The CDL channel models in the TR 38.827 are defined for specific scenarios, carrier frequency, with some assumptions of RMS delay spread and UE velocity.

It would be very important to study if the spatial channel model can be defined for all applicable carrier frequencies/ bands for each FR.

RAN4 to study if the same channel model can be defined for all carrier frequencies/ bands in given frequency range – example FR1/ FR2.

#### Channel Model simplification

In Rel-15 we defined simplified TDL channel models with maximum of 12 taps by taking the channel model in 38.901 and combining close by paths and having a resolution of 5ns. The CDL channel models in 38.901 and 38.827 are defined for a large number of clusters – 23/24 for NLOS cases. RAN4 needs to further study if they models can be simplified by taking a similar approach as described in 38.101-4 Annex B.2.1.

RAN4 to study if the CDL channel models can be simplified to smaller number of clusters.

## Comparison of methodologies

#### TDL and CDL Methodologies

Way Forward:

Channel Model Configurations:

- Option A1: CDL (TR 38.753) with Table 7.2.1-8 in TR 38.827

- Option A2: CDL (TR 38.753) with Table TBD

- Option B: TDL extensions as defined in (R4-2412762)

- Option C: TDL extensions as defined in (R4-2415382)

- Option D: TDL extensions as defined in (R4-2419895)

AAV Configurations:

- Option 1: with Antenna Array Virtualisation (AAV) using subarray configuration

  - (M,N,P,Ms,Ns)  = (8,8,2,8,4) for 4Tx CSI-RS Ports (option X)

  - (M,N,P,Ms,Ns)  = (8,8,2,8,2) for 8Tx CSI-RS Ports (option X)

  - (M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports (option Y)

  - (M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports (option Y)

- Option 2: with AAV without subarray (fully connected)

  - (M,N,P,Ms,Ns) = (8,8,2,8,8) for 8Tx and 4Tx CSI-RS Ports

- Option 3: with AAV with the following configuration:

  - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

  - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

RAN4 to define 38.753 CDL with randomness reduction framework and AAV framework.

In addition to the CDL and TDL based spatial channel model options, we should also include the existing TDL based channel models as a baseline for comparison. The performance with existing TDL channel models shall be compared to identify the limitation if any of existing methodology. As shown in [2], we don’t observe better performance with CDL channel model either for PDSCH performance for 4x4 with 4layers, or PMI reporting with eType II.

Add current TDL channel model for comparison against new spatial channel models to compare performance and identify limitations if any with existing methodology.

# 3. Conclusion

In this paper, we provide our views on scope of study for spatial channel models for defining demodulation and CSI reporting requirements in RAN4. Our observations and proposals are captured below:

General aspects

- UE processing is tested sufficiently for cancelling inter-user interference in MU-MIMO scenarios with TDL channel model.

- Further justification is needed for considering MU-MIMO for the study.

In case MU-MIMO is agreed use case for the study the test setup is limited with what RAN4 has used so far – single UE with random PMI for target UE.

In case MU-MIMO is agreed use case for the study the test setup shall not include PMI feedback and/or multiple UEs.

RAN4 to discuss test metrics to compare the methodologies that help identify limitations in the current methodology that impacts UE MIMO performance.

Compare performance with existing TDL channel models and newly proposed spatial channel models to identify limitation with existing channel models.

RAN4 to consider the following aspects in determining the feasibility of spatial channel model methodology:
- Test convergence time
- Alignment of results
- Test complexity

CDL based methodologies

Use mf,desired = mf,model in deriving angles of departure and arrival based on 38.827.

For BS antenna configurations with AAV use max receive power based BS antenna virtualizer.

- The CDL channel models in the TR 38.827 are defined for specific scenarios, carrier frequency, with some assumptions of RMS delay spread and UE velocity.

RAN4 to study if the same channel model can be defined for all carrier frequencies/ bands in given frequency range – example FR1/ FR2.

RAN4 to study if the CDL channel models can be simplified to smaller number of clusters.

Comparison of methodologies

Add current TDL channel model for comparison against new spatial channel models to compare performance and identify limitations if any with existing methodology.

# Reference

- R4-2419782, “Way Forward for [113][320] NR_SCM”, Nokia.

- R4-2500263, “Simulation results for Spatial Channel Modeling study”, Apple