3GPP TSG-RAN WG4 Meeting #114		R4-2500269

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.22.3.2

Source:	Apple

Title:	On RRM Requirements for MIMO Phase 5

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#113 the other RRM requirements impact for MIMO phase 5 was discussed and WF [1] was approved. In this contribution we discuss RRM requirements impact for objectives other than UE initiated beam management.

# 2. Discussion

UE Reporting enhancement for CJT calibration

Issue 2-2: Whether RRM performance requirements impacts exist by UE reporting enhancement for CJT calibration?

Way forward:

- Option 1: Define accuracy requirements.

- Option 2: FFS.

RAN4 shall discuss the necessity and feasibility to introduce accuracy requirements in performance part of WI.

RAN4 to discuss feasibility of accuracy requirements in performance part.

Issue 2-3: Whether report mapping tables should be specified by UE reporting enhancement for CJT calibration?

Way forward:

- Option 1: Define mapping tables for frequency/delay/phase offset reporting in performance part.

RAN4 could potentially define mapping tables for the delay quantities of not covered in RAN1 spec in performance part.

RAN4 to consider defining mapping tables for frequency/delay/phase offset reporting in performance part of WI.

Asymmetric DL sTRP/UL mTRP

Issue 4-1: How to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

In the last meeting (RAN4#112-bis) RAN4 agreed to update the RRM requirements for 2TA enhancement:

Issue 4-1: Whether RRM impacts exist by 2TA enhancement? Agreement: - RAN4 to update the RRM requirements for 2TA enhancement [for asymmetric DL sTRP/ UL mTRP] - As baseline, the Rel-18 framework can be considered as starting point, and the update is needed to accommodate the new scenario in this WID. - Further discuss the details in the next meeting.

Way forward:

- Alt 1: Define requirement for 2TA following the legacy principle.

- Alt 2: Wait for RAN1.

In Rel-18 RAN4 introduced timing requirements for 2TA for multi-DCI multi-TRP transmission scheme. Impacts for timing requirements for 2 TA with asymmetric DL sTRP/ UL mTRP needs to be further discussed in RAN4. In Rel-18 with 2 TA there was a reference point for each TAG associated with the DLorJoint TCI state with same TAG as UL signal. With asymmetric DL sTRP/ UL mTRP there is only 1 DL TRP, hence only a common reference point is possible for both TAGs.

- With asymmetric DL sTRP/ UL mTRP only 1 DL TRP and only common reference point is feasible for this case.

RAN4 to update the reference point definition for 2 TA with asymmetric DL sTRP/ UL mTRP.

In Rel-18 MIMO evo, for timing requirements with 2 TA, the gradual timing adjustment requirements were applicable to each TAG. In case of asymmetrical DL sTRP/ UL mTRP, there is only 1 DL/UL TRP with corresponding reference timing. Hence the gradual timing adjustment requirements shall be applicable only to TRP with both UL and DL.

The gradual timing adjustment requirements are only applicable to TRP with both UL and DL

Issue 4-2: Whether to define UL TCI state activation requirement based on SRS?

Way forward:

- Option 1: Not define requirement (Samsung, HW, vivo)

- Option 2: RAN4 to define UL TCI state activation requirement based on SRS. (Xiaomi, Ericsson, Nokia)

- Option 3: Further discuss based on the feasibility of defining requirement and testing (QC, Apple, MTK)

For UL TCI state switching in R17, the components of delay are MAC CE decoding timing, PL-RS measurement for PL-RS not maintained, L1-RSRP measurement time for unknown TCI state. For SRS based TCI state switch would only be based on MAC-CE decoding time. For UL only TRP, it would be PL-offset to be applied to reference PL from DL TRP. There is no known/unknown condition for SRS beam.

- For SRS based TCI state switch for UL only TRP, the delay would only be MAC-CE decoding time.

Also, the feasibility to test SRS based TCI state switch without a DL beam is unclear and were not defined sine R16 UL spatial relation info switch.

- Feasibility to test SRS based TCI state switch is unknown.

Hence, the usefulness and feasibility of defining SRS based TCI state is questionable.

RAN4 to discuss usefulness and feasibility of defining SRS based UL TCI state switch requirements since they were not introduced since Rel-16.

Issue 4-3: RRM core impacts of active uplink TCI state switching delay for unified TCI?

Way forward:

- Option 1: No RRM impacts on UL TCI state switching delay requirement for unified TCI states.

- Option 2: RAN4 to specify UL TCI state switching requirements.

- Option 3: FFS.

We support not to introduce any requirements for SRS based TCI state switch, hence we don’t see any impact to UL TCI state switching delay requirements.

No RRM impacts on UL TCI state switching delay requirement for unified TCI states.

# 3. Conclusion

In this paper, we provide our views on impact to RRM requirements from UE initiated beam management. Our observations and proposals are captured below:

UE Reporting enhancement for CJT calibration

- RAN4 to discuss feasibility of accuracy requirements in performance part.

RAN4 to consider defining mapping tables for frequency/delay/phase offset reporting in performance part of WI.

Asymmetric DL sTRP/UL mTRP

- With asymmetric DL sTRP/ UL mTRP only 1 DL TRP and only common reference point is feasible for this case.

RAN4 to update the reference point definition for 2 TA with asymmetric DL sTRP/ UL mTRP.

The gradual timing adjustment requirements are only applicable to TRP with both UL and DL

- For SRS based TCI state switch for UL only TRP, the delay would only be MAC-CE decoding time.

- Feasibility to test SRS based TCI state switch is unknown.

RAN4 to discuss usefulness and feasibility of defining SRS based UL TCI state switch requirements since they were not introduced since Rel-16.

No RRM impacts on UL TCI state switching delay requirement for unified TCI states.

# Reference

- R4-2420108, “WF on RRM requirements for NR_MIMO_Ph5_Part2”, MediaTek.