3GPP TSG- WG4 Meeting # 114	R4-2500282

# Athens, Greece, February 17 – 22, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-19 EESS

Agenda Item:	6.22.2

Document for:	Approval

Introduction

In RAN#105, the new WID on mmWave UE spurious emission was approved with the following objectives.

The objective of this WI includes the following aspects: - For UE, specify additional spurious emission requirements of -5dBm/200Mz for 23.6-24 GHz frequency range for n257 and n258, where - Introduce new corresponding Network Signalling(s) and associated A-MPR(s) if needed by taking into the consideration of the relationship with the existing NS_200, NS_202, NS_203, CA_NS_200, CA_NS_202 and CA_NS_203 - All the power classes are considered in the above work - For BS, conduct necessary modifications on protection of Earth Exploration Satellite Service requirements if any

In RAN4#113, the WF [1] was approved to introduce NS_205 for the limit of -5dBm/200 MHz in the 23.6-24.0 GHz frequency range for n257 and n258, and to update NS_202.

This paper focus on the leftover part, including, the A-MPR for band n257 and n258 CA.

Discussion

First of all, considering in frequency range 2, non-contiguous allocation per component carrier for UL RB allocation for CP-OFDM is not supported [2], the WF[1] should be updated by

<Way forward for A-MPR >: For NS_205: - For PC1: 7.0 dB if offset frequency < BWchannel, 6.0 dB otherwise - For other PCs: 2.0 dB if offset frequency < BWchannel, 0.0 dB otherwise The Offset frequency is defined as the frequency from the upper edge of the protected frequency range to the lower edge of the channel bandwidth. For CA_NS_205 - For PC1: For contiguous UL CA with single contiguous allocation, [9.0 dB] if offset frequency < BWintraCA, 6.0 dB otherwise. For NC UL CA, [13.0 dB] if offset frequency < BWintraCA, 6.0 dB otherwise. - For other PC: For contiguous UL CA with single contiguous allocation, [5.0 dB] if offset frequency < BWintraCA, 0.0 dB otherwise. For NC UL CA, [8.0 dB] if offset frequency < BWintraCA, 0.0 dB otherwise. The Offset frequency is defined as the frequency from the upper edge of the protected frequency range to the lower edge of the lowest CC among the configured UL CA. <Way forward > Updated NS_202 by replacing +1 dBm/200 MHz with -5dBm/200MHz EESS passive services protection in the 23.6GHz-24GHz for band n257 and n258, and define modifiedMPRbehaviour bit to differentiate between legacy NS_202 and new NS_202 requirements. - It is mandatory to report modifiedMPRbehaviour bit from Rel-15 for new UEs. - A-MPR for updated NS_202 - For PC1: 11.0 dB - For other PCs: 2.0 dB The Offset frequency is defined as the frequency from the upper edge of the protected frequency range to the lower edge of the channel bandwidth. - A-MPR for updated CA_NS_202 - For PC1: For contiguous UL CA with contiguous allocation, 11.0 dB For NC UL CA, [13.0 dB] if offset frequency < BWintraCA, 11.0dB otherwise. - For other PCs: For contiguous UL CA with contiguous allocation, [5.0 dB] if offset frequency < BWintraCA, 2.0dB otherwise. For NC UL CA, [8.0 dB] if offset frequency < BWintraCA, 2.0dB otherwise. The Offset frequency is defined as the frequency from the upper edge of the protected frequency range to the lower edge of the lowest CC among the configured UL CA.

Observation 1: Non-contiguous allocation per component carrier for UL RB allocation for CP-OFDM is not supported for FR2.

Proposal 1: Contiguous UL CA with non-contiguous RB allocation should not be considered for FR2.

For intra-band non-contiguous CA, the only uplink CA configuration for band n258 is CA_n258G. And the bandwidth class G is kind of contiguous CA configuration. Therefore, the A-MPR defined for band n258 non-contiguous CA is not required.

Excerpt Table 5.5A.2-1: NR CA configurations with single CA bandwidth class defined for intra-band non-contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | (BWChannel,block) (MHz) | BCS |
| CA_n257(2A) | - | 800 | 0 |
| CA_n257(2G) | CA_n257(2G) | 400 | 0 |
| CA_n258(2A) | - | 800 | 0 |
| CA_n258(3A) | - | 1200 | 0 |
| CA_n258(4A) | - | 1600 | 0 |
| CA_n258(5A) | - | 2000 | 0 |
| CA_n258(2G) | CA_n258G | 400 | 0 |

Observation 2: The uplink CA configuration defined for CA_n258 is contiguous.

Proposal 2: The A-MPR defined for band n258 non-contiguous CA is not required.

Excerpt Table 5.3A.4-1: CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4,5 |
| G | 100 MHz < BWChannel_CA ≤ 200 MHz | 2 | 3 |

For CA_n257(2G), the minimum aggregated channel bandwidth per component carrier is 100MHz. It should be considered when defining the A-MPR for band n257.

Conclusion

In this contribution we discussed on FR2 EESS protection emission requirement, according to the analysis, we have the following observations and proposals:

Observation 1: Non-contiguous allocation per component carrier for UL RB allocation for CP-OFDM is not supported for FR2.

Proposal 1: Contiguous UL CA with non-contiguous RB allocation should not be considered for FR2.

Observation 2: The uplink CA configuration defined for CA_n258 is contiguous.

Proposal 2: The A-MPR defined for band n258 non-contiguous CA is not required.

- References

[1] R4-2420356, WF on UE RF requirements for EESS protection, Huawei, China Unicom, RAN4#113

[2] TS38.214, sub-clause 6.1.2.2.1