3GPP RAN WG4 Meeting #114																	R4-2500300

Athens, Greece, 17th – 21st February 2025

Agenda item:	7.10.3.2

Source:	Apple

Title:	Power back-off values for the 3MHz channel in band n256

WI/SI:	NR_IoT_NTN_req_test_enh

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications.

After the RAN#104 meeting a new WI was agreed [2], one of the aims of which is to enable the 3MHz channel for the NTN bands including the band n256. During the previous RAN WG4 meetings initial power back-off simulation results were submitted to for technical discussions and were captured in [3]; however, there was no final conclusion on which A-MPR regions should be defined with associated power back-off values. Thus, in this discussion paper we present UE power back-off simulation results for the 3MHz channel in the NTN band n256 frequency range.

# 2	Power back-off for band n256

## 2.1	Simulation assumptions and scenarios

The satellite S-band (n256) was standardised already in Rel-17, whereupon the smaller considered channel bandwidth was 5MHz. Here we present power back-off results for the PC3 power class and the 3MHz channel with the following common assumptions:

-	Tx power: PC3 (+23dBm)

-	LO placement: always in the centre of the carrier

-	Regulations: NS_24

The power back-off simulations were conducted for the following center frequencies:

-	the right-most channel with the 5MHz offset, as "guard" band, from the upper edge of the band (Fc=2003.5MHz)

-	the upper channel with the 6.5MHz offset from the upper edge (Fc=2002MHz)

-	the upper channel with the 8MHz offset from the upper edge (Fc=2000.5MHz)

-	the upper channel with the 9.5MHz offset from the upper edge (Fc=1999MHz)

## 2.2	Power back-off simulations

Figures 2.2-1, 2.2-2 and 2.2-3 below present the power back-off simulation results for the 3MHz channel at three different locations as mentioned in the section 2.1. As anticipated, the upper most channel has the highest power back-off values. For instance, the full channel allocation of 15RB might require 6dB power back-off. And as the channel shifts to the left from the upper edge of the band, the power back-off decreases as well.

Figure 2.2-1: Power back-off results for NS_24 (Fc=2003.5MHz).

Figure 2.2-2: Power back-off results for NS_24 (Fc=2002MHz).

Figure 2.2-3: Power back-off results for NS_24 (Fc=2000.5MHz).

Figure 2.2-4: Power back-off results for NS_24 (Fc=1999MHz)

## 2.3	Summary and proposed A-MPR regions

Based on the power back-off simulations we suggest defining A-MPR regions for the 3MHz channel as presented below in Table 2.3-1 and 2.3-2.

Table 2.3-1: A-MPR regions for the band n256 3MHz channel

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region |  |  |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS | LCRB*12*SCS | A-MPR |
| 3MHz | 2002 < fc <= 2003.5 |  | > 1.08 | A1 |
|  |  | >= 2.52 | <= 1.08 > 0.72 | A2 |
|  | 2000.5 < fc <= 2002 |  | >= 1.44 | A3 |
|  | 1999 < fc <= 2000.5 |  | >= 1.8 | A4 |
|  | 1997.5 < fc <= 1999 |  | >= 2.52 | A2 |

Table 2.3-2: A-MPR values for the band n256 3MHz channel

|  | Modulation | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 6.0 | 1.0 | 3.5 | 2.0 |
|  | QPSK | 6.5 | 1.5 | 4.0 | 2.5 |
|  | 16QAM | 7.0 |  | 4.5 | 3.0 |
|  | 64QAM | 7.0 |  | 4.5 |  |
|  | 256QAM | 7.0 |  | 4.5 |  |
| CP-OFDM | QPSK | 9.0 |  | 6.5 | 5.0 |
|  | 16QAM | 9.0 |  | 6.5 | 5.0 |
|  | 64QAM | 9.0 |  | 6.5 | 5.0 |
|  | 256QAM | 9.0 |  | 6.5 |  |

Proposal 1:	Define A-MPR for the 3MHz channels at the upper edge of the NTN band n256 (1997.5 < Fc <= 2003.5MHz)

## 2.4	Other considerations

When the NTN band n256 was initially introduced to the Rel-17 core specifications it was agreed to leverage an existing NS_24 flag, A-MPR values for which are captured in TS 38.101-1. In other words, current definition of band n256 uses NS_24 with references to the terrestrial TS 38.101-1 specification. However, NS_24 does not have the 3MHz channel and there are no ongoing activities either to enable the 3MHz channel for the corresponding terrestrial bands. Thus, it should be discussed further how to solve this issue.

Our current view is that 3GPP can re-name NS_24 to e.g. NS_24N, definition of which would be contained in TS 38.101-5 and include power back-off values for the 3MHz channel.

Proposal 2a:	Specify 3MHz power back-off values for band n256 considering renaming NS_24 to NS_24N with definition of this flag in TS 38.101-5.

It is usually the case that if a band has additional emission and/or regulatory restrictions, the corresponding findings are captured in a TR, including the power back-off simulation results from all contributing companies. In case of the NTN band n256, the corresponding considerations were captured in TR 38.863, sub-clause 7.4.2.2.2.2. Thus we can consider leveraging the same TR to capture additional conclusions on the 3MHz channel bandwidth.

Proposal 2b:	Consider capturing the 3MHz power back-off results with related considerations in TR 38.863.

# 3	Conclusions

In this discussion paper we have presented our further results for the power back-off values for the 3MHz channel operating in the NTN band n256 frequency range. As can be seen from the results, a smaller 3MHz channel needs lower power back-off when compared to the 5MHz channel. Nevertheless, some of the 3MHz channel allocations still needs A-MPR when it is configured at the upper edge of the band.

Proposal 1:	Define A-MPR for the 3MHz channels at the upper edge of the NTN band n256 (1997.5 < Fc <= 2003.5MHz)

Proposal 2a:	Specify 3MHz power back-off values for band n256 considering renaming NS_24 to NS_24N with definition of this flag in TS 38.101-5.

Proposal 2b:	Consider capturing the 3MHz power back-off results with related considerations in TR 38.863.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"

- R4-2419789, "Way Forward for [113][311] NR_IoT_NTN_less_than_5MHz_UERF", Xiaomi