3GPP TSG-RAN WG4 Meeting #114	R4-2500303

# Athens, Greece,  17th February – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.741 | CR | draft | rev | < # > | Current version: | 18.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of the 3MHz channel to TR 38.741 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Globalstar |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Core, NR_NTN_LSband-Core |  |  |  |  |  | Date: |  |  | 2025-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When the satellite band n254 was introduced in Rel-18, the smallest channel bandwidth defined for NTN was 5MHz. With introduction of the 3MHz channel for the NTN core functionality, the corresponding changes have to be added to TR 38.741. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 3MHz channel is added to the satellite band n254 impacting the following aspects in the existing TR: - available channel bandwidths - applicable SS raster entries - asymmetric band combinations - Tx-Rx separation distance - power back-off values - reference sensitivity |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 3MHz channel will be missing in TR 38.741. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 6.2.1.2.2, 6.2.1.2.2.2.0(new), 6.2.1.2.4a(new), 6.2.1.2.5, 6.2.1.3, 6.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This is a draft running CR that presents anticipated changes to TR 38.741 needed for the 3MHz channel bandwidth. Some values are TBD subject to further changes based on the outcome of the RAN WG4 discussions. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 6	NR

## 6.1	Band plan and system parameters

Table 6.1-1: NTN L-/S-band n254

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n254 | 1610 – 1626.5 MHz | 2483.5 – 2500 MHz | FDD |
| NOTE: NTN satellite bands are numbered in descending order from n256. |  |  |  |

Table 6.1-2: NTN L-/S-band n254 channel bandwidths

| NTN satellite band | SCS kHz |  | UE Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 |  |
| n254 | 15 | 3 | 5 | 10 | 15 |  |
|  | 30 |  |  | 10 | 15 |  |
|  | 60 |  |  | 10 | 15 |  |

Table 6.1-3: NTN L-/S-band n254 applicable NR-ARFCN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n254 | 100 | 322000 – <20> – 325300 | 496700 – <20> – 500000 |
|  | 10 | 322000 – <2> – 325300 | 496700 – <2> – 500000 |
| NOTE: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. |  |  |  |

Table 6.1-4: NTN L-/S-band n254 applicable SS raster entries (for above 3MHz channel)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n254 | 15kHz | Case A | 6215 – <1> – 6244 |
|  | 30 kHz | Case C | 6218 – <1> – 6241 |
| NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

Table 6.1-4a: NTN L-/S-band n254 applicable SS raster entries (for the 3MHz channel)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n254 | 15kHz | Case A | 39059 – <1> – 39134 |
| NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

Table 6.1-5: NTN L-/S-band asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n254 | 3 | 5, 10, 15 | 0 |
|  | 5 | 10,15 | 0 |
|  | 10 | 15 | 0 |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated in TS38.306 [15], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. |  |  |  |

Table6.1-6: NTN L-/S-band UE TX-RX frequency separation

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n254 | 864 – 887 MHz |

******************* NEXT CHANGED SECTION *******************

##### 6.2.1.2.2	Additional maximum power reduction (company #1)

Maximum power reduction for the NTN L-/S-band is the same as for other NTN bands, such as L-band n255 and S-band n256.

6.2.1.2.2.1		Configuration cases

For the power back-off simulations the following common parameters are assumed:

-	Tx power: PC3 +23dBm

-	LO placement: always in the centre of the carrier

-	Regulations: ETSI and FCC

The following channel sizes with centre frequencies are considered:

-	NR 3MHz channel at the following centre frequencies:

a)	Fc=1611.5MHz

b)	Fc=1612.9MHz

c)	Fc=1623.4MHz

d)	Fc=1625MHz

-	NR 5MHz channel at the following centre frequencies:

a)	Fc=1612.5MHz

b)	Fc=1613.9MHz

c)	Fc=1615.5MHz

d)	Fc=1620.5MHz

e)	Fc=1622.4MHz

f)	Fc=1624MHz

-	NR 10MHz channel at the following centre frequencies:

a)	Fc=1615MHz.

b)	Fc=1620MHz

c)	Fc=1621.5MHz

-	NR 15MHz channel at the following centre frequencies:

a)	Fc=1617.5MHz

b)	Fc=1619MHz

Figure 6.2.1.2.2.1-1: Considered channels for A-MPR simulations.

6.2.1.2.2.2	Power back-off results for ETSI

###### 6.2.1.2.2.2.0	3MHz channel

Figure 6.2.1.2.2.2.0-1: Power back-off values for the 3MHz channel at Fc=1611.5MHz.

Figure 6.2.1.2.2.2.0-2: Power back-off results for the 3MHz channel at Fc=1612.9MHz.

Figure 6.2.1.2.2.2.0-3: Power back-off results for the 3MHz channel at Fc=1623.4MHz

Figure 6.2.1.2.2.2.0-4: Power back-off results for the 3MHz channel at Fc=1625MHz.

6.2.1.2.2.2.1	5MHz channel

******************* NEXT CHANGED SECTION *******************

##### 6.2.1.2.4a	Additional maximum power reduction (company #4)

This sub-clause contains 3MHz channel simulations at Fc=1611.5MHz center frequency, where the PA is calibrated based on the following assumptions:

-	PC3

-	1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB

-	Carrier Leakage: 28dBc, Image: 28dBc, CIM3: 60dBc, CIM5: 70dBc

-	ASE and A-SEM, SEM and ACLR are considered as gating factors (EVM was excluded)

Figure 6.2.1.2.4a-1: Power back-off values for the 3MHz channel at Fc=1611.5MHz

******************* NEXT CHANGED SECTION *******************

##### 6.2.1.2.5	Harmonised A-MPR values

Tables 6.2.1.2.5-0 - 6.2.1.2.5-3 below present "harmonised" A-MPR values for 3, 5, 10 and 15MHz channels at reference frequencies considered for the power back-off simulations and measurements. For the sake of simplicity only "full" RB allocation cases are used to compare results from different sources.

Table 6.2.1.2.5-0: Harmonised A-MPR values for the 3MHz channel.

|  | 3MHz channel center frequency |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1611.5MHz |  |  | 1612.9MHz |  |  | 1623.4MHz |  |  | 1625MHz |  |  |
|  | CP QP | DFT |  | CP QP | DFT |  | CP QP | DFT |  | CP QP | DFT |  |
|  |  | QP | PI/2 |  | QP | PI/2 |  | QP | PI/2 |  | QP | PI/2 |
|  | ETSI regulatory domain |  |  |  |  |  |  |  |  |  |  |  |
| #1 | 3.5 | 1.5 | 1.0 |  |  |  |  |  |  |  | 1.5 | 1.0 |
| #4 | 4.5 | 3.5 |  |  |  |  |  |  |  |  |  |  |
| => | [TBD] | [TBD] | [TBD] |  |  |  |  |  |  | [TBD] | [TBD] | [TBD] |
|  | FCC regulatory domain |  |  |  |  |  |  |  |  |  |  |  |
| #1 | 3.5 | 1.5 | 1.0 |  |  |  |  |  |  |  |  |  |
| #4 | 4.5 | 3.5 |  |  |  |  |  |  |  |  |  |  |
| => | [TBD] | [TBD] | [TBD] |  |  |  |  |  |  |  |  |  |

Table 6.2.1.2.5-1: Harmonised A-MPR values for the 5MHz channel.

|  | 5MHz channel center frequency |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1612.5MHz |  |  | 1613.9MHz |  |  | 1615.5MHz |  |  | 1620.5MHz |  |  | 1622.4MHz |  |  | 1624MHz |  |  |
|  | CP QP | DFT |  | CP QP | DFT |  | CP QP | DFT |  | CP QP | DFT |  | CP QP | DFT |  | CP QP | DFT |  |
|  |  | QP | PI |  | QP | PI |  | QP | PI |  | QP | PI |  | QP | PI |  | QP | PI |
|  | ETSI regulatory domain |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #1 | 6 | 3.5 | 2.5 | 4 | 2.5 | 1 | 3 | 1 | 0.5 | 3 | 1 | 0.5 | 3 | 1 | 1 | 3 | 1.5 | 1.5 |
| #2 | 6 | 4 | 3 | - | - | - | 3 | 1.5 | 0.1 | 3 | 0.8 | - | - | - | - | 3 | 1.2 | 0.7 |
| #3 | 5.5 | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| => | 6 | 4 | 3 | 4 | 2.5 | 1 |  |  |  |  |  |  | 3 | 1 | 1 | 3 | 1.5 | 1.5 |
|  | FCC regulatory domain |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #1 | 6 | 3.5 | 2.5 | 4 | 2.5 | 1 | 3 | 1 | 0.5 | 3 | 1 | 0.5 |  |  |  | 3 | 1 | 0.5 |
| #3 | 5.5 | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| => | 6 | 4 | 3 | 4 | 2.5 | 1 |  |  |  |  |  |  |  |  |  | 3 | 1 | 0.5 |

Table 6.2.1.2.5-2: Harmonised A-MPR values for the 10MHz channel.

|  | 10MHz channel center frequency |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1615MHz |  |  | 1620MHz |  |  | 1621.5MHz |  |  |
|  | CP-OFDM QPSK | DFT-s-OFDM |  | CP-OFDM QPSK | DFT-s-OFDM |  | CP-OFDM QPSK | DTT-s-OFDM |  |
|  |  | QPSK | PI/2 BPSK |  | QPSK | PI/2 BPSK |  | QPSK | PI/2 BPSK |
|  | ETSI regulatory domain |  |  |  |  |  |  |  |  |
| #1 | 12.5 | 7 | 6 | 4 | 3 | 2 | 4 | 2.5 | 2 |
| #2 | 7.5 | 5 | 3.5 |  |  |  | 4.5 | 2.5 | 2 |
| #3 | 7 | 5 |  |  |  |  |  |  |  |
| => | 9 | 6 | 4 | 4.5 | 3 | 2 | 4.5 | 2.5 | 2 |
|  | FCC regulatory domain |  |  |  |  |  |  |  |  |
| #1 | 12.5 | 7 | 6 | 4 | 2 | 1.5 | 3 | 2 | 1.5 |
| #3 | 7 | 5 |  |  |  |  |  |  |  |
| => | 9 | 6 | 4 | 4 | 2 | 1.5 | 3 | 2 | 1.5 |

Table 6.2.1.2.5-3: Harmonised A-MPR values for the 15MHz channel.

|  | 15MHz channel center frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1617.5MHz |  |  | 1619MHz |  |  |
|  | CP-OFDM QPSK | DTF-s-OFDM |  | CP-OFDM QPSK | DTF-s-OFDM |  |
|  |  | QPSK | PI/2 BPSK |  | QPSK | PI/2 BPSK |
|  | ETSI regulatory domain |  |  |  |  |  |
| #1 | 13.5 | 11.5 | 11.5 | 11 | 9.5 | 7.5 |
| #2 | 7.5 | 5 | 6.5 | 7 | 4.5 | 3.5 |
| #3 | 8.5 | 5.5 |  |  |  |  |
| => | 10 | 7 | 6.5 | 9 | 6 | 5.5 |
|  | FCC regulatory domain |  |  |  |  |  |
| #1 | 13.5 | 9.5 | 9 | 11 | 8.5 | 6 |
| #3 | 8.5 | 5.5 |  |  |  |  |
| => | 10 | 7 | 6.5 | 9 | 6 | 5.5 |

#### 6.2.1.3	Emission requirements and NS values

There are three additional NS values defined for this band as summarised in Table 6.2.1.3-1 below:

a)	NS_03N is for the FCC regulatory domain. Since FCC does not introduce any special in-band emission requirements for sub-ranges, same requirements will be applicable for the whole range and for all the channel sizes.

b)	ETSI regulations have different in-band emission masks depending on the operating sub-range, so two NS flags are needed to differentiate between them.

c)	NS_04N applicable range is limited to 1610-1618.25MHz according to the ETSI requirements. And since the total spectrum block size is 8.25MHz, it can accommodate 3MHz and 5MHz standard NR channels.

d)	NS_04N is for the ETSI range 1618.25-1626.5MHz, which can also accommodate only 3MHz and 5MHz standard NR channel. However, ETSI document also clarifies that the same in-band emission requirements will apply if the channel is partially configured in that range. It means that any 10 and 15MHz channel configured in 1610-1626.5MHz range will inevitably fall into the 1618.25-1626.5MHz range and thus they will follow NS_04N requirements.

Table 6.2.1.3-1: NTN L-/S-band NS values

| Regulatory domain | NS value | Applicable range (MHz) | Applicable channel (MHz) |
| --- | --- | --- | --- |
| FCC | NS_03N | 1610-1626.5 | 3, 5, 10, 15 |
| ETSI | NS_04N | 1610-1618.25 | 3, 5 |
|  | NS_05N | 1618.25-1626.5 | 3, 5 |
|  |  | 1610-1626.5 | 10, 15 |

Mapping between values of additionalSpectrumEmission and NS values is given in the Table 6.2.3.1-2 below.

Table 6.2.1.3-2: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n254 | NS_01 | NS_03N | NS_04N | NS_05N |  |  |  |  |

******************* NEXT CHANGED SECTION *******************

### 6.2.2	UE receiver characteristics

#### 6.2.2.1	Reference sensitivity

For band n254, band n53 REFSENS is re-used with additional 0.5dB insertion loss. The corresponding band n254 REFSENS values are presented in Table 6.2.2.1-1 below.

Table 6.2.2.1-1: NTN L-/S-band two antenna port reference sensitivity QPSK PREFSENS

|  | Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band |  | SCS kHz | 3 MHz (dBm | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n254 |  | 15 | [-101.2] | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  |  | 60 |  |  | -97.0 | -94.9 |  |  |  |  |  |  |  |