3GPP TSG-RAN WG4 Meeting #114	R4-2500336

Athens, Greece, 17th Feb 2025 – 21st Feb 2025

Agenda item:	7.16.2

Source:	MediaTek Inc.

Title:	Simulation results of Spatial Channel Modelling

Document for:	Discussion

# Introduction

In RAN4#113, RAN4 agreed set of alignment simulations for Spatial Channel Modelling [1]. In this contribution, we provide our simulation results for Spatial Channel Modelling.

# Simulation results

## 2.1 Simulation parameters

In this Chapter, we provide our simulation setup and results for Spatial Channel Modelling. In RAN4#113, RAN4 agreed that general simulation configuration can follow existing 38.101-4 8RX TDD requirement configuration specified in Chapter 5.2.4.2.1 [2]. Common parameters are listed in Table 1. We have assumed 0% TxEVM and LMMSE MIMO demodulation.

Table 1: Common test parameters of 4RX and 8RX tests (Table 5.2.4.2.1-2 in [2])

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 for rank <= 4 2 for rank > 4 |
| Codebook configuration | CodebookType |  | typeI-SinglePanel |
|  | CodebookMode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) for 4Tx (4,1) for 8Tx |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Test specific MCS with common system bandwidth, subcarrier spacing, and TDD UL-DL pattern are listed in Table 2. Note that demodulation tests use only Ranks 4 and 8, whereas PMI tests use only Ranks 2 and 4. Also note that PDSCH data is allocated only in full slots, i.e. not is special slots in our simulations.

Table 2: Test parameters of 4RX and 8RX demodulation and PMI tests

| Test | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Test type |
| --- | --- | --- | --- | --- |
| Rank 2 | 40 / 30 | 16QAM, 0.48 (MCS13) | FR1.30-1 | PMI |
| Rank 4 | 40 / 30 | 16QAM, 0.48 (MCS13) | FR1.30-1 | Demodulation, PMI |
| Rank 8 | 40 / 30 | 16QAM, 0.48 (MCS13) | FR1.30-1 | Demodulation |

Our CDL channel model implementation strictly follows TR38.827. CDL channel parameters are listed in Table 3 for general parameters and in Table 4 for transmit beamforming related parameters. The concept of antenna array virtualization (AAV) via TX beamforming is illustrated in Figure 1. For CDL channel we have simulated 4 different AAV options. 2 different subarray virtualizers have been simulated for Fixed array size AAV option. These options were listed in previous WF [1]. Note that for demodulation tests UE speed is always 30 km/h, whereas in PMI tests both UE speeds 3 and 30 km/h are simulated.

Table 3: CDL channel parameters

| Carrier frequency | 3.5 GHz |
| --- | --- |
| UE speed | 3 or 30 km/h |
| UE travelling direction (fv, qv) | (65°, 90o) |
| Channel profile | 827 UMa CDL-C |
| Antenna array virtualization for 4RX | - No AAV (1, 2, 2, 1, 1) (Direct passthrough / no AAV) - AAV (8, 2, 2, 8, 1) (Fixed subarray size, Opt Y) - AAV (8, 8, 2, 8, 4) (Fixed array size, Opt X) - AAV by 827 (Fully connected) |
| Antenna array virtualization for 8RX | - No AAV (1, 4, 2, 1, 1) (Direct passthrough / no AAV) - AAV (8, 4, 2, 8, 1) (Fixed subarray size, Opt Y) - AAV (8, 8, 2, 8, 2) (Fixed array size, Opt X) - AAV by 827 (Fully connected) |
| Subarray virtualizer for Fixed subarray size | - Broadside, i.e. all ones |
| Subarray virtualizer for Fixed array size | - Broadside, i.e. all ones - The 1st strongest direction |
| BS (α, β, γ) | (0°, 10°, 0°) |
| BS polarization slant | (45°, -45o) |
| UE (α, β, γ) | (180°, 0°, 0°) |
| UE polarization slant | (0°, 90o) |
| BS antenna radiation pattern | As defined in TR38.901 Table 7.3-1 |
| UE antenna radiation pattern | Omnidirectional |

Table 4: CDL transmit beam directions for Option B 827 AAV without subarray

| The strongest beam | Ranks 2, 4, 8 | AOD: -7.27° | ZOD: 100° |
| --- | --- | --- | --- |
| The 2nd strongest beam | Ranks 2, 4 | AOD: -21.82° | ZOD: 100° |
| The 3rd strongest beam | Rank 8 | AOD: -36.36° | ZOD: 100° |
| The 4th strongest beam | Rank 8 | AOD: -50.91° | ZOD: 100° |

Figure 1: Antenna array virtualization (AAV) concept

## 2.2 PDSCH demodulation testing

In Figure 2 we show PDSCH demodulation performance of Rank 4 MCS13 using random precoding.

In Figure 3 we show PDSCH demodulation performance of Rank 8 MCS13 using random precoding. On the left is total throughput and on the right is codeword specific throughputs.

In Figure 4 we show PDSCH demodulation performance of Rank 8 MCS13 using fixed precoding. On the left is total throughput and on the right is codeword specific throughputs.

Figure 2: Simulation results of 4RX TDD Rank 4 with random precoding.

Figure 3: Simulation results of 8RX TDD Rank 8 with random precoding
Left: Total throughputs, Right: CW specific throughputs

Figure 4: Simulation results of 8RX TDD Rank 8 with fixed precoding
Left: Total throughputs, Right: CW specific throughputs

In Table 5 and Table 6 we list required SNR in each test to achieve 70% relative throughput in 4RX TDD Rank 4 and 8RX TDD Rank 8 using random precoding.

In Table 7 we list required SNR in each test to achieve 70% relative throughput in 8RX TDD Rank 8 using the best fixed precoding for each channel.

Table 5: Simulation results of 4TX-4RX TDD Rank 4 with random precoding

| CDL-C-UMa | Rank 4 |
| --- | --- |
|  | MCS13 |
| no AAV (1,2,2,1,1) | 17.5dB |
| Opt Y / AAV (8,2,2,8,1) / Broadside | 17.8dB |
| Opt X / AAV (8,8,2,8,4) / Broadside | 16.6dB |
| Opt X / AAV (8,8,2,8,4) / 1st strongest beam | 14.3dB |
| AAV by 827 TX-BF (2 beams) | 14.1dB |

Table 6: Simulation results of 8TX-8RX TDD Rank 8 with random precoding

| CDL-C-UMa | Rank 8 MCS13 |  |  |
| --- | --- | --- | --- |
|  | Total | CW0 | CW1 |
| no AAV (1,4,2,1,1) | 21.8dB | 19.4dB | 29.6dB |
| Opt Y / AAV (8,4,2,8,1) / Broadside | 22.3dB | 19.7dB | N/A |
| Opt X / AAV (8,8,2,8,2) / Broadside | 16.7dB | 14.3dB | 25.7dB |
| Opt X / AAV (8,8,2,8,2) / 1st strongest beam | 16.3dB | 14.0dB | 24.6dB |
| AAV by 827 TX-BF (4 beams) | 22.4dB | 23.5dB | 21.6dB |

Table 7: Simulation results of 8TX-8RX TDD Rank 8 with fixed precoding

| CDL-C-UMa | Rank 8 MCS13 |  |  | Precoder [i1,1, i1,2, i1,3, i2] |
| --- | --- | --- | --- | --- |
|  | Total | CW0 | CW1 |  |
| no AAV (1,4,2,1,1) | 21.4dB | 20.5dB | 22.4dB | [4, 0, 0, 0] |
| Opt Y / AAV (8,4,2,8,1) / Broadside | 21.8dB | 20.2dB | 24.4dB | [4, 0, 0, 0] |
| Opt X / AAV (8,8,2,8,2) / Broadside | 17.8dB | 17.2dB | 18.4dB | [6, 0, 0, 0] |
| Opt X / AAV (8,8,2,8,2) / 1st strongest beam | 17.4dB | 16.5dB | 18.1dB | [6, 0, 0, 0] |
| AAV by 827 TX-BF (4 beams) | 21.1dB | 19.6dB | 24.2dB | [4, 0, 0, 0] |

## 2.3 Follow-PMI testing

In Figures 5-8 we show performance of different PMI reporting options (Type1, eType2, and Random). On the left is Rank 2 with MCS13. On the right is Rank 4 with MCS13. PMI delay is the usual 6.5 ms.

In 827 AAV without subarray (fully connected) in Figure 8 we have also included fixed port selection precoder performance.

Port selection precoder of rank2 is defined as [1 0 0 0 0 0 0 0; 0 0 0 0 1 0 0 0]T.

Port selection precoder of rank4 is defined as [1 0 0 0 0 0 0 0; 0 0 0 0 1 0 0 0; 0 1 0 0 0 0 0 0; 0 0 0 0 0 1 0 0]T.

Figure 5: Simulation result plots of 8TX-4RX PMI reporting with CDL-C-UMa no AAV (Direct passthrough)

Figure 6: Simulation result plots of 8TX-4RX PMI reporting with CDL-C-UMa with AAV Fixed subarray size

Figure 7: Simulation result plots of 8TX-4RX PMI reporting with CDL-C-UMa with AAV Fixed array size

Figure 8: Simulation result plots of 8TX-4RX PMI reporting with CDL-C-UMa with AAV 827 Fully connected

In Table 8 and Table 9 we list required SNR in each test to achieve 90% relative throughput in 8TX-4RX TDD Rank 2 and Rank 4 PMI tests with different PMI reporting options.

In Table 10 and Table 11 we list gammas (relative throughput between follow and random PMI) in each test in 8TX-4RX TDD Rank 2 and Rank 4 PMI tests with different PMI reporting options.

Table 8: SNR of 90% relative tput of 8TX-4RX TDD Rank 2 PMI reporting with different channel models

| CDL-C-UMa | eType2 follow PMI | Type1 follow PMI | Type1 random PMI | eType2 follow PMI | Type1 follow PMI | Type1 random PMI |
| --- | --- | --- | --- | --- | --- | --- |
|  | CDL: 3 km/h |  |  | CDL: 30 km/h |  |  |
| no AAV (1,4,2,1,1) | 3.5dB | 3.6dB | 14.2dB | 4.9dB | 4.6dB | 14.2dB |
| Opt Y / AAV (8,4,2,8,1) / Broadside | 3.4dB | 3.6dB | 14.6dB | 4.6dB | 4.1dB | 14.6dB |
| Opt X / AAV (8,8,2,8,2) / Broadside | 3.4dB | 3.9dB | 13.0dB | 6.2dB | 5.8dB | 13.1dB |
| AAV by 827 TX-BF (4 beams) | 4.9dB | 7.1dB | 8.5dB | 7.2dB | 7.8dB | 8.7dB |

Table 9: SNR of 90% relative tput of 8TX-4RX TDD Rank 4 PMI reporting with different channel models

| CDL-C-UMa | eType2 follow PMI | Type1 follow PMI | Type1 random PMI | eType2 follow PMI | Type1 follow PMI | Type1 random PMI |
| --- | --- | --- | --- | --- | --- | --- |
|  | CDL: 3 km/h |  |  | CDL: 30 km/h |  |  |
| no AAV (1,4,2,1,1) | 14.9dB | 12.8dB | 32.7dB | 16.0dB | 13.8dB | 31.8dB |
| Opt Y / AAV (8,4,2,8,1) / Broadside | 14.4dB | 12.9dB | N/A | 15.8dB | 13.7dB | N/A |
| Opt X / AAV (8,8,2,8,2) / Broadside | 13.0dB | 13.0dB | 20.9dB | 14.8dB | 13.8dB | 21.9dB |
| AAV by 827 TX-BF (4 beams) | 17.8dB | 16.3dB | 18.4dB | 18.8dB | 17.0dB | 18.8dB |

Table 10: Gammas of 8TX-4RX TDD Rank 2 PMI reporting with different channel models

| CDL-C-UMa | eType2 follow PMI | Type1 follow PMI | eType2 follow PMI | Type1 follow PMI |
| --- | --- | --- | --- | --- |
|  | CDL: 3 km/h |  | CDL: 30 km/h |  |
| no AAV (1,4,2,1,1) | 2.4 | 2.4 | 2.0 | 2.1 |
| Opt Y / AAV (8,4,2,8,1) / Broadside | 2.5 | 2.4 | 2.1 | 2.2 |
| Opt X / AAV (8,8,2,8,2) / Broadside | 2.2 | 2.1 | 1.6 | 1.7 |
| AAV by 827 TX-BF (4 beams) | 1.6 | 1.2 | 1.2 | 1.1 |

Table 11: Gammas of 8TX-4RX TDD Rank 4 PMI reporting with different channel models

| CDL-C-UMa | eType2 follow PMI | Type1 follow PMI | eType2 follow PMI | Type1 follow PMI |
| --- | --- | --- | --- | --- |
|  | CDL: 3 km/h |  | CDL: 30 km/h |  |
| no AAV (1,4,2,1,1) | 1.6 | 1.8 | 1.5 | 1.7 |
| Opt Y / AAV (8,4,2,8,1) / Broadside | 1.6 | 1.8 | 1.6 | 1.6 |
| Opt X / AAV (8,8,2,8,2) / Broadside | 1.6 | 1.6 | 1.5 | 1.6 |
| AAV by 827 TX-BF (4 beams) | 1.0 | 1.2 | 1.0 | 1.2 |

## 2.4 SNR normalization factor

Let normalized AAV beamformer vector of length Ms∙Ns (i.e. sub-array size) for one antenna port be

α1,1⋯αMs,Ns=1GBF1Ms∙Nsexp-j2π∙θ1,1 ⋯ exp⁡(-j2π∙θMs,Ns)

Here, amplitude normalization is separated into two factors

- 1Ms∙Ns : split transmit power of each AP evenly over Ms∙Ns transmit antenna elements

- 1GBF : normalize with measured transmit beamforming (power) gain GBF

Measured SNR normalization factors, in dB and linear values, are listed in Tables 12-14.

Table 12: SNR normalization factors of 4TX-4RX

| CDL-C-UMa | GBF |  |
| --- | --- | --- |
|  | dB | Linear |
| no AAV (1,2,2,1,1) | -0.159 | 0.964 |
| Opt Y / AAV (8,2,2,8,1) / Broadside | 7.885 | 6.144 |
| Opt X / AAV (8,8,2,8,4) / Broadside | 10.893 | 12.283 |
| Opt X / AAV (8,8,2,8,4) / 1st strongest beam | 12.222 | 16.682 |
| AAV by 827 TX-BF (2 beams) | 13.138 | 20.596 |

Table 13: SNR normalization factors of 8TX-8RX

| CDL-C-UMa | GBF |  |
| --- | --- | --- |
|  | dB | Linear |
| no AAV (1,4,2,1,1) | -0.162 | 0.963 |
| Opt Y / AAV (8,4,2,8,1) / Broadside | 7.885 | 6.144 |
| Opt X / AAV (8,8,2,8,2) / Broadside | 9.864 | 9.692 |
| Opt X / AAV (8,8,2,8,2) / 1st strongest beam | 10.337 | 10.806 |
| AAV by 827 TX-BF (4 beams) | 10.947 | 12.437 |

Table 14: SNR normalization factors of 8TX-4RX

| CDL-C-UMa | GBF |  |
| --- | --- | --- |
|  | dB | Linear |
| no AAV (1,4,2,1,1) | -0.160 | 0.964 |
| Opt Y / AAV (8,4,2,8,1) / Broadside | 7.884 | 6.144 |
| Opt X / AAV (8,8,2,8,2) / Broadside | 9.863 | 9.690 |
| Opt X / AAV (8,8,2,8,2) / 1st strongest beam | 10.335 | 10.804 |
| AAV by 827 TX-BF (4 beams) | 10.946 | 12.433 |

## 2.5 Eigenmode power distribution

To enable further channel model alignment, we measured the average eigenmode powers of the MIMO channel for the different AAV options. We sampled the MIMO channel frequency response over frequency and time domains over the simulation, and for each Nrx×Ntx MIMO channel sample, singular values (SVD) were calculated and sorted from largest to smallest. Squared singular values were then averaged over the simulation and converted to relative dB values shown in Tables 15, 16, and 17.

Table 15: Average relative eigenmode powers [dB] of 4TX-4RX

| CDL-C-UMa | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| no AAV (1,2,2,1,1) | 0.00 | -4.58 | -12.22 | -19.55 |
| Opt Y / AAV (8,2,2,8,1) / Broadside | 0.00 | -4.62 | -12.95 | -20.21 |
| Opt X / AAV (8,8,2,8,4) / Broadside | 0.00 | -5.16 | -12.72 | -20.24 |
| Opt X / AAV (8,8,2,8,4) / 1st strongest beam | 0.00 | -4.34 | -9.44 | -16.62 |
| AAV by 827 TX-BF (2 beams) | 0.00 | -3.57 | -8.00 | -14.77 |

Table 16: Average relative eigenmode powers [dB] of 8TX-8RX

| CDL-C-UMa | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| no AAV (1,4,2,1,1) | 0.00 | -3.11 | -6.52 | -10.16 | -16.43 | -20.44 | -25.57 | -33.80 |
| Opt Y / AAV (8,4,2,8,1) / Broadside | 0.00 | -3.15 | -6.71 | -10.52 | -17.97 | -22.33 | -27.57 | -35.49 |
| Opt X / AAV (8,8,2,8,2) / Broadside | 0.00 | -2.87 | -5.66 | -9.31 | -14.55 | -19.23 | -25.86 | -33.40 |
| Opt X / AAV (8,8,2,8,2) / 1st strongest beam | 0.00 | -2.64 | -5.29 | -8.64 | -13.60 | -18.30 | -24.97 | -32.67 |
| AAV by 827 TX-BF (4 beams) | 0.00 | -2.34 | -4.83 | -7.79 | -12.50 | -17.48 | -25.24 | -33.59 |

Table 17: Average relative eigenmode powers [dB] of 8TX-4RX

| CDL-C-UMa | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| no AAV (1,2,2,1,1) | 0.00 | -3.78 | -8.22 | -13.80 |
| Opt Y / AAV (8,2,2,8,1) / Broadside | 0.00 | -3.85 | -8.50 | -14.35 |
| Opt X / AAV (8,8,2,8,4) / Broadside | 0.00 | -3.60 | -7.60 | -12.79 |
| AAV by 827 TX-BF (4 beams) | 0.00 | -3.05 | -6.65 | -11.59 |

# Conclusion

In this paper we provided our simulation setup and results for Spatial Channel Modelling.

# Reference

- R4-2419782, “Way Forward for [113][320] NR_SCM”, Nokia

- 3GPP TS 38.101-4 V18.6.0 (2024-12), User Equipment (UE) radio transmission and reception, Part 4: Performance requirements