3GPP TSG-RAN WG4 Meeting # 114	R4-2500341

# Athens, Greece, 17th – 21st February 2025

Source: 	China Telecom

Title: 	Further study on SRS IL imbalance

Agenda Item:	7.1.1.3.4

Document for:	Discussion

Introduction

In RAN4#113, we made some progress on SRS IL imbalance solution but there is still a distance to convergence. The candidate solution framework was captured as below although the views are divergent among companies [1]. But we still need a compromised solution.

So in this paper we reiterate our view and also try to provide some compromised options.

Discussion

- 2.1 Reporting trigger condition and necessity analysis

Firstly, we shall clarify our point again that SRS IL is non-negligible and we support continuing to pursue a solution to the SRS IL imbalance issue based on the candidate solution in [1].

Proposal1: Continue to pursue a solution to the SRS IL imbalance issue based on the candidate solution.

The output power of SRS transmission is the minimum value of PCMAX,f,c and the actually calculated SRS power (denoted by PSRS_cal) as defined in 38.213.

While the insertion loss, i.e. the ∆TRxSRS can only influence the PCMAX,f,c part so the self-compensation should not be triggered unless the following equation is satisfied:

PCMAX,f,c - PSRS_cal ≤ ∆TRxSRS

Where PCMAX,f,c is not considering IL. In another word, the compensation will execute on PCMAX,f,c and only when the gap between the original PCMAX,f,c and PSRS_cal is small enough that the behavior is needed. self-compensation should be triggered only if the actual PCMAX,f,c with IL influence is smaller than the necessary SRS power.

Proposal2: The self-compensation should be triggered only if the PCMAX,f,c with IL influence is smaller than the necessary SRS power.

Besides, there are situation that the self-compensation cannot fully compensate the power imbalance, and NW can still do power adjustment through those RRC signaling but need necessary information. Compensation is not the target, but our purpose is to get a precise PMI estimation. So the assistance information reporting should be introduced for NW doing post-compensation no matter reporting a 0-1 flag or a numerical value.

Porposal3: Assistance information reporting should be introduced in Rel-19.

- 2.3 Potential solutions

In this part we provide some workable assistance information reporting solutions that impact the spec as less as possible, the reporting is on condition that UE do self-compensation as much as it can.

- Enhanced PHR on SRS

The type 3 PHR conveys the difference between PCMAX,f,c and estimated power for SRS transmission. So through monitoring the type 3 PHR the network can tell if UE is doing self-compensation and if it can compensate all the IL or not. Since PHR has the field for PCMAX,f,c and the elevation of PCMAX,f,c value means compensation is undertaking. On the other hand, if the value of PH field is not zero, the UE has enough power to compensate and the network don’t have to intervene.

There are already been type 3 PHR definition in TS 38.321 so the impact of specification would be quiet slight that we only need to care about the reporting granularity and trigger condition.

At last but not the least, type 3 PHR was only introduced in multiple entry PHR, i.e. the CA/EN-DC scenarios and single entry PHR can only report type 1 PHR as specified in TS 38.321 as below. So if this approach is adopted, we need to inform RAN2 to fix the defect.

Single entry PHR MAC CE

Multiple entry PHR MAC CE

Proposal4: Type 3 PHR for SRS transmission is a reasonable way to convey IL compensation information and does little impact on specification.

- Direct IL imbalance value report

As we mentioned many times, SRS insertion loss value reporting is more precise and intuitive and can still provide numerical information for NW doing post-compensation even the UE runs out of all the power headroom. But this method need new allocated field and signaling that has much impact.

Observation1: Direct IL imbalance value report is precise and intuitive but has much impact.

- Necessary capability report and signaling

After all, self-compensation should be an optional feature in Rel-19, so UE should report whether supporting SRS IL self-compensation or not, and the compensation as well as reporting should all be activated by configuration.

Proposal5: UE should report whether supporting SRS IL self-compensation or not, and the compensation as well as reporting should all be activated by configuration.

Conclusion

Observation1: Direct IL imbalance value report is precise and intuitive but has much impact.

Proposal1: Proposal1: Continue to pursue a solution to the SRS IL imbalance issue based on the candidate solution.

Proposal2: The self-compensation should be triggered only if the PCMAX,f,c with IL influence is smaller than the necessary SRS power.

Proposal3: Assistance information reporting should be introduced in Rel-19.

Proposal4: Type 3 PHR for SRS transmission is a reasonable way to convey IL compensation information and does little impact on specification.

Proposal5: UE should report whether supporting SRS IL self-compensation or not, and the compensation as well as reporting should all be activated by configuration.

References

- R4-2420376, WF on for 6Rx UE requirements, RAN4#113, AT&T