3GPP TSG-RAN WG4 Meeting #114											R4-2500344

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.2.3

Source: 	Fujitsu

Title: 	Discussion on BS OBB for 14800 to 15350 MHz frequency range

Document for:	Approval

# 1. Introduction

At RAN4#113 meeting, the LS for IMT parameters in 14800 to 15350 MHz frequency range [1] was sent to ITU-R WP5D and remaining issues were captured in the WF [2]. This contribution shares our thoughts on the remaining issues related to BS OBB.

# 2. Discussion

Firstly, we should clarify the overall completion date of this study item. It was originally Dec. 2024 and related LSs have already been sent to ITU-R WP5D on schedule. However, it was extended to Mar. 2025 because further discussion is needed for the remaining issues.

Observation 1: The overall completion date of this study item is Mar. 2025; it is extended from Dec. 2024.

Issue 4-2-8: Blocking response

Agreement

- ΔfOBUE and ΔfOOB 100MHz

- Define requirement as follows, where interferer is defined in radiated way

  - FFS how to define the requirements for 1-H

| Frequency range of interfering signal (MHz) | Wanted signal mean power (dBm) | Interferer RMS field-strength for OTA requirements (voltage/meter) | Type of interfering signal |
| --- | --- | --- | --- |
| 30 to 12750 | EISREFSENS + 6 dB | 0.36 | CW |
| 12750 to FUL,low – 100 | EISREFSENS + 6 dB | 0.1 | CW |
| FUL,high + 100 to 2nd harmonic of the upper frequency edge of the operating band | EISREFSENS + 6 dB | 0.1 | CW |

Considering the above observation, RAN4 should conclude the remaining issues in this meeting. Regarding the BS OBB, it is captured in the WF [2] and the excerpt is as follows.

The issue comes from the frequency ranges for BS type 1-H. In TS38.104, they are defined from 1 MHz to FUL,low - ΔfOOB and from FUL,high + ΔfOOB to up to 12750 MHz. It means that the upper bound of frequency range is 12750 MHz and does not cover the target frequency range, i.e. 14800 - 15350 MHz.

Observation 2: The frequency range of current BS type 1-H OBB requirement does not cover 14800 - 15350 MHz.

To define the OBB requirements for BS type 1-H, the upper bound of frequency range needs to be extended. However, such an extension of frequency range needs to be discussed carefully in RAN4 considering both feasibility and testability perspectives.

Observation 3: The extension of frequency range needs to be discussed carefully in RAN4 considering both feasibility and testability perspectives.

We think that it is very difficult to reach consensus for this issue in only one meeting. For this issue, our understanding is that RAN4 does not need to conclude it in this study item. We can further discuss it in the 6G study phase. Considering it, our proposal is as follows.

Proposal: If RAN4 does not define BS type 1-H OBB requirement for 14800 - 15350 MHz in this study item, RAN4 continue to discuss it in 6G study phase.

# 3. Conclusions

In this contribution, we shared our thoughts on BS OBB for 14800 to 15350 MHz frequency range as follows.

Observation 1: The overall completion date of this study item is Mar. 2025; it is extended from Dec. 2024.

Observation 2: The frequency range of current BS type 1-H OBB requirement does not cover 14800 - 15350 MHz.

Observation 3: The extension of frequency range needs to be discussed carefully in RAN4 considering both feasibility and testability perspectives.

Proposal: If RAN4 does not define BS type 1-H OBB requirement for 14800 - 15350 MHz in this study item, RAN4 continue to discuss it in 6G study phase.

# 4. Reference

- R4-2420385, LS Reply on Parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, Ericsson

- R4-2420330, WF on 15GHz BS parameters, ZTE