# 3GPP TSG-RAN WG4 Meeting #114	                                R4-2500345

Athens, GR, 17th – 21st February, 2025

Source:                Tejas Networks Pvt. Ltd.

Title:                     Discussion on existing Tx requirements for SBFD operation

Agenda Item:       7.23.2.2

Document for:     Discussion

# Introduction

In this contribution, we present overview of impact on existing BS RF TX requirements for unwanted emissions.

# Discussion

## Unwanted emissions

In the RAN4 #112 bis meeting, RAN4 agreed to further study on ACLR and OBUE with following agreement and WF [1].

FFS on the followings: - Whether to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB firstly? - Option 1: Yes, since legacy ACLR and OBUE requirement might not be sufficient to ensure feasible coexistence. - Option 2: No, since specific co-existence requirements can be covered by regional requirements or customer specific proprietary requirements. - How to define the additional co-existence requirements: - Option 1: based on CL assumption for BS2BS CLI interference and BS DESENS level. - [1dB DESENS in the victim SBFD receiver] - [typical CL values based on typical SBFD-capable gNB implementation] - [frequency offset for the additional OBUE requirement is declared] - Option 2: by improving ACLR1 and ACLR2 requirements - Option 3: others

In the RAN4 #113 meeting, RAN4 agreed to further study on ACLR and OBUE with following agreement and WF [2].

Agreement: - FFS whether/how to introduce the additional ACLR and/or OBUE requirements for FR1 WA BS.

### ACLR/OBUE

We would like to present our views on FR1 macro deployments when coexisting with a new SBFD system in an adjacent channel. We strongly recommend further research and refinement of the ACLR and OBUE requirements for FR1 WA-BS, particularly when these base stations are coexisting with SBFD systems operating on adjacent channels. The primary challenges of coexistence, especially those associated with gNB-to-gNB CLI, require more stringent isolation standards and enhanced RF specifications to guarantee the reliable and scalable deployment of SBFD-capable gNBs in complex deployment scenarios. Existing legacy standards for ACLR and OBUE are no longer sufficient to meet the requirements for SBFD coexistence in dense macro-cell environments, and they fail to account for the specific needs of gNB-to-gNB CLI scenarios under SBFD conditions. This limitation creates a significant gap in current guidelines, which necessitates focused improvements to ensure stable and high-performance operation in practical deployment settings, where interference mitigation is critical. A feasibility study outlined in [3] for SBFD base station co-location highlights several key coexistence requirements for FR1 WA-BS, which are crucial for ensuring that adjacent systems can operate without excessive interference. Specifically, the study recommends a base station output power of 49 dBm, a channel bandwidth of 100 MHz, and a degradation in victim base station sensitivity of no more than 1 dB. In addition to these basic parameters, the study emphasizes that to achieve reliable and interference-free coexistence, the total isolation between the aggressor and victim base stations must exceed 144 dB. This isolation is a sum of several factors, including the ACLR of the systems, the coupling loss resulting from the physical co-location of the base stations, and the rejection characteristics of the RF BPF used after the PA in the signal chain. Each of these elements plays a critical role in reducing interference and ensuring that signals from adjacent base stations do not adversely affect each other’s performance.

Recent advancements in PA linearization techniques, such as Digital Predistortion (DPD), have enabled FR1 macro base stations to achieve significantly improved ACLR performance. Specifically, these techniques allow base stations to reach an adjacent +1 ACLR of 55 dBc or better, which represents a notable improvement over older system. However, despite these technological advancements, meeting the required 144 dB isolation in real-world deployments remains a significant challenge. To achieve this level of isolation, it may be necessary to introduce schemes for handling gNB-to-gNB CLI interference. This could involve deploying additional filtering solutions or implementing further design modifications aimed at enhancing isolation between neighbouring systems. Such measures are essential to ensure robust coexistence and prevent interference that could undermine the performance and reliability of SBFD systems in operational environments.

Observation 1: The current ACLR and OBUE requirements for FR1 macro-BSs are inadequate to support coexistence with SBFD systems, particularly in dense macro-cell deployments where interference challenges are more pronounced.

Observation 2: While modern DPD techniques have significantly improved adjacent channel ACLR, they may still fall short of ensuring reliable coexistence without supplementary measures.

Observation 3: Enhancing the ACLR1 and ACLR2 requirements can improve the rejection performance of BPF, contributing to more effective interference management in co-located deployments.

Observation 4: Manufacturers can implement advanced filtering techniques and innovative hardware designs that meet the stringent isolation requirements for SBFD coexistence, providing effective and practical solutions for deployment in dense and complex environments.

Proposal 1: Establish scenario-specific ACLR and OBUE requirements. For SBFD co-located deployments, tighten the ACLR1 and ACLR2 requirements to achieve the minimum necessary co-location ACLR values. In isolated or non-co-located scenarios, retain the existing legacy ACLR and OBUE requirements to prevent over-specification and ensure efficiency in low-risk cases.

Proposal 2: Enhance the adjacent +1 and +2 ACLR requirements (55 dBc) to further improve the rejection performance of RF BPF and reduce adjacent channel interference, thereby supporting more reliable coexistence.

Proposal 3: Use of a CLI mitigation framework to manage gNB-to-gNB interference in adjacent channels. This framework should include clear guidelines for dynamic interference management, as well as co-location requirements tailored to SBFD-capable gNBs.

# Conclusion

In this contribution, we would like to present our perspectives on the RF TX requirements for SBFD BS, outlined as follows.

Observation 1: The current ACLR and OBUE requirements for FR1 macro-BSs are inadequate to support coexistence with SBFD systems, particularly in dense macro-cell deployments where interference challenges are more pronounced.

Observation 2: While modern DPD techniques have significantly improved adjacent channel ACLR, they may still fall short of ensuring reliable coexistence without supplementary measures.

Observation 3: Enhancing the ACLR1 and ACLR2 requirements can improve the rejection performance of BPF, contributing to more effective interference management in co-located deployments.

Observation 4: Manufacturers can implement advanced filtering techniques and innovative hardware designs that meet the stringent isolation requirements for SBFD coexistence, providing effective and practical solutions for deployment in dense and complex environments.

Proposal 1: Establish scenario-specific ACLR and OBUE requirements. For SBFD co-located deployments, tighten the ACLR1 and ACLR2 requirements to achieve the minimum necessary co-location ACLR values. In isolated or non-co-located scenarios, retain the existing legacy ACLR and OBUE requirements to prevent over-specification and ensure efficiency in low-risk cases.

Proposal 2: Enhance the adjacent +1 and +2 ACLR requirements (55 dBc) to further improve the rejection performance of RF BPF and reduce adjacent channel interference, thereby supporting more reliable coexistence.

Proposal 3: Use of a CLI mitigation framework to manage gNB-to-gNB interference in adjacent channels. This framework should include clear guidelines for dynamic interference management, as well as co-location requirements tailored to SBFD-capable gNBs.

# Reference

- R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF

- R4-2419908, Way Forward for [113][307] NR_duplex_evo_BSRF

- R4-2417788, “Discussion on existing Tx requirements for SBFD operation”, Tejas Networks