3GPP TSG-RAN WG4 Meeting #114	R4-2500373

Athens, Greece, 17-21 Feb 2025

Title: 	TP into TR 36.719-01-01: Addition of PC2 for UL CA_40A-42A

Agenda Item:	6.4.2

Source: 	Nokia

Document for:	Approval

# 1	Introduction

This TP adds PC2 operation for UL CA_40A-42A.

************************* Start of TP ******************************************

## 5.3	LTE-A inter-band CA for 2 bands DL with 2 bands UL

### 5.3.x	CA_40A-42A.

#### 5.3.x.1	Channel bandwidths per operating band for CA

CA_40A-42A has already been specified for PC3.

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 4) | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_40A-42A | CA_40A-42A | 40 |  |  |  | Yes | Yes | Yes | 40 | 0 |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |

#### 5.3.x.2	Maximum output power

To enable PC 2 operation for CA_40A-42A the highlighted change is needed for specification.

Table 6.2.2A-0: UE Power Class for uplink interband CA (two bands)

| E-UTRA CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_40A-42A |  |  | 26 | +2/-32 | 23 | +2/-32 |  |  |
| NOTE 1: Void NOTE 2: 2 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB for transmission bandwidths confined within 815 MHz and 818 MHz. |  |  |  |  |  |  |  |  |

#### 5.3.x.3	Co-existence studies

These studies have been done for CA_40A-42A PC3 operation and necessary requirements have been specified.

#### 5.3.x.4	∆TIB and ∆RIB values

These studies have been done for CA_40A-42A PC3 operation and necessary requirements have been specified.

#### 5.3.x.5	REFSENS requirements

There is MSD specified for this CA configuration for PC3, see below.

Table 7.3.1A-0f: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_40A-42A | 40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | 42 | 3500 | 5 | 25 | 3550 | 5 |  | IMD4 |
|  | 40 | 2350 | 5 | 25 | 2350 | 5 | TDD | IMD4 |
|  | 42 | 3525 | 5 | 25 | 3525 | N/A |  | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Void NOTE 6: For operations with 4 antenna ports, the MSD in the applicable bands shall be modified by the absolute value of ΔRIB,4R in Table 7.3.1-1a when MSD > 0. |  |  |  |  |  |  |  |  |

However, in dTib and  dRib tables note 4 is associated for CA_40-42

NOTE 4:	Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx.

MSD specification and note 4 are contradicting.

To us it seems that MSD TP is invalid as TDD operation must be syncronous thus TDD-TDD combination does not have MSD. Secondly it seems that Note 4 in dTib and  dRib tables for CA_40-42 should be removed as dual uplink operation is allowed.

With these explanations and corrention CRs under mainteneace to fix above discreapancies, PC2 operation can be allowed for UL CA_40A-42A.

************************* End of TP ******************************************