3GPP TSG RAN WG4 Meeting #114	R4-2500396

# Athens, Greece，17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2626 | rev | - | Current version: | 16.22.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (5G_V2X_NRSL-Core) CR to 38.101-1 Rel-16 Cat-F for 7.3E Reference sensitivity for V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For Reference sensitivity power level for V2X concurrent operation: - In the current Rel-16 spec, Table 7.3E.2-5 is NR Uu uplink configuration for NR V2X sidelink REFSENS, Table 7.3E.2-6 is Sidelink TX configuration for NR Uu REFSENS. - In the current Rel-16 spec, NR Uu uplink configuration and Sidelink TX configuration are limited to 10MHz Channel Bandwidth. - In the Rel-17 and Rel-18 specs, Table 7.3E.2-2 is Sidelink TX configuration for NR V2X sidelink REFSENS; Table 7.3.2-3 is NR Uu uplink configuration for NR Uu REFSENS. - In the Rel-17 and Rel-18 specs, NR Uu uplink configuration in Table 7.3.2-3 and Sidelink TX configuration in Table 7.3E.2-2 are not limited to 10MHz Channel Bandwidth. - In the Rel-17 and Rel-18 specs, Reference sensitivity power level for V2X concurrent operation is specified in a seperate subclause: 7.3E.3, and it is also aligned with the clause structure of other V2X requirements in the current Rel-16 spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Reference sensitivity power level for V2X concurrent operation: - Reference sensitivity power level for V2X concurrent operation was moved to a seperate subclause: 7.3E.3. - NR V2X sidelink REFSENS was revised to be with Sidelink TX configuration in Table 7.3E.2-2 and NR Uu REFSENS was revised to be with NR Uu uplink configuration in Table 7.3.2-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement for Reference sensitivity power level for V2X concurrent operation will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3E.2, 7.3E.3 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

7.3E.2	Minimum requirements

When UE is configured for NR V2X reception non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.7.2 with parameters specified in Table 7.3E.2-1.

Table 7.3E.2-1: Reference sensitivity of NR V2X Bands (PC5)

|  |  | Channel bandwidth / PREFSENS_V2X(dBm) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band | SCS kHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz | Duplex Mode |
| n38 | 15 | -96.5 | -93.2 | -91.4 | -90.1 | HD |
|  | 30 | -96.1 | -93.4 | -91.7 | -90.2 | HD |
|  | 60 | -96.9 | -93.1 | -91.9 | -90.4 | HD |
| n47 | 15 | -92.5 | -89.2 | -87.4 | -86.1 | HD |
|  | 30 | -92.1 | -89.4 | -87.7 | -86.2 | HD |
|  | 60 | -92.9 | -89.1 | -87.9 | -86.4 | HD |
| NOTE 1: Reference measurement channel is defined in A.7.2. NOTE 2: The signal power is specified per antenna port. NOTE 3: Void. |  |  |  |  |  |  |

Table 7.3E.2-2: Sidelink TX configuration for reference sensitivity of NR V2X Bands (PC5)

| NR Band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band | SCS kHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz | Duplex Mode |
| n38 | 15 | 50 | 105 | 160 | 216 | HD |
|  | 30 | 24 | 50 | 75 | 105 | HD |
|  | 60 | 102 | 24 | 36 | 50 | HD |
| n47 | 15 | 50 | 105 | 160 | 216 | HD |
|  | 30 | 24 | 50 | 75 | 105 | HD |
|  | 60 | 102 | 24 | 36 | 50 | HD |
| NOTE 1: The sidelink allocated RB (LCRB) size could be adjusted according to resource pool configuration in [7]. NOTE 2: For the case, 11 RB is allowed for S-SSB Block. |  |  |  |  |  |  |

Table 7.3E.2-3: Void

Table 7.3E.2-4:Void

Table 7.3E.2-5: Void

Table 7.3E.2-6:Void

7.3E.3	Reference sensitivity power level for V2X concurrent operation

When UE is configured for NR V2X reception on V2X carrier concurrent with NR uplink and downlink, NR V2X sidelink throughput for the carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.7.2 with parameters specified in Table 7.3E.2-1 and 7.3E.2-2. Also the NR downlink throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.3 with parameters specified in Table 7.3.2-1, 7.3E.3-1 and 7.3.2-3. The reference sensitivity is defined to be met with all downlink component carriers active. The REFSENS of Uu downlink and PC5 sidelink will be tested at the same time.

Table 7.3E.3-1: ΔRIB,V2X (two bands)

| V2X inter-band concurrent band Combination | NR Band | ΔRIB,V2X [dB] |
| --- | --- | --- |
| V2X_n71-n47 | n71 | 0.0 |

7.3F	Reference sensitivity for shared spectrum channel access

<< End of changes >>