# 3GPP TSG-RAN WG4 Meeting #114		R4-2500401

# Athens, Greece, 17– 21 February, 2025

Agenda item:			7.24.3

Source:	Qualcomm Incorporated

Title:	Discussion on Ambient IoT Testability

Document for:	Approval

# 1  Introduction

In RAN#106, the new WI on solutions for Ambient IoT (A-IoT) in NR was approved in [1]. RAN4 is tasked to study and develop the test methodology for A-IoT device 1 according to D1T1-B scenario.

- Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

In RAN4#113, the agreements for A-IoT device RF requirements were captured in [2]. In this paper, we provide our views on the test methodology for RF requirements of A-IoT device.

# 2  OTA test methdology for A-IoT device

## 2.1 General test setup for A-IoT device testing

In SI, it was concluded that with highly integrated architecture, OTA testing is considered as the baseline test methodology for device 1 [3]. Hence the conformance measurement shall be done in an anechoic chamber (AC). The traditional far-field test system of AC for handheld UE TRP and TRS testing specified in section A.3 of TS 38.161 [4] can be considered as the starting point for A-IoT device testing. Since the target scenario for Rel-19 A-IoT WI is D1T1-B in which CW node is outside, the possible test setup shown in Figure 1 with separate CW/RF energy source and measurement antenna descripted in [3] can be used as the test setup for A-IoT device OTA testing. Different from traditional AC for handheld UE TRP and TRS testing, at least the following test setup parameters shall be defined for A-IoT device testing.

Figure 1: An example of test setup for A-IoT device OTA testing

- DCW2D: distance between CW node and DUT

- DMA2D: distance between measurement antenna and DUT

- IsoCW2M: Isolation between CW node and measurement antenna

The distance between CW node and DUT and distance between measurement antenna and DUT shall be defined based on the far-field criteria. Specifically, the minimum isolation between CW node and measurement antenna shall be defined to avoid the coupling between CW node/RF energy source and measurement antenna.

Proposal 1: The traditional test system of AC for handheld UE radiated testing can be considered as the starting point for A-IoT device testing.

Proposal 2: The following test setup parameters shall be defined for A-IoT device OTA testing:

- DCW2D: distance between CW node and DUT

- DMA2D: distance between measurement antenna and DUT

- IsoCW2M: Isolation between CW node and measurement antenna

## 2.2 Test methodology for Tx RF requirements of A-IoT device

As agreed in [2], RAN4 will need to specify power backscattering loss and occupied bandwidth for device 1 as the Tx RF requirements.

For backscattering losing testing, the objective of this test is to verify that device can provide required backscattering signal level to be successfully received and detected by the reader.

The test setup shown in Figure 1 can be used for backscattering losing testing. The preliminary test procedure could be given by following steps:

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Determine the power level of RF energy source, duration time of harvesting, and ON-OFF testing pattern to ensure TE can start to transmit CW signal when DUT turns on and then record the backscattering signal from DUT.

- Step 3: Set the target test frequency and transmit power for signal generator and CW signal. The received power at the antenna of DUT shall be Pin. Pin includes both CW and R2D signal. The specific value of Pin is FFS.

- Step 4: Measure and record the backscattering signal level from DUT and calculate the backscattering power level transmitting from DUT antenna, i.e., Pout, based on the calibration data from step 1

- Step 5: Calculate the ratio of CW power level to D2R signal power (i.e., before and after backscattering) according to the equation: backscattering loss (dB) = Pout – Pin

For occupied bandwidth testing, the objective of this test is to verify that device can transmit the appropriate modulation waveform within the target bandwidth.

Similar as backscattering loss testing, the test setup shown in Figure 1 can be used for occupied bandwidth measurement. The step 1-4 in above test procedure can be reused here and the only difference is to determine the occupied bandwidth of DUT based on the signal recorded in the TE in step 5.

Proposal 3: Take the following test procedure as the start point for backscattering losing and occupied bandwidth testing.

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Determine the power level of RF energy source, duration time of harvesting, and ON-OFF testing pattern to ensure TE can start to transmit CW signal when DUT turns on and then record the backscattering signal from DUT.

- Step 3: Set the target test frequency and transmit power for signal generator and CW signal. The received power at the antenna of DUT shall be Pin. Pin includes both CW and R2D signal. The specific value of Pin is FFS.

- Step 4: Measure and record the backscattering signal level from DUT and calculate the backscattering power level transmitting from DUT antenna, i.e., Pout, based on the calibration data from step 1

- Step 5a (backscattering losing testing): Calculate the ratio of CW power level to D2R signal power (i.e., before and after backscattering) according to the equation: backscattering loss (dB) = Pout – Pin

- Step 5b (occupied bandwidth testing): Determine the bandwidth of backscattering signal

## 2.3 Test methodology for Rx RF requirements of A-IoT device

In [2], it was agreed that reference sensitivity shall be specified for A-IoT device. Reference sensitivity is to verify the received performance of R2D link. While different from legacy NR device testing, it is not feasible to measure the reference sensitivity for R2D link via throughput metric, e.g., >95% of maximum throughput. One feasible way is to verify the device’s respond timing relationship between R2D and the corresponding D2R transmission with a minimum mean power applied for device antenna, i.e., reference sensitivity. We provided a companion paper to discuss the ways defining the reference sensitivity in [5]. The following test procedure is given as an example.

Figure 2: Example of timing relationship between R2D and D2R

Proposal 4: The reference sensitivity of device can be verified by measuring device’s respond timing relationship between R2D and the corresponding D2R transmission with a minimum mean power applied for device antenna. FFS on how to set the side condition.

With proposal 4, we have the following proposal for the test procedure of reference sensitivity.

Proposal 5: The following test procedure of reference sensitivity for A-IoT device 1 can be considered as the starting point:

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Determine the power level of RF energy source, duration time of harvesting, and ON-OFF testing pattern to ensure TE can start to transmit CW signal when DUT turns on and then record the backscattering signal from DUT.

- Step 3: Set the target test frequency and transmit power for signal generator and CW signal. The transmit power of signal generator shall be set as that the received power at DUT’s antenna is equal to minimum reference sensitivity requirement of device. FSS on the CW signal level.

- Step 4: Determine whether DUT can send the correct response in D2R channel within timing window, e.g., TR2D_min

# 3  Conclusion

In summary, we have following proposals.

Proposal 1: The traditional test system of AC for handheld UE radiated testing can be considered as the starting point for A-IoT device testing.

Proposal 2: The following test setup parameters shall be defined for A-IoT device OTA testing:

- DCW2D: distance between CW node and DUT

- DMA2D: distance between measurement antenna and DUT

- IsoCW2M: Isolation between CW node and measurement antenna

Proposal 3: Take the following test procedure as the start point for backscattering losing and occupied bandwidth testing.

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Determine the power level of RF energy source, duration time of harvesting, and ON-OFF testing pattern to ensure TE can start to transmit CW signal when DUT turns on and then record the backscattering signal from DUT.

- Step 3: Set the target test frequency and transmit power for signal generator and CW signal. The received power at the antenna of DUT shall be Pin. Pin includes both CW and R2D signal. The specific value of Pin is FFS.

- Step 4: Measure and record the backscattering signal level from DUT and calculate the backscattering power level transmitting from DUT antenna, i.e., Pout, based on the calibration data from step 1

- Step 5a (backscattering losing testing): Calculate the ratio of CW power level to D2R signal power (i.e., before and after backscattering) according to the equation: backscattering loss (dB) = Pout – Pin

- Step 5b (occupied bandwidth testing): Determine the bandwidth of backscattering signal

Proposal 4: The reference sensitivity of device can be verified by measuring device’s respond timing relationship between R2D and the corresponding D2R transmission with a minimum mean power applied for device antenna. FFS on how to set the side condition.

Proposal 5: The following test procedure of reference sensitivity for A-IoT device 1 can be considered as the starting point:

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Determine the power level of RF energy source, duration time of harvesting, and ON-OFF testing pattern to ensure TE can start to transmit CW signal when DUT turns on and then record the backscattering signal from DUT.

- Step 3: Set the target test frequency and transmit power for signal generator and CW signal. The transmit power of signal generator shall be set as that the received power at DUT’s antenna is equal to minimum reference sensitivity requirement of device. FSS on the CW signal level.

- Step 4: Determine whether DUT can send the correct response in D2R channel within timing window, e.g., TR2D_min

# 4  References

[1] RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN1 Vice-chair (Huawei)

[2] R4-2420470, WF on RF requirements for A-IoT, Huawei

[3] 3GPP TR 38.769 V2.0.1

[4] 3GPP TS 38.161 V18.1.0

[5] R4-2500490, Ambient IoT device requirements, Qualcomm