# 3GPP TSG-RAN WG4 Meeting #114                                                                R4-XXXXXX

Athens, Greece, 17-21 February, 2025

Title: 	PRB misalignment for 12 PRB on 3MHz and 20 PRB on 5MHz

Source: 	Huawei, HiSilicon

Agenda item:	5.8.1

Document for:	Discussion

# Introduction

In Rel-18 3MHz CBW was introduced for which special 12 and 20 PRB allocations were considered for n100, TS 38.101-1 Table 5.4.3.1-3. These special PRB allocations were introduced to increase the spectrum utilization of the operator of the n100 band in Europe. In the following section we are going to discuss the misalignment that can be caused by these special PRB values.

TS38.101-1 Table 5.4.3.1-3: Additional GSCN parameters for band n100 SS Block frequency position SSREF (MHz); GSCN; Note. 920.73; 41637; Only applicable for 12 PRB transmission bandwidth configuration within 3 MHz channel with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1.. 921.45; 41638; Only applicable for 20 PRB transmission bandwidth configuration within 5 MHz channel with unpunctured PBCH defined in TS 38.211 [6] clause 7.4.3.1..

# Discussion

The approach that RAN4 took in Rel-18 (to define 12 PRB out of 15 and 20 PRB out of 25 for n100) was studied in details during the study item for irregular channel bandwidth, which is available in TR 38.844. In TS 38.844 section 6.1.2.1.2 the following is captured, which shows that to have the center of the subcarrier of the resource grid PRB aligned and a 100KHz channel raster, both maximum transmission bandwidth and the actual transmission bandwidth should be either even or odd values (yellow highlight with its logic highlighted in blue). This rule was not respected in TS 38.101-1 Table 5.4.3.1-3.

TS 38.844 section 6.1.2.1.2: The assumption that UE performance can only be ensured for RRC configurations according to the conformance test specification implies restrictions on the location and bandwidth of UE specific CHBW in operating bands with a 100 kHz channel raster. If the carrierBandwith of the carrier resource grid is an odd/even number of PRBs and the maximum transmission bandwidth configuration for the UE CHBW is even/odd for SCS = 15k, then the corresponding UE-specific CHBW cannot be located anywhere within a wider BS carrier resource grid: the said transmission bandwidth configuration cannot both be PRB aligned and separated by a multiple of 100 kHz to the center SC of the resource grid on the channel raster for m*180 + 90 = n*100 has no solution for any integers. If, on the other hand, the carrierBandwidth of the carrier resource grid is an odd/even number and the maximum transmission bandwidth configuration for the UE CHBW is odd/even for SCS = 15k, then the UE-specific CHBW can only be located with 5 PRB granularity, solutions to m*180 = n*100. Notwithstanding the above we assume that symmetric UE-specific channel bandwidths (MHz) can be set at the default duplex spacing: the centre frequencies of the UL and DL channel bandwidths as set by the parameters of the downlinkChannelBW-PerSCS-List and uplinkChannelBW-PerSCS-List should be at default duplex spacing that is, the default TX-RX carrier centre frequency separation as specified in clause 5.4.4 of 38.101-1. We also assume that the BWP#0 are configured such that the UE can locate symmetric CHBW at default duplex spacing during initial access. Next, we illustrate the method for a 7 MHz block within an operating band with a 100k channel raster and assuming the restrictions that - the SIB1 carrierBandwidth and any UE specific channel bandwidth on the 100k channel raster, carrier bandwidths and maximum transmission bandwidth configurations for UL and DL either all odd or all even, otherwise impossible to put a narrower channel inside a wider BS/cell bandwidth - the BS channel bandwidth supports a single NR RF carrier in the uplink or downlink as specified in 38.104. UL/DL carrierBandwidth in SIB1 only to a UE channel bandwidth (Rel-15)

For 3MHz and 5MHz CBW, the maximum transmission bandwidth (i.e., carrierBandwith) is 15 PRB and 25 PRB, respectively. Therefore, the actual PRB allocations must be odd numbers, too. As an example, based on the current specification, to have 12 PRB in 3MHz CBW (maximum transmission bandwidth 15 PRB), after the RRC reconfiguration the gNb should configure the UE with:

-	ServingCellConfig-> downlinkChannelBW-PerSCS-List-> carrierBandwidth = 15 PRBs

-	ServingCellConfig-> downlinkBWP-ToAddModList-> bwp-Common-> genericParameters-> locationAndBandwidth = 12 PRBs

However, based on the yellowed text, the actual transmission bandwidth (locationAndBandwidth) and the maximum transmission bandwidth (carrierBandwidth) must be both odd or even, which is not respected in our example. With the current PRB configuration the two sync rasters at 920.73MHz and 921.45 MHz will never be deployed because of this PRB misalignment, therefore the PRB configuration of these two sync rasters need to be updated .

Another solution would be to map 12 PRB and 20 PRB transmission bandwidth to CBWs higher than 3 and 5MHz respectively, such as 10MHz (with maximum transmission bandwidth configuration value at 52 PRB). Such a method is also indicated in TS 38.844 as “the next-after-next larger bandwidth”. However, this would mean that the sync rasters 920.73MHz and 921.45 MHz should be added to the legacy sync raster (TS 38.101-1 Table 5.4.3.1-1)., but it will allow the operator to use its 12 PRB (or 20 PRB) allocation properly.

Observation 1: Per TR 38.844 to have the center of the subcarrier of the PRB aligned and respect a channel raster of 100KHz both maximum transmission bandwidth and the actual transmission bandwidth should be either even or odd values.

Proposal 1:

Option 1: Either update the actual PRB values (currently 12 PRB and 20 PRB) to odd values to be aligned with the maximum transmission bandwidth configuration values (currently 15 PRB and 25 PRB).

Option 2:   Use the next-after-next larger bandwidth method and map 12 PRB and 20 PRB transmission bandwidth to 10MHz CBW with maximum transmission bandwidth configuration value at 52 PRB (i.e. even values).

Option 3: Remove TS 38.101-1 Table 5.4.3.1-3 as 12 and 20 PRB cannot be configured within a 3 or 5MHz CBW properly in the real field

# Conclusions

Observation 1: Per TR 38.844 to have the center of the subcarrier of the PRB aligned and respect a channel raster of 100KHz both maximum transmission bandwidth and the actual transmission bandwidth should be either even or odd values.

Proposal 1:

Option 1: Either update the actual PRB values (currently 12 PRB and 20 PRB) to odd values to be aligned with the maximum transmission bandwidth configuration values (currently 15 PRB and 25 PRB).

Option 2:   Use the next-after-next larger bandwidth method and map 12 PRB and 20 PRB transmission bandwidth to 10MHz CBW with maximum transmission bandwidth configuration value at 52 PRB ( i.e. even values).

Option 3: Remove TS 38.101-1 Table 5.4.3.1-3 as 12 and 20 PRB cannot be configured within a 3 or 5MHz CBW properly in the real field