3GPP TSG-RAN WG4 Meeting #114	R4-2500428

Athens, GR, 17th – 21st February, 2025

Source:	Nokia

Title:	Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements

Agenda item:	7.16.2

Document for:	Information

Introduction

This informative contribution is intended to help other interested parties to implement a TS 38.827 based CDL channel model and enable alignment with our version. Furthermore, this tdoc serves as the companion contribution to our FS_NR_demod_SCM discussion contribution [1]; it contains notably the setup and results delivered for alignment to RAN4#114.
Previous results, which dive deep into the motivation of spatial channel models (SCMs), can be found in [3] and [2] along with implementation notes.

In summary, this contribution contains the following parts:

- Full set of CDL based SCM alignment results (all AAV options).

- Additional alignment intermediate results (e.g., power normalization)/

- TP for 38.827 based CDL channel model (“CDL”) updated with agreements from RAN4#112 and RAN4#112bis.

No observations nor proposals are made in this simulation contribution.

Simulation results

While we use this tdoc to explain and copy paste the figures, it is recommended to look at the simulation collection summary from RAN4#114 to have a more easily handleable spreadsheet format.

Simulation setup

All simulation results in this tdoc and our companion discussion tdoc [1] use the configuration detailed in Table 1 (based on WF [4]) unless explicitly stated otherwise.

Table 1: Informative simulation alignment configuration.

| Parameter |  | SU-MIMO PDSCH (FR1) | SU-MIMO PMI (FR1) |  | MU-MIMO PDSCH (FR1) |
| --- | --- | --- | --- | --- | --- |
| Duplex mode |  | TDD |  |  |  |
| TDD Slot Configuration Pattern |  | 7DS2U |  |  |  |
| FR / Carrier frequency |  | FR1 / 3.5 GHz |  |  |  |
| UE receiver type |  | IRC Baseline | IRC Baseline |  | IRC, E-IRC |
| Number of Tx Ports |  | 8,4 | 8 |  | 4 |
| Number of Rx Ports/Antennas |  | 8,4 | 4 |  | 4 |
| Number of layers |  | 8,4 | 4,2 |  | 2+2 |
| PMI |  | Random, FFS Fixed Fixed based on follow PMI histogram max count. | Type I: Follow, Random eType II: Follow |  | UE1: Random UE2: Random and not equal to UE1, Orthogonal |
| Waveform |  | CP-OFDM with normal CP |  |  |  |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |  |  |  |
| MCS |  | 13 (64 QAM table) |  |  |  |
| PDSCH configuration | Mapping type | Type A |  |  |  |
|  | k0 | 0 |  |  |  |
|  | Starting symbol (S) | 2 |  |  |  |
|  | Length (L) | 12 |  |  |  |
|  | PDSCH aggregation factor | 1 |  |  |  |
|  | Resource allocation type | Type 0 |  |  |  |
|  | VRB-to-PRB mapping type | Non-interleaved |  |  |  |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |  |  |  |
| PDSCH DMRS configuration | DMRS Type | Type 1 |  |  |  |
|  | Number of additional DMRS | 1 |  |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 (for rank > 4) 1 (for rank <= 4) |  |  |  |
| Codebook configuration | CodebookType | type-I-SP | type-I-SP, eType-II |  | type-I-SP |
|  | Codebook configuration | For 8Tx (N1,N2,O1,O2) = (4,1,4,1) For 4Tx (N1,N2,O1,O2) = (2,1,4,1) | For typeI-SP: (N1,N2,O1,O2) = (4,1,4,1) For eType-II: (N1,N2,O1,O2) = (4,1,4,1) paramCombination-r16=5 (L=4, pν=1/4, β=3/4) N_3=1 (“WB”) |  | (N1,N2,O1,O2) = (2,1,4,1) |
| PDSCH DMRS Precoding Configuration |  | For Random precoding: for every PRB Bundle (size=2) For Fixed precoding: Wideband PMI | For typeI-SP (random and follow): Wideband PMI For eType-II with follow and random: TBD WB |  | For Random precoding: for every PRB Bundle (size=2) |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type | Aperiodic |  |  |  |
|  | Number of CSI-RS ports (X) | 4 CSI-RS Ports (2,1) for 4 Layer 8 CSI-RS Ports (4,1) for 8 Layer | 8 CSI-RS Ports (4,1) |  | TBD |
|  | Density (ρ) | 1 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) | (13) |  |  |  |
|  | Row | 4 CSI-RS Ports: row = 4 8 CSI-RS Ports: row = 6 |  |  |  |
|  | Power offset | βCSI-RS in accordance with Table C.3.1-1 in TS 38.101-4 |  |  |  |
| CQI/RI/PMI delay |  | N/A | 6.5 ms | N/A |  |
| Number of HARQ Processes |  | 8 |  |  |  |
| Maximum HARQ transmissions |  | 4 |  |  |  |
| Channel Models |  | The purpose of the study item. Low speed (3kph/10Hz Doppler) and mid speed (30kph/100Hz Doppler) conditions. |  |  |  |
| Testing Metric |  | Full Throughput Curves |  |  |  |
| Channel Geometry (CDL) | LCS UE | α = 180°, β=0°, γ = 0° |  |  |  |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |  |  |  |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |  |  |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |  |  |
|  | BS Antenna Polarisation | (+45, -45) |  |  |  |
|  | BS Radiation Pattern | defined in TR38.901 Table 7.3-1. |  |  |  |
|  | UE Antenna Polarisation | (0, 90) |  |  |  |
|  | UE Antenna Radiation Pattern | Omnidirectional |  |  |  |
|  | Antenna Panel Placement | YZ Plane |  |  |  |

AAV and channel definitions

Based on WF [4].

Channel model configurations:

| Option | Description |
| --- | --- |
| A1 | CDL (TR 38.753) with Table 7.2.1-8 in TR 38.827 |
| A2 | CDL (TR 38.753) with Table TBD |
| B | TDL extensions as defined in (R4-2412762) |
| C | TDL extensions as defined in (R4-2415382) |
| D | TDL extensions as defined in (R4-2419895) |

AAV configurations:

| Option | Description | Note |
| --- | --- | --- |
| 1X | (M,N,P,Ms,Ns) = (8,8,2,8,4) for 4Tx CSI-RS Ports (M,N,P,Ms,Ns) = (8,8,2,8,2) for 8Tx CSI-RS Ports | Partially-connected, constant panel size |
| 1Y | (M,N,P,Ms,Ns) = (8,2,2,8,1) for 4Tx CSI-RS Ports (M,N,P,Ms,Ns) = (8,4,2,8,1) for 8Tx CSI-RS Ports | Partially-connected, 1 column per port |
| 2 | (M,N,P,Ms,Ns) = (8,8,2,8,8) for 8Tx and 4Tx CSI-RS Ports | Fully-connected, 827 beam directions |
| 3 | (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports | Pass-through |

SU PDSCH - 8Rx: 8 Layer, MCS 13 on both codewords (Table 1) (type I codebook) – PMI Choice (FFS Fixed, Random)

Figure 3: Case 1, 10Hz Doppler

Figure 3: Case 1, 100Hz Doppler

SU PDSCH - 4Rx: 4 Layer, MCS 13 (Table 1)  (type I codebook) – PMI Choice (FFS Fixed, Random)

Figure 3: Case 2, 10Hz Doppler

Figure 3: Case 2, 100Hz Doppler

SU PMI - [8Tx/]4Rx: 4 Layer (type I and eType II) – Full Throughput Curves (PMI Follow, PMI Random)

Figure 3: Cases 3-4, 10Hz Doppler

Figure 3: Cases 3-4, 100Hz Doppler

SU PMI - 4Rx: 2 Layer (type I and eType II) – Full Throughput Curves (PMI Follow, PMI Random)

Figure 3: Cases 5-6, 10Hz Doppler

Figure 3: Cases 5-6, 100Hz Doppler

MU PDSCH - 2+2 layers with IRC and E-IRC (type I orthogonal and random precoding, to distinguish target and co-scheduled UE)

Figure 3: Cases 7-8, 10Hz Doppler, random

Figure 3: Cases 7-8, 10Hz Doppler, orthogonal

38.827 vs. 38.901 CDL ChMod implementation

| 10Hz | 100Hz |
| --- | --- |
| 10Hz Doppler, orthogonal |  |

Figure 2: CDLC Uma 365ns performance differences between TR 38.827 and TR 38.901 implementation
(using both channel model parameters and derivation procedures from respective specifications).

Channel model normalization/power intermediate results

Following the background presented in our discussion paper [1], these are our long term (full simulation time) average channel model gains per receive antenna, and for each receive antenna, under the 8x8/4x4 CDLC 363ns channel models (38.827 compliant):

|  | 4x4 CDLC 363ns |  |  |  | 8x8 CDLC 363ns |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AAV | 1X | 1Y | 2 | 3 | 1X | 1Y | 2 | 3 |
| Avg RX branch | 1.13 | 2.31 | 0.50 | 1.50 | 1.56 | 2.04 | 0.52 | 2.55 |
| Each RX branch | 1.23 1.11 1.14 1.04 | 2.45 2.28 2.31 2.20 | 0.54 0.50 0.48 0.46 | 1.62 1.51 1.44 1.41 | 1.62 1.86 1.63 1.33 1.50 1.62 1.55 1.40 | 2.08 2.30 2.08 1.72 2.00 2.15 2.09 1.92 | 0.52 0.58 0.54 0.45 0.51 0.55 0.52 0.47 | 2.54 2.80 2.62 2.28 2.48 2.66 2.60 2.41 |
| Assumptions: - 1/MeNe AAV weight normalization for all AAVs. - Average over the whole simulation duration (10’000 slots). - Average over 100 random starting seeds (all seeds converge to same value eventually) |  |  |  |  |  |  |  |  |

Table 3: Long term average channel model gains

TP for 38.827 based CDL channel model (SU version)

This section gives a description of a TR38.827 based SU CDL channel model (often referred to as “CDL” or “827 CDL”) in the form of a potential TP for 38.101-4, which was used to derive all our results in our contributions. Note that explanatory parts of TR38.827, that are not needed for implementation, have been removed, and the extension for conducted/virtual cable test setup have been added.	
Minor modifications based on discussions and agreements in RAN4#112/112bis have been included, where appropriate

Interested contributors are encouraged to implement 827 CDL based on the following TP and exchange with Nokia to achieve alignment.

<Start of TP>

## B.2.X1	Cluster Delay Line Channel Model

### B.2.X1.Y1	General

The channel model is based on the description and clarifications given in TR 38.827.

For NR FR1 and FR2 MIMO testing, the number of samples for sequence length at each testing point is FFS.

### B.2.X1.Y1	Channel Models

This section describes amendments to the stepwise procedure of the CDL subclause 7.7.1 in TR38.901 for generating fast fading radio channel realizations. This channel model methodology considers non-Jakes spectrum with the multi-path fading propagation conditions between the gNB emulator and DUT modelled based on Clustered Delay Line (CDL) methodology.

The tables provided in section B.2.X1.Y1 are to be used directly in the procedure in TR38.901 for generating fast fading radio channel realizations. The given coefficients have been scaled to achieve the following target delay spread values

Table 7.2-1. Target delay spread values.

| Frequency | Scenario | DSdesired |
| --- | --- | --- |
| FR1 | UMi | 100 ns |
| FR1 | UMa | 365 ns |
| FR2 | UMi | 60 ns |
| FR2 | InO | 30 ns |

Similarly, the cluster-wise rms azimuth spread of arrival angles (cluster ASA, cASA), and similar spreads in the tables, are scaled to achieve the following desired AS, for the given cluster AOA (n,AOA) and similar azimuth/zenith angles (Tables 7.7.1.1 – 7.7.1.5 of TR 38.901).

Table 7.2-4: Desired AS for UMi and UMa at 3.5 GHz (FR1)

| Model | ASdesiredASdesiredASdesired [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| UMi NLOS (CDL-A, B, C) | 23.9751 | 57.2457 | 0.7762 | 7.8320 |
| UMi LOS (CDL-D, E) | 15.0432 | 47.6149 | 0.6166 | 4.6204 |
| UMa NLOS (CDL-A, B, C) | 25.7620 | 74.1138 | 4.8978 | 18.2050 |
| UMa LOS (CDL-D, E) | 14.0180 | 64.5654 | 3.4674 | 8.9125 |
| Note: For UMa frequency fc = 6 as stated in [2], and other parameters hUMa = 25, hUMi = 10, hUT = 1.5, and D2D = 100. |  |  |  |  |

Once the channel realization procedure arrives at the step of coupling the AOD angles to AOA angles within a cluster n (step 2). Instead of random procedure, the coupling shall be performed using the fixed coupling pattern specified in Table 7.2-6. The same fixed coupling pattern is applied for all clusters n.

Table 7.2-6: Fixed coupling pattern of ray angles to be applied for each cluster

|  | m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 12 | 5 | 10 | 8 | 11 | 16 | 14 | 18 | 9 | 20 | 4 | 2 | 15 | 7 | 13 | 19 | 17 | 3 | 1 |
|  | 20 | 9 | 12 | 1 | 13 | 18 | 10 | 4 | 8 | 2 | 6 | 14 | 11 | 19 | 7 | 3 | 17 | 5 | 15 | 16 |
|  | 2 | 16 | 3 | 11 | 18 | 9 | 5 | 17 | 4 | 19 | 15 | 20 | 13 | 7 | 10 | 1 | 8 | 12 | 6 | 14 |
|  | 15 | 18 | 13 | 1 | 12 | 9 | 6 | 7 | 5 | 3 | 2 | 8 | 14 | 17 | 19 | 16 | 11 | 20 | 10 | 4 |

In the next steps, the linear cross polarization power ratios (XPR) are calculated for each ray m of each cluster n as

,	(7.2-7)

where X is the per-cluster XPR in dB from the tables provided in section B.2.X1.Y1.

The gNB beam pattern including the assumptions for gNB antenna for definitions and symbols of subclause 7.3 of TR38.901 for FR1 and FR2 are summarized in Table 7.2-7.

Table 7.2-7: BS Antenna Parameters

| Parameter description | Symbol | Parameter value |  |  |
| --- | --- | --- | --- | --- |
|  |  | FR1 ≤2.5GHz | FR1 >2.5GHz | FR2 |
| Antenna panels in vertical dimension | Mg | 1 | 1 | 1 |
| Antenna panels in horizontal dimension | Ng | 1 | 1 | 1 |
| Elements per panel in vertical dimension | Me | 4 | 8 | 8 |
| Elements per panel in horizontal dimension | Ne | 8 | 8 | 16 |
| Number of polarizations per panel | P | 2 | 2 | 2 |
| Element spacing in horizontal dimension () | dH | 0.5 | 0.5 | 0.5 |
| Element spacing in vertical dimension () | dV | 0.5 | 0.5 | 0.5 |

Antenna element radiation patterns, including orientation of the element main polarization components as well as orientation of the antenna array for both FR1 and FR2 are as in the example pattern in Table 7.3-1 of TR38.901. The antenna element has ±45 polarization components and the radiation pattern parameters are 3dB = 65, 3dB = 65, Amax = 30dB, SLAv = 30dB, GE,max =8 dBi.

Note this chosen AE pattern may impact the RMS delay spread scaling and necessitates coordinate transformations as defined in TR 38.901 section 7.3.2, but this has not been taken into account for deriving the tables provided in section B.2.X1.Y1.

The BS antenna panel is oriented to have 10 of mechanical tilt with respect to the horizon (10 rotation around the y-axis).

It is assumed the co-polarized elements of the array are combined to a single RF port, i.e. they compose an antenna array that can form beams by setting certain weights per element. Weight vector for the first polarization and for the second polarization is

α1,1…αMe,Ne=1MeNeexp-j2πrtx,max T ∙ dtx,me,neλ0∈C1×MeNe,	(7.2-8)

where dtx,me,ne is the location vector of transmit antenna element me=1,…,Me and ne=1,…,Ne, and rtx,max is a spherical unit vector denoting the target beam direction. Note that the target beam directions, directions of departure (DoD) angles of the transmitter array, i.e., ZoD and AoD, and the locations of antenna elements, are referenced to the global coordinate system (GCS). Determination of beam directions rtx,max is done as follows:

-	For FR1: A grid of 60 fixed beams is constructed to a grid of five elevation angles from –20 to +20 with 10 steps and 12 azimuth angles from –80 to +80 with ~15 steps；

For FR1 4x4, the two strongest transmitting beams are selected from the pre-defined beam grid based on their proximity to the strong clusters of each FR1 channel model. These beams should have different azimuth directions and can provide the highest receive power for UE.

For FR1 8x8, the four strongest transmitting beams are selected from the pre-defined beam grid based on their proximity to the strong clusters of each FR1 channel model. These beams should have different azimuth directions and can provide the highest receive power for UE.

In detail, beam directions for channels model given in Section B.2.X1.Y1 are

-	For UMa CDL-C, the beam directions in the global coordinate system are:

-	Strongest beam: AoD: -7.27°, ZoD: 100°

-	2nd strongest beam: AoD: -21.82°, ZoD: 100°

-	3rd strongest beam: AoD: -36.36°, ZoD: 100°

-	4th strongest beam: AoD: -50.91°, ZoD: 100°

For the UE it is assumed that RF ports map directly to receive antenna elements, where drx,u is the location vector of receive antenna elements, which are configured as a ULA with Cross-pol (1, n/2, 2), i.e. [+ + + +] for n=8, antenna elements with omni radiation pattern, 0.5 spacing, and the AE slant is rotated 45deg to the transmitter polarizations. Note that “n” is the number of Rx elements, and the directions of arrival (DoA) angles of the receiver array, i.e., ZoA and AoA, are referenced to the GCS.

The UE antenna panel is oriented to point towards the front of the BS panel without any mechanical tilt with respect to the horizon (no rotation around the y-axis).

The random initial phases  are not used for the different polarization combinations (θθ, θϕ, ϕθ, ϕϕ). Instead, a fixed and pre-defined set of initial phases Φmθθ,Φmθϕ,Φmϕθ,Φmϕϕ of Table 7.2-8 and a scalar random initial phase term Φn,m~Uni(-π,π) is used for each ray m of each cluster n.

Table 7.2-8: Fixed initial phases for 2x2 polarization matrices. These values are drawn from uniform distribution

| m | Φmθθ [rad] | Φmϕθ [rad] | Φmθϕ [rad] | Φmϕϕ [rad] |
| --- | --- | --- | --- | --- |
| 1 | 1.7609 | -0.6928 | -1.6230 | -0.6037 |
| 2 | -2.5356 | -2.3124 | 2.7775 | 2.8660 |
| 3 | 0.4725 | -2.7660 | -1.6664 | -0.9226 |
| 4 | 2.0181 | -3.0448 | -2.8713 | -2.0798 |
| 5 | 0.9369 | 1.4560 | 0.9283 | -0.3084 |
| 6 | 0.2954 | -1.2798 | 1.5375 | -1.9544 |
| 7 | 1.1735 | -1.9886 | -0.8263 | 0.7893 |
| 8 | 1.7607 | -2.6319 | 2.6979 | 1.7324 |
| 9 | -0.0830 | -0.4030 | -0.3344 | -1.2167 |
| 10 | 0.0535 | 0.0677 | 1.9957 | 1.8525 |
| 11 | 0.9068 | -0.7627 | 1.9577 | 0.2062 |
| 12 | -0.9379 | 2.7583 | 2.3621 | 0.3151 |
| 13 | 0.7695 | 0.5469 | -1.8363 | -1.2488 |
| 14 | -0.1827 | -1.6934 | 2.1634 | -1.9179 |
| 15 | -1.7221 | -2.0690 | -1.7111 | -0.4040 |
| 16 | -1.1869 | 2.6602 | -0.4385 | -1.9804 |
| 17 | 2.5439 | 3.0143 | -0.3841 | -2.4434 |
| 18 | -1.5201 | -0.5735 | 0.5962 | -1.4941 |
| 19 | 0.6462 | 1.3271 | -1.7483 | -2.4038 |
| 20 | -1.2775 | -1.1386 | -0.4765 | 0.0494 |

To determine the channel all clusters are treated as "weaker cluster", i.e., no further sub-clusters in delay should be generated. The BS beamforming weights αme,neαme,nedefined in Equation 7.2-8 for antenna elements are used and the BS antenna signals are summed for BS beamforming. The BS transmits downlink signals with S beams. Index s=1,…,S denotes the formed beam index. Each beam may have different and thus the beamforming weight of eq. (7.2-8) becomes specific for index s as αs,me,ne; it should be noted though that there are always two orthogonally polarized beams to the same direction. Here, the random initial phases Φn,m are used for sub-paths, but not for the different polarization combinations (θθ, θϕ, ϕθ, ϕϕ). The channel coefficient for time instant t, Rx antenna/beam u, Tx beam s, and cluster n is defined by the following equations. They apply for the NLOS clusters:

Hu,s,nNLOSt=PnMm=1Mne=1Neme=1MeFrx,u,θθn,m,ZOA,φn,m,AOAFrx,u,φθn,m,ZOA,φn,m,AOATexpjΦmθθκn,m-1expjΦmθϕκn,m-1expjΦmϕθexpjΦmϕϕFtx,s,me,ne,θθn,m,ZOD,φn,m,AODFtx,s,me,ne,φθn,m,ZOD,φn,m,AOD

αs,me,neexpjΦn,mexpj2πrrx,n,mT.drx,uλ0expj2πrtx,n,mT.dtx,s,me,neλ0expj2πrrx,n,mT.vλ0t,	(7.2-9)

where Ftx,s,me,ne,θ, Ftx,s,me,ne,φ, and dtx,s,me,ne are the theta and phi polarized radiation patterns and the position vector of the BS antenna element me,ne of sub-array s, respectively. Symbols Frx,u,θ , Frx,u,ϕ, , , and , are determined as in TR 38.901, which includes the generation of AOA (ϕn,m,AOA), AOD (), ZOA (), and ZOD () to derive  and . The velocity vector is determined as

(7.2-11)

UE velocity v is defined as follows: 30km/h for FR1 vs 3 km/h (Indoor Office) and 12 km/h (UMi) for FR2. The UE travelling direction (v, v) are as follows for FR1:

-	(135°,90o) for UMi CDL A channel model

-	(127.05°],90o) for UMi CDL C channel model

-	(182.17°,90o) for UMa CDL A channel model

-	(65°,90o) for UMa CDL C channel model

The UE travelling direction (v, v) are as follows for FR2:

-	(112.51°,90°) for InO CDL-A channel model

-	(74.11°,90°) for UMi CDL-C channel model

Note: the FR2 cases may be mixed up in 38.827, though this has no impact in the CDL-C UMa case we are focusing on.

### B.2.X1.Y1	Channel Model Parameters for FR1

The Channel model parameter tables for CDL-A, B, C, D, and E for UMa and UMi at 3.5 GHz are presented in this subclause without the effect of base station antenna filtering.

For FR1, the baseline emulated propagation environment is 2D without elevation modelling, i.e., all ZOA are set 90° and ZSA is 0° in the following tables.

Tables 7.2.1-6—7.2.1-10 tabulate channel model parameters for UMa CDL-A—CDL-E models at 3.5 GHz, respectively.

In the determination of desired angle spreads (ASdesired), frequency is set 6 GHz as stated in Table 7.5-6 Part-1 of TR38.901.

For the determination of desired zenith spread of departure (ZSDdesired) from table 7.5-7 of TR38.901, the following parameters are used hUT = 1.5 m, and d2d = 100 m.

Table 7.2.1-8: Channel model parameters for UMa CDL-C at 3.5 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -37.4195 | -96.4031 | 96.7645 | 90 |
| 2 | 76.6135 | -1.25 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 3 | 80.9935 | -3.4684 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 4 | 85.0085 | -5.2294 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 5 | 79.424 | -2.5215 | -33.5316 | -124.0196 | 100.8594 | 90 |
| 6 | 232.359 | 0 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 7 | 235.352 | -2.2185 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 8 | 239.44 | -3.9794 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 9 | 240.316 | -7.4215 | 41.4581 | 51.4188 | 106.3995 | 90 |
| 10 | 289.6275 | -7.1215 | -49.2149 | 62.9864 | 94.4761 | 90 |
| 11 | 299.7745 | -10.7215 | 46.1367 | -41.2744 | 107.4834 | 90 |
| 12 | 340.764 | -11.1215 | -70.697 | 42.5606 | 92.3083 | 90 |
| 13 | 448.4025 | -5.1215 | -43.1524 | 64.6538 | 104.5929 | 90 |
| 14 | 477.5295 | -6.8215 | -49.0831 | -62.7423 | 105.1951 | 90 |
| 15 | 792.196 | -8.7215 | -58.4403 | 78.6184 | 91.7061 | 90 |
| 16 | 989.3325 | -13.2215 | 60.9633 | 25.6781 | 105.1951 | 90 |
| 17 | 1554.4985 | -13.9215 | 58.6569 | -23.4063 | 93.9944 | 90 |
| 18 | 1679.1095 | -13.9215 | 51.8037 | -2.3553 | 91.8265 | 90 |
| 19 | 2003.923 | -15.8215 | -73.86 | -20.5926 | 90.7426 | 90 |
| 20 | 2046.8105 | -17.1215 | 54.0442 | 3.7933 | 108.2061 | 90 |
| 21 | 2301.8725 | -16.0215 | 54.7691 | -0.4794 | 91.7061 | 90 |
| 22 | 2422.651 | -15.7215 | 63.5332 | -5.5859 | 91.5856 | 90 |
| 23 | 2570.5855 | -21.6215 | 72.0338 | -29.1381 | 106.3995 | 90 |
| 24 | 3158.0895 | -22.8215 | -88.2912 | 28.7003 | 109.5309 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.3179 | 15.632 | 3.6131 | 0 | 7 |  |

<End of TP>

Conclusion

Within this contribution we have provided simulation results and CDL implementation reference for use in the Spatial Channel Model for Demodulation Performance Requirements study, the results will be used in our companion discussion contribution [1].

- References

- R4-2500427, “Discussion on Spatial Channel Model for Demodulation Performance Requirements”, Nokia, RAN4#114.

- R4-2415283, “Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements”, Nokia, RAN4#112bis.

- R4-2418044, “Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements”, Nokia, RAN4#113.

- R4-2419782, “Way Forward for [113][320] NR_SCM”, Moderator (Nokia), Nokia, RAN4#113.