3GPP TSG-RAN WG4 Meeting #114                              						         R4-2500436

# Athens, Greece, 17th – 21th Feburary, 2025

Source: 	Apple

Title: 	RF requirement impact for type 4 UE

Agenda Item:	7.5.2.1

Document for:	Discussion

# Introduction

Type 4 UE capability and new BS signaling have been discussed in the past RAN4 meetings. Though not concluded, it is anticipated that the signaling design will be different from Rel-18. This contribution will present our views on RF requirement impact by the new signaling design.

# Discussion

In Rel-18, a default UE behaviour in non-collocated condition is assumed for type 2 and introduced BS signalling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC for type 2 UE for the case of maxMIMO-Layers with value less than or equal to 2. If nonCollocatedTypeNR-CA/ nonCollocatedTypeMRDC is indicated, type 2 UE will reconfigure its RF to type 1 and type 1 requirements for collocated scenario apply. Otherwise, requirements for non-collocated scenario apply. If maxMIMO-layers ＞2, it is collocated. The description for nonCollocatedTypeNR-CA in TS 38.331 is reproduced as below.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

nonCollocatedTypeMRDC

This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements.

As discussed in [2], for type 4 UE, signaling design should consider different way, where the following can be assumed.

When maxMIMO-layers ＞4, similar consideration can be borrowed from Rel-18. In this case the UE is type 1.

When maxMIMO-layers =4, no need to define a default UE behavior since every status is clearly commanded by network signaling. Collocated and non-collocated behavior are both explicitly commanded by network. E.g.

- when network signalling [nonCollocatedTypeNR-CA-R19/ nonCollocatedTypeMRDC-R19] is present with 0, the UE is in non-collocated scenario.

- when network signalling [nonCollocatedTypeNR-CA-R19/ nonCollocatedTypeMRDC-R19] is present with 1, the UE is in collocated scenario.

When maxMIMO-layers =2,

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1.

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2.

The non-collocated requirement in 7.10A (with further update for type 4 in table 7.10A.2-1) should have different consideration and accommodate the case when maxMIMO-layers=4 and maxMIMO-layers=2 for type 4 UE.

For a UE capable of intraBandNR-CA-non-collocated-r18 for the following CA band combinations in Table 7.10A.2-2, the Power imbalance requirements are applicable with 2Rx antenna ports for each component carrier if it is not provided with nonCollocatedTypeNR-CA-r18 and is configured with maxMIMO-Layers with value less than or equal to 2.

For a UE capable of intraBandNR-CA-non-collocated-r19 for the following CA band combinations in Table 7.10A.2-2, the Power imbalance requirements are applicable

- with 2Rx antenna ports for each component carrier if it is not provided with nonCollocatedTypeNR-CA-r18 and is configured with maxMIMO-Layers with value less than or equal to 2.

- With 4Rx antenna ports for each component carrier if nonCollocatedTypeNR-CA-R19 is present with 0 and is configured with maxMIMO-Layers with value equal to 4.

Proposal 1: Non-collocated requirement in 7.10A should accommodate the case when maxMIMO-layers=4 and maxMIMO-layers=2 for type 4 UE. E.g. below text is proposed.

Proposal 2: To simplify the test, it is proposed to focus on testing of 4Rx antenna port only for type 4 UE.

# Summary

This contribution presented our consideration on RF requirement impact for type 4 UE. The following proposal is concluded.

For a UE capable of intraBandNR-CA-non-collocated-r18 for the following CA band combinations in Table 7.10A.2-2, the Power imbalance requirements are applicable with 2Rx antenna ports for each component carrier if it is not provided with nonCollocatedTypeNR-CA-r18 and is configured with maxMIMO-Layers with value less than or equal to 2.

For a UE capable of intraBandNR-CA-non-collocated-r19 for the following CA band combinations in Table 7.10A.2-2, the Power imbalance requirements are applicable

- with 2Rx antenna ports for each component carrier if it is not provided with nonCollocatedTypeNR-CA-r18 and is configured with maxMIMO-Layers with value less than or equal to 2.

- With 4Rx antenna ports for each component carrier if nonCollocatedTypeNR-CA-R19 is present with 0 and is configured with maxMIMO-Layers with value equal to 4.

Proposal 1: Non-collocated requirement in 7.10A should accommodate the case when maxMIMO-layers=4 and maxMIMO-layers=2 for type 4 UE. E.g. below text is proposed.

Proposal 2: To simplify the test, it is proposed to focus on testing of 4Rx antenna port only for type 4 UE.

# References

[1] R4-2500437, On signalling support for type 4 UE, KDDI, RAN4#114