3GPP TSG-RAN WG4 Meeting #114                               						     R4-2500438

# Athens, Greece, 17th – 21st Feburary, 2025

Source: 	Apple

Title: 	On definition of UE types for NonCol_intraB

Agenda Item:	7.5.2.3

Document for:	Discussion

# Introduction

The issue on introducing definition for different type of UEs has ever been discussed in past meetings but not agreed. In the last RAN4 meeting, a WF on introducing type 1 and type 2 UE definitions [1] was agreed. This contribution will give our view and proposals on this issue.

# Discussion

The following agreement was approved in RAN4#113 meeting.

### <Issue 1-1:  UE Capability Type definition for EN-DC and NR-CA>

Way forward:

- Continue further discussion on following options and conclude it in the next meeting, so that RAN4 can begin Rel-19 38.101-1/101-3 CR work in April meeting.

- Proponents of Option 2 need to propose specific examples in Feb. meeting.

- Option 1:

Add the table into TS 38.101-3: 7.10B	Power imbalance for EN-DC and 38.101-1: 7.10A	Power imbalance for NR-CA.

- Option 2:

Not adding the table in 38.101-1/3 and refer to 38.331 and its sections.

We understand the motivation of option 1 is to make the specification reading easier. While the existing way of using “UE indicating [signalling name]” was achieved after a lot of discussion in RF and RRM room. Though it is a long sentence, it is clear and no ambiguity. With option 1, the agreed wording in [2] is to duplicate piece of information in the signalling description into RAN4 specification. This will cause ambiguity of UE type due to misaligned description and also spec maintenance issue.

We are ok not to introduce anything into the RAN4 specification, e.g. option 2. We are also open to introduce definition for different type of UEs with updated description as agreed in [2].

Option 1 (Modified):

| UE Type | Type description |
| --- | --- |
| Type 1 | A UE supporting intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and an (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133. |
| Type 2 | A UE indicating support of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18. |
| Type 4 | A UE indicating support of [intraBandNR-CA-non-collocated-r19] and nonCollocatedTypeNR-CA-r19. |

Option 2:

- Add the term “type 1 UE”, “type 2 UE” and “type 4 UE”as a clarification in the signaling description in RAN2 spec 38.306 as below

Proposal 1: Consider either option 1 (Modified) or option 2 if RAN4 is going to introduce UE type definition.

The next issue is where to put the definition in 38.101-1/-3. Since type 1 UE is not relevant to section 7.10A, we prefer not to put it in section 7.10A if modified option 1 is adopted. The other candidate places could be section 3 or section 7.1 in 38.101-1/-3.

Proposal 2: Section 3 or section 7.1 in 38.101-1/-3 is better place than section 7.10A.

# Summary

This contribution presented our consideration on UE type definition. The following proposals are presented for discussion.

Proposal 1: Consider either option 1 or option 2 if RAN4 is going to introduce UE type definition.

Proposal 2: Section 3 or section 7.1 in 38.101-1/-3 is better place than section 7.10A.

# References

[1]	R4-2420399, WF on non-collocated intra-band EN-DC and NR-CA, KDDI, RAN4#113

[2] R4-2417114, WF on Rel-19 non-collocated scenario, KDDI, RAN4#112bis