3GPP TSG-RAN WG4 Meeting #114	R4-2500469

Athens, Greece, 17-21 February 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	FR2-1 SSB based L3 measurement delay reduction with CSSF optimization

Agenda item:	7.17.2.2

Document for:	Discussion

Introduction

The discussion about RRM phase 5 enhancements for L3 measurement delay reduction started in the RAN4#110bis meeting. At RAN4#113 meeting, we prioritized the discussion on the optimized CSSF solutions and managed to reach some tentative way forward.  In this contribution we continue the discussion on the details of the solutions and other open issues based on the agreed way forward [1].

General discussion

Measurement procedures

In RAN4#112bis meeting, all the measurement procedures were agreed to be discussed. As the CSSF optimization implies new UE measurement behavior, it would be easier to move forward focusing on very basic measurement procedures. We prefer starting from SSB-based intra-frequency and inter-frequency measurements without MG to simplify the discussion. The applicability to other measurement procedures can be revisited after the baseline solution is concluded.

Issue 2-2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF Agreement in online session: - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Aspect 2: SSB based Inter-frequency measurement without MG, including: - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - CSSFinter for inter-frequency measurement without gap. - Aspect 3: Inter-RAT SSB measurement without MG, including: - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement. - MG related features to be considered in aspect 1/2/3 including: - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP - R17 NCSG measurement with ‘nogap-noncsg’ - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”

- To start from SSB-based intra-frequency and inter-frequency measurements without MG to simplify the CSSF optimization discussion.

Prioritization of EN-DC/SA scenarios

At RAN4#112 meeting, it was agreed to consider the CSSF optimization for all the scenarios. There was further proposal to prioritize some of the scenarios. In our view, we can focus the discussion on the CSSF optimization solutions in SA scenarios due to simplicity. The same principle can be applied to update CSSF tables in other scenarios afterwards.

- To focus the discussion on the CSSF optimization solutions in SA scenarios. The same principle can be applied to update CSSF tables in other scenarios afterwards.

Solutions for CSSF optimization

Tentative Solution 1: measure one serving CC per band

In last meeting, there was consensus to allow the UE to measure one serving CC per band if multiple serving CCs are in the same band. But companies still have different opinions if such optimized measurement is based on UE determination i.e. Option 1A or network indication i.e. Option 1B. After some offline coordination, a tentative way forward was recommended as below:

Issue 2-4-1: Tentative solution 1: measure one serving CC per band - Option 1 (CATT, Nokia, CMCC, CTC, Apple, vivo, Ericsson, ZTE, MTK): UE only needs to measure one serving CC per band if multiple serving CCs are in the same band - Option 1A (CATT, Apple, vivo, ZTE): Reuse the legacy principle for neighbour cell measurement (UE can measure one serving CC per band if multiple serving CCs are in the same band). - Measure PCC if PCC in band or - Measure PSCC if PSCC in band or - Measure SCC with neighbor cell MO or - UE implementation - Option 1B (Nokia, CMCC, CTC, Ericsson, ZTE): Based on network indication - The other CCs (not measured based on network indication) need not to be measured and reported. Network assume the same measurement results on the measured CCs and unmeasured CCs in the same band. - Network indicates which of the MOs to be measured. The UE is required to measure the indicated CCs i.e. MO and network can trigger measurement reporting on any of the MOs as in legacy - In case of no NW configuration/indication, UE is allowed to determine the CC(s) to be measured, but the rules shall be studied further. (Ericsson) - Further discussion is needed to justify the introduction of a new NW indication for Option 1B (Huawei) [Way forward]: FFS the following solution: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured, - Reuse the legacy principle for neighbour cell measurement - Measure PCC if PCC in band or - Measure PSCC if PSCC in band or - Measure SCC with neighbour cell MO or - UE implementation - FFS whether indication is per-UE or per-band - Other options are not precluded. - otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

In our view, we can see the benefit of minimizing the measurements on multiple CCs in some scenarios e.g. FR2 intra-band, but this needs to be under network control.

According to legacy measurement framework, the UE measurement behaviour follows network configuration, which configures all potential candidate target cells to be measured for handover decision. However, when UE is measuring one CC per band, network needs to understand this measurement result is applied to all the carriers on the same band. Due to unexpected channel variation, it is at the risk of inaccurate measurement result for non-measured cells and may impact the mobility performance.

Hence, the solution of measuring one CC per band introduces new UE measurement behaviour and may lead to different system performance which shall be allowed by network. An explicit network indication is needed to allow the UE to measure one serving CC per band. If network does not provide the indication, the UE will follow legacy measurement behaviour i.e. to measure all serving CCs configured with measurement.

- The solution of measuring one CC per band introduces new UE measurement behaviour and may lead to different system performance which shall be allowed by network.

- An explicit network indication is needed to allow the UE to measure one serving CC per band. If network does not provide the indication, the UE will follow legacy measurement behaviour i.e. to measure all serving CCs configured with measurement.

In last meeting, it was raised to discuss if the indication is per-UE or per-band. For simplicity, network may indicate a UE to perform relaxed measurements on all the bands in the band combination. We would see per-UE indication as a default type. Besides the network may further indicate to perform optimized measurements on specific bands which provide more flexible control on the UE measurement behavior. Hence, we suggest a per-UE indication as a default. Network may provide per-band indication if needed.

- The indication is per-UE as a default. Network may provide per-band indication if needed.

When the optimized measurement is allowed by network, another question is how to determine the single CC to be measured on the band. There was proposal to rely on network configuration i.e. to configure with servingCellMO only on one CC per band and no MOs are configured on the other CCs. With such configuration, the UE is not expected to measure the other CCs nor sending any measurement report associated with these CCs. In current network deployment, network cannot configure any measurement events on these CCs due to lack of MOs, hence not possible to command handover to any of these cells including serving and neighbor cells. This would degrade the mobility performance due to limited single target cell for handover.

In order to ensure the mobility performance and minimize the impact to network behaviour, network shall be allowed to configured multiple MOs in a band irrespective if the UE is performing optimized measurement. In case a UE is indicated to measure one serving CC on the band, it will select one CC to be measured and skip the measurement on other configured MOs.

- If the UE is configured with single MO per band, it is not able to trigger a measurement report for a CC which the UE is not measuring, and network cannot command handover to any of the other cells on the CC.

- Network is allowed to configure multiple MOs in a band irrespective if the UE is performing optimized measurement i.e. measuring one serving CC per band.

According to the requirements for FR2 measurement in TS 38.133 clause 9.2.3.2, UE is allowed to select one CC per band for neighbor cell measurement. Some principles are specified as below which we believe can be the baseline for measuring one serving CC per band.

TS38.133:

9.2.3.2	Requirements for FR2

For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	6 identified cells, and

-	24 SSBs with different SSB index and/or PCI,

where this single intra-frequency layer shall be:

-	PCC when UE is configured with SA NR operation mode with PCC in the band; or

-	PSCC when UE is configured with EN-DC with PSCC in the band; or

-	PSCC when UE is configured with NR-DC with PSCC in the band; or

-	One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.

The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

However, there is ambiguity at network side on the selected measured CC if there is no PCC/PSCC or an SCC configured with SSB based measurement in the band. To keep the aligned understanding on the measured CC in a band, network can further indicate on which of the MOs the measurement is to be performed. Based on the indication, the UE will measure the corresponding CCs indicated by network in the band.

- Network can further indicate which of the MOs to be measured in a band if there is no PCC/PSCC or an SCC configured with SSB based measurement in the band.

In legacy FR2 measurement requirements, a UE is already allowed to perform neighbor cell measurement on single CC per band. On the band without no PCC/PSCC or an SCC configured with SSB based measurement, it is up to UE implementation to select the CC to be measured. When the same principle is applied to serving cell measurement, the selected CC may be different from the CC to be measured by the network. To simplify the UE measurement, if network indicates which of the MOs to be measured in a band, UE will perform both serving and neighbor cell measurements on the indicated CC.

- if network indicates which of the MOs to be measured in a band, UE will perform both serving and neighbor cell measurements on the indicated CC in a band.

In order to show our understanding on the solution 1, we provide the text proposal about how the measurement requirements may be impacted by measuring one CC per band.

Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs

Another proposal for CSSF optimization is allow the UE to adjust the searcher occupancy ratio assumed for the measurement. The idea is to release the searcher reserved for PCC/PSCC measurement to be used by the SCCs.

Issue 2-4-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs [Wayforward]: FFS: whether solution 2 is needed if solution 1 is agreed in issue 2-4-1: - UE may change the searcher occupancy ratio of SCC measurement to speed up measurement. - FFS whether network indication is needed - Details is FFS - FFS on whether and how PCC and PSCC resource sharing is supported

In current specification, CSSF values are derived based on 2 searcher assumption with certain occupancy ratios. In particular, one searcher is dedicatedly used for PCC/PSCC measurement, and the other searcher is shared for the measurement on all SCCs. And for the second searcher, it may not be equally shared among all SCCs. In some cases the FR2 SCC configured with neighbor cell measurement is further assumed to consume 50% of the searcher hence only 50% is used by the other FR1 and FR2 SCCs. With the increased number of SCCs, the measurement delay can be quite long due to limited searcher capacity.

However, the increase of searcher occupancy ratio for one CC is at the cost of sacrificing the measurement performance on some other CCs due to the overall fixed searcher capacity. If the searcher dedicated for PCC/PSCC measurement is used by SCCs, this would lead to extended measurement delay on PCC/PSCC which will bring severe consequences e.g. on radio link monitoring, handover etc. In addition, UE is already allowed to release the searcher for PCC/PSCC measurement to be used by other CCs if it is not used in implementation, but the measurement requirement shall not be impacted. Hence the solution shall not relax the measurement requirement on PCC/PSCC measurement.

- The increase of searcher occupancy ratio for one CC is at the cost of sacrificing the measurement performance on some other CCs due to the overall fixed searcher capacity.

- The solution of changing searcher occupancy ratio shall not relax the measurement requirement on PCC/PSCC measurement.

To improve the measurement delay for SCCs, we may consider redistributing the occupancy ratio of the second searcher among all SCCs. For example, the 50% of the searcher dedicated to one FR2 SCC, if available, can be used by other SCCs. In order to minimize the negative impact on the prioritized SCC, we may further discuss the conditions under which the change of searcher occupancy ratio is feasible.

- To discuss the conditions under which the change of searcher occupancy ratio is feasible.

While the CSSF optimization solutions are being discussed separately, we need to consider the combination of these candidate solutions in real deployment. If solution 1 is configured by the network, UE is required to measure one CC in a band. This would leave smaller number of SCCs to be measured e.g. the same as the number of bands in a CA band combination. The benefits from changing searcher occupancy ratio would be rather limited if solution 1 is enabled by network. RAN4 shall discuss the applicability of this solution considering the other CSSF optimization solutions.

- The benefits from changing searcher occupancy ratio would be rather limited if solution 1 i.e. measuring one CC per band is enabled by network.

- RAN4 to discuss the applicability of changing searcher occupancy ratio keeping the other CSSF optimization solutions in mind.

Tentative solution 3: 3 searchers for CSSF

In last meeting, there were different views on the applicable scenarios where 3 searchers can be assumed. We understood there was consensus on supporting 3 searchers in FR1 scenarios, but some UE vendors have difficulty supporting 3 searchers on FR2 SCCs which requires higher buffering capacity than on FR1.

Issue 2-4-3: Tentative solution 3: 3 searchers for CSSF Agreement: RAN4 to study the scenario of using 3 searchers for CSSF reduction. The scenario is: - FFS: FR1 scenario (CA and NR-DC) only. - FFS: FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC) only. - FFS: All scenarios.

In our views, the debate is not if FR2 is comprised in the CA scenarios as listed below, instead, the key issue is how the 3rd searcher is distributed among the CCs. For instance, in FR1+FR2 scenario, if the 3rd searcher is not used for FR2 measurement i.e. the same searcher occupancy ratio is assumed for FR2 SCCs, this will be equivalent to applying the 3 searchers to FR1 scenarios only. To move forward, we can consider 3 searchers at least for FR1 scenarios. In addition, RAN4 shall discuss the maximum number of searchers that can be assumed for FR2 SCCs in FR1+FR2 scenario.

- The debate on the 3 searchers is not if FR2 is comprised in the CA scenarios as listed below, instead, the key issue is how the 3rd searcher is distributed among the CCs.

- To consider 3 searchers at least for FR1 scenarios (CA and NR-DC).

- RAN4 to discuss the maximum number of searchers that can be assumed for FR2 SCCs in FR1+FR2 scenario.

Comparing to the legacy 2 searcher assumption, one more additional searcher is available in this solution. To simplify the CSSF calculation, we suggest not changing the searcher occupancy ratios for the legacy 2 searchers, and focus the discussion on how to use the 3rd searcher when scenarios are concluded.

- We suggest not changing the searcher occupancy ratios for the legacy 2 searchers, and focus the discussion on how to use the 3rd searcher when scenarios are concluded.

Other candidate solutions

For the measurements on SCCs, UE has been assumed to share one searcher and perform the measurements over multiple SCCs in alternating manner. This leads an increased measurement delay being scaled by the number of CCs to be measured. However, the UE is able to receive simultaneously from the serving CCs e.g. in CA operation, hence it is able to receive SSBs from multiple configured CCs at the same time e.g. within one SMTC window if it is configured with same SMTC on the multiple CCs. This will allow the UE to get the SSB samplings from the multiple CCs in one shot. Afterwards, the UE may process the samples in sequential way offline in order to derive the measurement results for the multiple CCs respectively.

With such measurement behaviour, some additional processing time may be needed but the measurement delay is not scaled by the number of configured CCs hence can be significantly decreased. We would like to define the optimized CSSF assuming the UE is able to sample SSBs simultaneously from multiple CCs and process these SSB samples offline.

- UE is able to receive SSBs (i.e. get SSB samplings) from multiple configured CCs at the same time e.g. within one SMTC window.

- RAN4 to define optimized CSSF assuming the UE is able to sample SSBs simultaneously from multiple CCs and process these SSB samples offline.

Impact from CSSF optimization

In above discussion, several candidate solutions are being developed to allow optimized measurements on multiple CCs. In particular, solution 1 allows the UE to measure one CC in a band which minimizes the measurements hence reduce the measurement delay. However, this is at the cost of skipping the measurements on some serving cells. The missing of the measurements and reporting for these cells will bring impacts on other RRM requirements.

- If a UE is measuring one CC in a band, the missing of the measurements and reporting for the non-measured cells will bring impacts on other RRM requirements.

One example is SCell activation delay. Currently SCell activation delay is based on if the UE has sent a valid measurement reporting for the to-be-activated SCell before receiving the activation command. If UE is indicated to enable the optimized measurement i.e. to measure one CC per band, the other serving cells on the same band will not be measured or reported. These serving cells will be considered as unknown according to current definition hence a longer SCell activation delay requirement would apply.

However, the UE measuring one CC per band is under network control which implies the measurement report for the single CC will be assumed applicable to the whole band. The SCell, although not measured in practice, could have been activated as if it is known by referring to the measurement reporting for the measured cell in the same band. Hence the SCell activation delay requirement shall be improved considering the optimized measurement for CSSF reduction.

- The SCell activation delay requirement shall be improved considering the optimized measurement for CSSF reduction.

Conclusion

The following observations and proposals were made in this paper:

- To start from SSB-based intra-frequency and inter-frequency measurements without MG to simplify the CSSF optimization discussion.

- To focus the discussion on the CSSF optimization solutions in SA scenarios. The same principle can be applied to update CSSF tables in other scenarios afterwards.

- The solution of measuring one CC per band introduces new UE measurement behaviour and may lead to different system performance which shall be allowed by network.

- An explicit network indication is needed to allow the UE to measure one serving CC per band. If network does not provide the indication, the UE will follow legacy measurement behaviour i.e. to measure all serving CCs configured with measurement.

- The indication is per-UE as a default. Network may provide per-band indication if needed.

- If the UE is configured with single MO per band, it is not able to trigger a measurement report for a CC which the UE is not measuring, and network cannot command handover to any of the other cells on the CC.

- Network is allowed to configure multiple MOs in a band irrespective if the UE is performing optimized measurement i.e. measuring one serving CC per band.

- Network can further indicate which of the MOs to be measured in a band if there is no PCC/PSCC or an SCC configured with SSB based measurement in the band.

- if network indicates which of the MOs to be measured in a band, UE will perform both serving and neighbor cell measurements on the indicated CC in a band.

- The increase of searcher occupancy ratio for one CC is at the cost of sacrificing the measurement performance on some other CCs due to the overall fixed searcher capacity.

- The solution of changing searcher occupancy ratio shall not relax the measurement requirement on PCC/PSCC measurement.

- To discuss the conditions under which the change of searcher occupancy ratio is feasible.

- The benefits from changing searcher occupancy ratio would be rather limited if solution 1 i.e. measuring one CC per band is enabled by network.

- RAN4 to discuss the applicability of changing searcher occupancy ratio keeping the other CSSF optimization solutions in mind.

- The debate on the 3 searchers is not if FR2 is comprised in the CA scenarios as listed below, instead, the key issue is how the 3rd searcher is distributed among the CCs.

- To consider 3 searchers at least for FR1 scenarios (CA and NR-DC).

- RAN4 to discuss the maximum number of searchers that can be assumed for FR2 SCCs in FR1+FR2 scenario.

- We suggest not changing the searcher occupancy ratios for the legacy 2 searchers, and focus the discussion on how to use the 3rd searcher when scenarios are concluded.

- UE is able to receive SSBs (i.e. get SSB samplings) from multiple configured CCs at the same time e.g. within one SMTC window.

- RAN4 to define optimized CSSF assuming the UE is able to sample SSBs simultaneously from multiple CCs and process these SSB samples offline.

- If a UE is measuring one CC in a band, the missing of the measurements and reporting for the non-measured cells will bring impacts on other RRM requirements.

- The SCell activation delay requirement shall be improved considering the optimized measurement for CSSF reduction.

- References

- R4-2420093, Way Forward for NR_RRM_Ph5_Part1, Apple

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5

- Appendix

### <Start of Change 1>

##### 9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps

For UE in SA operation mode, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements, inter-frequency SSB-based measurements performed outside measurements gaps, E-UTRA inter-RAT measurement object without measurement gap, intra-frequency CSI-RS L3 measurement and RSSI/channel occupancy measurement with no measurement gap on a carrier subject to CCA when SMTC and RMTC are overlapping will be as specified in table 9.1.5.1.2-1, which shall also be applied for a UE configured with NE-DC operation.

For UE capable of measuring one CC per FR2 band in SA operation mode, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements, inter-frequency SSB-based measurements performed outside measurements gaps, E-UTRA inter-RAT measurement object without measurement gap, intra-frequency CSI-RS L3 measurement and RSSI/channel occupancy measurement with no measurement gap on a carrier subject to CCA when SMTC and RMTC are overlapping will be as specified in table 9.1.5.1.2-2 provided network indicates  [measuringOneservingCCperBand] to the UE, which shall also be applied for a UE configured with NE-DC operation.

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap | CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 only CA | 1+NPCC_CSIRS + NPCC_CCA_RSSI/CO | NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO | N/A | N/A | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| FR2 only intra band CA | N/A | N/A | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| FR2 only inter band CA | N/A | N/A | 1+NPCC_CSIRS | 2*(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| FR1 +FR2 CA (FR1 PCell) Note 1 | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+Z+2* NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | N/A | 2x(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| FR1 +FR2 CA (FR2 PCell) Note 1 | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| NOTE 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. NOTE 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. NOTE 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i =2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. NOTE 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. NOTE 5: Only two NR FR2 operating bands are included for FR2 inter-band CA. NOTE 6: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. NOTE 7: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured NOTE 8: NSCC_CSIRS_FR2_NCM=1 if FR2 SCC, where neighbour cell measurement is required, is with either both SSB and CSI-RS configured or only CSI-RS measurement configured; otherwise, NSCC_CSIRS_FR2_NCM=0. NOTE 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. For UE capable of measuring one CC per FR2 band, measurin NOTE 10: NPCC_CCA_RSSI/CO= 1 if PSCC is configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping; NSCC_CCA_RSSI/CO = Number of MOs for SCell(s) configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping. NOTE 11: Z is the number of configured E-UTRA inter-RAT MOs without MG that are being measured outside of MG; otherwise, it is 0. |  |  |  |  |  |  |  |

Table 9.1.5.1.2-2: CSSFoutside_gap,i scaling factor for SA mode for UE capable of measuring one CC per band

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap | CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 only CA | 1+NPCC_CSIRS + NPCC_CCA_RSSI/CO | NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO | N/A | N/A | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| FR2 only intra band CA | N/A | N/A | 1+NPCC_CSIRS | N/A | N/A | Y+Z | Y+Z |
| FR2 only inter band CA | N/A | N/A | 1+NPCC_CSIRS | 2*(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | N/A | 2*(Y+Z) | 2*(Y+Z) |
| FR1 +FR2 CA (FR1 PCell) Note 1 | 1+NPCC_CSIRS | 2*(NSCC_SSB_FR1+Y+Z+2* NSCC_CSIRS_FR1) | N/A | 2*(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | N/A | 2×( NSCC_SSB_FR1 +Y+Z+2x NSCC_CSIRS_FR1) | 2×( NSCC_SSB_FR1 +Y+Z+2x NSCC_CSIRS_FR1) |
| FR1 +FR2 CA (FR2 PCell) Note 1 | N/A | NSCC_SSB_FR1 +Y+Z+2x NSCC_CSIRS_FR1 | 1+NPCC_CSIRS | N/A | N/A | NSCC_SSB_FR1 +Y+Z+2x NSCC_CSIRS_FR1 | NSCC_SSB_FR1 +Y+Z+2x NSCC_CSIRS_FR1 |
| NOTE 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. NOTE 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. NOTE 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i =2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. NOTE 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. NOTE 5: Only two NR FR2 operating bands are included for FR2 inter-band CA. NOTE 6: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. NOTE 7: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured NOTE 8: NSCC_CSIRS_FR2_NCM=1 if FR2 SCC, where neighbour cell measurement is required, is with either both SSB and CSI-RS configured or only CSI-RS measurement configured; otherwise, NSCC_CSIRS_FR2_NCM=0. NOTE 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. NOTE 10: NPCC_CCA_RSSI/CO= 1 if PSCC is configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping; NSCC_CCA_RSSI/CO = Number of MOs for SCell(s) configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping. NOTE 11: Z is the number of configured E-UTRA inter-RAT MOs without MG that are being measured outside of MG; otherwise, it is 0. NOTE 12: NSCC_SSB_FR1=Number of configured FR1 SCell(s) with only SSB based L3 measurement configured, which is measured without MG. NOTE 13: NSCC_SSB_FR2=Number of FR2 SCell(s) with only SSB based L3 measurement configured, which is measured without MG. NOTE 13: NSCC_CSIRS_FR1=Number of configured FR1 SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured. |  |  |  |  |  |  |  |

### <End of Change 1>

### <Start of Change 2>

#### 9.2.3.2	Requirements for FR2

For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	6 identified cells, and

-	24 SSBs with different SSB index and/or PCI,

where this single intra-frequency layer shall be:

-	PCC when UE is configured with SA NR operation mode with PCC in the band; or

-	PSCC when UE is configured with EN-DC with PSCC in the band; or

-	PSCC when UE is configured with NR-DC with PSCC in the band; or

-	One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.

The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

#### 9.2.3.2x	Requirements for FR2 for UE capable of measuring one serving CC per band

For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	6 identified cells, and

-	24 SSBs with different SSB index and/or PCI,

where this single intra-frequency layer shall be:

-	PCC when UE is configured with SA NR operation mode with PCC in the band; or

-	PSCC when UE is configured with EN-DC with PSCC in the band; or

-	PSCC when UE is configured with NR-DC with PSCC in the band; or

-	One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists; or

-	The SCC explicitly indicated to be measured when neither PCC nor PSCC is in the same band and there is no SCC where the UE is configured with SS-RSRP measurement reporting, otherwise the selected SCC is determined by UE implementation.

The requirement applies only if network indicates [measuringOneservingCCperBand] to the UE.

### <End of Change 2>