3GPP RAN WG4 Meeting #114		R4-2500481

# Athens, Greece, 17th – 21st February 2025

Agenda item:	7.14.2.3

Source:	Apple

Title:	On FR1 MIMO OTA Dynamic Channel Model

WI/SI:	TRP_TRS_MIMO_OTA_Ph3

Release:	Rel-19

Document for:	Endorsement

# 1	Introduction

During RAN4 #113 a Text Proposal [1] to TR 38.762 [2] was submitted for approval.

This TP to TR 38.762 brings the following Introduction and discussion:

As indicated in the statement highlighted in yellow, indeed, CTIA MIMO OTA Sub-Working Group (MOSG)has been undergoing an extensive effort for the last 3 years to define the FR1 MIMO OTA Dynamic Channel Model. Keysight and Spirent are championing the alignment between Channel Emulators and supporting system integrators with CE loaners and defining Channel Model validation targets. These efforts are being registered in a CTIA still unpublished test plan “Test Methodology, MIMO, Multi-Probe Anechoic Chamber, Dynamic Channel Model with Link Adaptation”. Despite not being published yet, the latest draft of this test plan is available to all CTIA OTA members.

# 2	Evaluation of FR1 MIMO OTA Dynamic Channel Model Validation targets

The following Figures and Tables indicates the discrepancy between CTIA and R4-24119397 [1] for UMa, as follows:

Figure 1a, CTIA Target PDPs Segments of the Dynamic UMa Channel Model

Figure 1b, R4-24119397 [1] proposal for Target PDPs Segments of the Dynamic UMa Channel Model, relevant difference from CTIA test plan.

Table 2a, CTIA Dynamic PDP Targets of the UMa Route

| Segment # |  | Target PDP |
| --- | --- | --- |
| 1 | Delay τ [ns] | [0, 130, 170, 245, 380, 745, 885] |
|  | Power P1τ [dB] | [0, -18.5, -20.2, -22.5, -36.0, -25.5, -31.8] |
| 2 | Delay τ [ns] | [0, 130, 170, 245, 380, 745, 885] |
|  | Power P2τ [dB] | [0, -19.2, -20.9, -23.2, -32.8, -23.9, -34.5] |
| 3 | Cluster index l | 1, 2, ..., 23 |
|  | Power P3,l [dB] | [-9.7, 0.0, -1.8, -6.2, -8.1, -6.4, -3.3, -8.8, -8.4, -11.5, -8.6, -16.3, -9.9, -13.3, -12.1, -12.9, -29.3, -28.9, -18.3, -19.4, -17.6, -31.9, -29.9] |
| 4 | Cluster index l | 1, 2, ..., 23 |
|  | Power P4,l [dB] | [-15.1, 0.0, -2.0, -9.9, -12.0, -14.5, -3.9, -13.5, -6.8, -21.2, -6.4, -15.9, -19.3, -17.6, -12.3, -11.3, -28.2, -29.1, -22.6, -18.2, -17.3, -30.4, -28.0] |
| 5 | Cluster index l | 1, 2, ..., 23 |
|  | Power P5,l [dB] | [-8.2, -0.7, -3.5, -3.4, -5.1, 0.0, -1.6, -3.7, -10.2, -9.8, -13.1, -22.3, -7.8, -13.5, -16.0, -18.6, -23.0, -21.4, -22.9, -22.2, -20.0, -26.5, -32.8] |
| 6 | Delay τ [ns] | [0, 80, 235, 300, 340, 445, 475, 790, 985, 1550, 1675, 2000, 2040, 2295, 2415, 2565] |
|  | Power P6τ [dB] | [-13.0, -3.4, 0.0, -12.6, -33.4, -15.9, -19.8, -25.2, -17.9, -17.7, -16.3, -39.7, -20.4, -19.0, -20.6, -29.3] |
| 7 | Delay τ [ns] | [0, 50, 65, 175, 240, 335, 495, 1090, 1875] |
|  | Power P7τ [dB] | [0, -17.1, -21.2, -15.0, -21.0, -24.9, -19.9, -32.9, -32.8] |
| 8 | Cluster index l | 1, 2, ..., 23 |
|  | Power P8,l [dB] | [0.0, -3.6, -5.9, -9.5, -13.1, -14.2, -7.5, -12.7, -12.9, -19.0, -11.2, -20.0, -23.4, -20.6, -24.7, -26.0, -21.2, -23.7, -24.4, -31.5, -34.0, -25.6, -36.9] |
| 9 | Delay τ [ns] | [0, 60, 130, 170, 245, 380, 745, 885, 1180] |
|  | Power P9τ [dB] | [0, -36.1, -17.9, -19.5, -22.0, -38.8, -27.0, -29.5, -38.7] |

Table 2b, R4-24119397 [1] proposal for Dynamic PDP Targets of the UMa Route, differences from CTIA test plan.

| Segment # |  | UMa Target PDP |
| --- | --- | --- |
| 1 | Delay τ [ns] | [0, 130, 170, 245, 380, 745, 885] |
|  | Power P1τ [dB] | [0, -18.5, -20.2, -22.5, -36.0, -25.5, -31.9] |
| 2 | Delay τ [ns] | [0, 130, 170, 245, 380, 745, 885] |
|  | Power P2τ [dB] | [0, -19.2, -20.9, -23.2, -32.8, -23.9, -34.5] |
| 3 | Cluster index l | 1, 2, ..., 23 |
|  | Power P3,l [dB] | [-9.8, 0.0, -1.9, -6.3, -8.2, -6.6, -3.3, -8.9, -8.3, -11.7, -8.5, -16.3, -10.0, -13.4, -12.1, -12.8, -29.3, -29.0, -18.4, -19.3, -17.5, -31.9, -29.8] |
| 4 | Cluster index l | 1, 2, ..., 23 |
|  | Power P4,l [dB] | [-15.0, 0.0, -2.0, -9.7, -11.8, -14.2, -3.9, -13.3, -6.9, -20.8, -6.5, -16.0, -19.0, -17.5, -12.3, -11.3, -28.2, -29.1, -22.3, -18.2, -17.2, -30.3, -28.0] |
| 5 | Cluster index l | 1, 2, ..., 23 |
|  | Power P5,l [dB] | [-8.2, -0.8, -3.7, -3.5, -5.2, 0.0, -1.6, -3.7, -10.4, -10.1, -13.4, -22.5, -7.9, -13.5, -16.2, -18.9, -23.1, -21.4, -23.4, -22.5, -20.3, -26.5, -33.1] |
| 6 | Delay τ [ns] | [0, 80, 235, 300, 340, 445, 475, 790, 985, 1550, 1675, 2000, 2040, 2295, 2415, 2565] |
|  | Power P6τ [dB] | [-13.0, -3.3, 0.0, -12.6, -33.3, -15.8, -19.7, -25.1, -18.0, -17.8, -16.4, -39.6, -20.4, -19.0, -20.7, -29.4] |
| 7 | Delay τ [ns] | [0, 50, 65, 175, 240, 335, 495, 1090, 1875] |
|  | Power P7τ [dB] | [0, -17.1, -21.2, -15.0, -21.0, -24.9, -19.9, -32.8, -32.8] |
| 8 | Cluster index l | 1, 2, ..., 23 |
|  | Power P8,l [dB] | [0.0, -5.7, -7.9, -11.5, -15.2, -15.4, -9.1, -14.1, -14.0, -19.6, -12.3, -19.9, -25.2, -20.9, -26.6, -27.9, -23.1, -25.6, -26.3, -33.4, -35.9, -27.5, -38.8] |
| 9 | Delay τ [ns] | [0, 60, 130, 170, 245, 380, 745, 885, 1180] |
|  | Power P9τ [dB] | [0, -36.1, -17.9, -19.5, -22.0, -38.8, -27.0, -29.5, -38.7] |

Observation 1:	The CTIA test plan prioritizes UMa. Testing using the UMi route is planned for a future release of this test plan. Therefore, UMi discrepancies are not analyzed in this paper.

Observation 2:	The main PDP validation targets discrepancy between CTIA and proposed on R4-24119397 [1], are on segments #3, 5, and 8.

Table 2a, CTIA Dynamic Temporal Correlation Targets of the UMa Route

| Segment # | Target TCF at Δt=4.6 ms | Target TCF at Δt=20.2 ms |
| --- | --- | --- |
| 1 | [0.970] | [0.876] |
| 2 | [0.961] | [0.879] |
| 3 | [0.994] | [0.886] |
| 4 | [0.440] | [0.378] |
| 5 | [0.643] | [0.351] |
| 6 | [0.521] | [0.093] |
| 7 | [0.798] | [0.221] |
| 8 | [0.864] | [0.205] |
| 9 | [0.940] | [0.616] |

Table 2b, R4-24119397 [1] proposal for Dynamic Temporal Correlation Targets of the UMa Route, differences from CTIA test plan.

| Segment # | Target TCF at Δt=4.6 ms | Target TCF at Δt=20.2 ms |
| --- | --- | --- |
| 1 | [0.970] | [0.876] |
| 2 | [0.961] | [0.879] |
| 3 | [0.994] | [0.886] |
| 4 | [0.436] | [0.355] |
| 5 | [0.648] | [0.345] |
| 6 | [0.523] | [0.100] |
| 7 | [0.801] | [0.228] |
| 8 | [0.837] | [0.399] |
| 9 | [0.940] | [0.616] |

Observation 3:	The main TCF validation targets discrepancy between CTIA and proposed on R4-24119397 [1], are on segments #4, 5, 6, 7, and 8.

Table 3a, CTIA Dynamic Spatial Correlation Targets of the UMa Route for 2450 MHz

| Segment # | Target SCF at Δd=22.9 mm =0.19 λ | Target SCF at Δd=90.5 mm =0.74 λ | Target SCF at Δd=208.0 mm =1.70 λ |
| --- | --- | --- | --- |
| 1 | [0.986] | [0.908] | [0.932] |
| 2 | [0.964] | [0.620] | [0.355] |
| 3 | [0.800] | [0.171] | [0.113] |
| 4 | [0.699] | [0.729] | [0.481] |
| 5 | [0.829] | [0.414] | [0.153] |
| 6 | [0.793] | [0.390] | [0.450] |
| 7 | [0.927] | [0.464] | [0.324] |
| 8 | [0.792] | [0.213] | [0.527] |
| 9 | [0.959] | [0.578] | [0.238] |

Table 3b, R4-24119397 [1] proposal Dynamic Spatial Correlation Targets of the UMa Route for 2450 MHz, differences from CTIA test plan.

| Segment # | Target SCF at Δd=22.9 mm =0.19 λ | Target SCF at Δd=90.5 mm =0.74 λ | Target SCF at Δd=208.0 mm =1.70 λ |
| --- | --- | --- | --- |
| 1 | [0.986] | [0.908] | [0.932] |
| 2 | [0.963] | [0.611] | [0.326] |
| 3 | [0.800] | [0.160] | [0.111] |
| 4 | [0.699] | [0.721] | [0.492] |
| 5 | [0.831] | [0.414] | [0.156] |
| 6 | [0.794] | [0.393] | [0.454] |
| 7 | [0.928] | [0.481] | [0.367] |
| 8 | [0.834] | [0.283] | [0.605] |
| 9 | [0.959] | [0.574] | [0.220] |

Observation 4:	The main Dynamic Spatial Correlation validation targets discrepancy between CTIA and proposed in R4-24119397 [1], are on segments # 5, 7, and 8.

Observation 5:	The Parameter Tables for the Dynamic UMa channel Model are identical between CTIA Test Plan and the R4-24119397 [1] proposal.

The proponents from R4-24119397 [1] theorized that adopting the Path Loss defined in TR 38.901 [3], and considering the different environmental conditions adopted by CTIA (SIR limited) vs. 3GPP (noise limited). Those are the reasons why the proposed PDP, TCF, and Spatial Correlation targets are different.

One can argue that strictly adjusting the Path Loss, can result in power level change. Thus, changing the Path Loss plot and not the PDP since is normalized.

Additionally, the definition of Dynamic channel Model validation targets should not dependent on the definition of environmental variables.

Observation 6:	Path Loss adjustment should not have impact on normalized PDP.

Observation 7:	The Dynamic channel Model validation targets should not depend on the definition of environmental variables.

# 3	Observations and Proposal

Observation 1:	The CTIA test plan prioritizes UMa. Testing using the UMi route is planned for a future release of this test plan. Therefore, UMi discrepancies are not analyzed in this paper.

Observation 2:	The main PDP validation targets discrepancy between CTIA and proposed on R4-24119397 [1], are on segments #3, 5, and 8.

Observation 3:	The main TCF validation targets discrepancy between CTIA and proposed on R4-24119397 [1], are on segments #4, 5, 6, 7, and 8.

Observation 4:	The main Dynamic Spatial Correlation validation targets discrepancy between CTIA and proposed in R4-24119397 [1], are on segments # 5, 7, and 8.

Observation 5:	The Parameter Tables for the Dynamic UMa channel Model are identical between CTIA Test Plan and the R4-24119397 [1] proposal.

Observation 6:	Path Loss adjustment should not have impact on normalized PDP.

Observation 7:	The Dynamic channel Model validation targets should not depend on the definition of environmental variables.

Proposal 1:	RAN4 to adopt a single set of channel model validation values independent of environmental variables for FR1 MIMO OTA Dynamic Channel Model, unifying 3GPP and CTIA CM validation targets.

# 4				References

- R4-24119397, “On Dynamic MIMO OTA Testing”, Keysight Technologies UK Ltd and Spirent Communications.

- TR 38.762, “Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) dynamic test methodology for FR1 UEs”, V0.1.0 Rel 19.

- TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”, V18.0.0 Rel 18.