3GPP TSG-RAN WG4 Meeting #114	R4-2500482

Athens, GR, Feb. 2025

Source	:	Qualcomm Incorporated

Title	:	On the proposed FR2 PC8 (new power class) UE

Agenda item	:	5.33.1

Document for	:	Approval

# 1.	Introduction

Emerging FR2 markets may well have denser deployment and more use cases than current practice in North America. A previous proposal [1] suggested that these new scenarios will be better served by a new UE type that combines the RF front end of a PC7 (RedCap UE) with the full featured baseband capabilities of PC3 (handheld UE). In this contribution we discuss next steps.

# 2. 	Discussion

It was previously proposed to create an FR2 UE (‘PC8’) by combining a full-featured baseband section with an FR2 Redcap UE front end (i.e. PC7, which has  diminished RF capability compared to PC3). In a previous contribution [2], we showed the incompatibility of the stated use cases [1] and the proposed low EIRP capability.

Further discussions showed that there are other scenarios where the low EIRP suffices [3]. While these scenarios may exist, their limited nature and special conditions are cause for concern with standardizing this type of low EIRP UE for non-RedCap eMBB devices for more general usage.

On the one hand we think the FR2 eco-system will benefit from a UE with a different (lower) performance and price point from legacy power classes, so we support the PC8 initiative. On the other hand, we also recognize that PC7 spherical coverage ‘gain drop’ leaves PC8 with an UL that may be useful only in extremely select situations.

For this reason, a compromise would be to improve the ‘gain drop’ to the 50th % direction for PC8. To largely minimize the impact of introduction of these devices in existing networks, PC8 should have the same EIRP spherical coverage requirement as PC3. While this may seem self-defeating for a low-cost device, the state of the art has improved considerably from Rel-15, primarily in power output but also in antenna design. In fact, single module PC3 devices are considered feasible with today’s improvements from 7 years ago. For Rel-19, achieving the same EIRP as PC3 along the 50th %ile direction from 7 years ago may be a good ‘middle of the road’ goal.

Proposal 1: To make the PC8 proposal more palatable to all stakeholders, RAN4 to consider adopting the following requirements for n257/n258/n261:

- min pk. EIRP: 16.4 dBm

- EIRP spherical coverage requirement: 11.5 dbm @ 50th% ile direction

# 3.	References

[1] 	R4-2408997, ‘Discussion on extension FR2 UE power class 7 to non-RedCap UE’,  Huawei, RAN4#111, May. 2024

[2] 	R4-2413227, ‘Discussion on feasibility of FR2 UEs with low EIRP’, Qualcomm, RAN4#112, Aug. 2024

[3] 	R4-2414420, ‘WF on introduction of new FR2 Power Class 8’, Huawei, RAN4#112, Aug. 2024