3GPP TSG-RAN WG4 Meeting #114	R4-2500483

Athens, GR, Feb. 2025

Source	:	Qualcomm Incorporated

Title	:	LS on ambiguity with maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16

Agenda item	:	4.2.1

Document for	:	Approval

# 1.	Introduction

The inconsistency in the default values of dutycycle associated with the capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 between RAN2 and RAN4 must be resolved. RAN4 has clarified that the wording in 38.101-3 was consistent with RAN4 intent, but RAN2 wording seemed to indicate otherwise. So this must be communicated to RAN2.

# 2.	Discussion

## 2.1	RAN4 intent with maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16

In a previous contribution [1], we established the history behind the intent for maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 in RAN4 and identified the discrepancy between RAN2 execution and RAN4 intent.

Confirmation of RAN4 intent based on online discussion is recorded in the meeting report from RAN4#113. In the exceprt below. Te documentation first includes the definition of the issue in the moderator summary [2] and then the outcome of online discussion from the meeting report

Observation 1: RAN4 intended that if maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not signaled, 30% is applied as the default duty cycle capability for the NR branch

Observation 2: RAN4 think RAN2 wording needs to be aligned with 38.101-3

## 2.2	RAN2 signaling definition

On the other hand, 38.306 records the following pertainting to the parameter maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 Indicates the maximum percentage of symbols during a certain evaluation period that can be scheduled for NR uplink transmission under different EUTRA TDD uplink-downlink configurations so as to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. This field is only applicable for inter-band TDD+TDD EN-DC power class 2 UE as specified in TS 38.101-3 [4]. If the field is absent, 30% shall be applied to all EUTRA TDD uplink-downlink configurations. If eutra-TDD-Configx is absent, 30% shall be applied to the corresponding EUTRA TDD uplink-downlink configuration. Value n20 corresponds to 20%, value n40 corresponds to 40% and so on. | BC | No | TDD only | FR1 only |

This wording come from RAN4, in the Rel-16 feature list [3]:

Observation 3: RAN2 execution of maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 in signaling is consistent with RAN4’s request

The highlighted section documents that RAN4 asked RAN2 to apply the default value to (an unspecified parameter inside) each of the EUTRA TDD uplink-downlink configurations. This unspecified parameter of course is the UL duty cycle limit pertaining to the NR branch, but its absence can cause readability problems.

Observation 4: There is a readability problem with how RAN4 requested the signaling for maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 be implemented

Readability would be improved for non-RAN2 readers if  the following inclusion is made:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 Indicates the maximum percentage of symbols during a certain evaluation period that can be scheduled for NR uplink transmission under different EUTRA TDD uplink-downlink configurations so as to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. This field is only applicable for inter-band TDD+TDD EN-DC power class 2 UE as specified in TS 38.101-3 [4]. If the field is absent, 30% for NR uplink transmission shall be applied to all EUTRA TDD uplink-downlink configurations. If eutra-TDD-Configx is absent, 30% for NR uplink transmission shall be applied to the corresponding EUTRA TDD uplink-downlink configuration. Value n20 corresponds to 20%, value n40 corresponds to 40% and so on. | BC | No | TDD only | FR1 only |

Proposal 1: Send LS ot RAN2 requesting readability enhancement to be added to the originally requested wording.

# 3.	Conclusion

Observation 1: RAN4 intended that if maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not signaled, 30% is applied as the default duty cycle capability for the NR branch

Observation 2: RAN4 think RAN2 wording needs to be aligned with 38.101-3

Observation 3: RAN2 execution of maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 in signaling is consistent with RAN4’s request

Observation 4: There is a readability problem with how RAN4 requested the signaling for maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 be implemented

Proposal 1: Send LS ot RAN2 requesting readability enhancement to be added to the originally requested wording.

# 4.	References

[1]	R4-2417636, ‘Discrepancy between 38.306 and 38.101-3 on maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16’, Qualcomm, RAN4#113

[2]	R4-2418123, ‘Topic summary for [113][101] Upto_R17_UERF_maintenance’, Moderator (Oppo), RAN4#113

[3]	R4-2118537, Updated RAN4 UE features list for Rel-16 ‘’, CMCC, RAN4#101-e

# 5.	LS draft

Title:	LS on ambiguity with maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16

Response to:	-

Release:	Rel-16

Work Item:	DC_R17_1BLTE_1BNR_2DL2UL

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Sumant Iyer

sumanti_at_qti.qualcomm.com

# 1	Overall description

The IE maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is correctly implemented by RAN2 per the request from RAN4 during the Rel-16 work phase. There however may be a readability problem with the originally proposed RAN4 wording.

The clarified version of maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 according to RAN4 is:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 Indicates the maximum percentage of symbols during a certain evaluation period that can be scheduled for NR uplink transmission under different EUTRA TDD uplink-downlink configurations so as to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. This field is only applicable for inter-band TDD+TDD EN-DC power class 2 UE as specified in TS 38.101-3 [4]. If the field is absent, 30% for NR uplink transmission shall be applied to all EUTRA TDD uplink-downlink configurations. If eutra-TDD-Configx is absent, 30% for NR uplink transmission shall be applied to the corresponding EUTRA TDD uplink-downlink configuration. Value n20 corresponds to 20%, value n40 corresponds to 40% and so on. | BC | No | TDD only | FR1 only |

# 2	Actions

ACTION: 	RAN4 requests RAN2 to kindly update the definition of maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 at their convenience.

# 3	Dates of next TSG RAN WG 4 meetings

3GPP RAN4#114-Bis       		        April 2025				 Orlando, US

3GPP RAN4#115       		        May 2025                           		 Malta