3GPP TSG-RAN WG4 Meeting #114	R4-2500507

# Athens, Greece, 17th - 21th, Feb 2025

Title:	Some initial test for RC harmonization at FR1 low band

Source:	Samsung

Agenda item:	7.14.3.1

Document for:	Discussion

1.	Introduction

In RAN plenary December meeting, RC chamber harmonization with AC was added as Rel-19 objective [1]:

In this contribution we share some initial test results for information.

2. 	Discussion

2.1 Performance metric

Some potential solutions to improve RC performance at FR1 low band was given in [2]:

Issue 4-2-6: For RC low band harmonization issue in Rel-19 Agreements: - Update WID in next RAN-P to add RC harmonization scope. RAN4 further discuss how to resolve the low band RC harmonization issue. - Consider the following solutions: - Increase the number of sample points per TRS measurement. - Provide a separate uplink path in the chamber for TRS. - Define a minimum system dwell time at each position for device adaptation.

Among above 3 solutions, we have tried testing with the first two solutions, i.e., increasing the TRS sample points to 200, and providing a separate uplink path for TRS. The initial results are summarized below:

| Samsung RC lab | Old (202407) | New (202502) | Reference value (dBm) | AC Reference value (dBm) |
| --- | --- | --- | --- | --- |
| LAD3 (head+hand) | -85.8 | -85.2 | -83.05 | -84.53 |
| LAD4 (head+hand) | -84.5 | -83.7 | -82.35 | -83.36 |
| LAD3 (hand only) | -88.5 (previous disclosed data -88.2 is typo and corrected to -88.5 lately) | -88.3 | -85.89 | N/A |
| LAD4 (hand only) | -87.1 | -86.7 | -85.30 | N/A |

It can be observed that new test results becoming closer to the AC reference value or the RC reference value.

Observation 1:	Initial test by increasing the TRS sample points to 200 and providing a separate uplink path for TRS improves RC harmonization performance by 0.2~0.8dB at FR1 low band TRS.

3. 	Conclusion

Observation 1:	Initial test by increasing the TRS sample points to 200 and providing a separate uplink path for TRS improves RC harmonization performance by 0.2~0.8dB at FR1 low band TRS.

4. 	References

- RP-243277    “Revised WID: WI on FR1 TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3”, vivo, CAICT

- R4-2419808    “WF for [113][325] TRP_TRS_Ph3”, vivo