3GPP TSG-RAN WG4 Meeting # 114											R4-2500534

Athens, GR, 17th ‒ 21st February, 2025

Agenda item:			7.22.4

Source:	Moderator (Samsung)

Title:	Topic summary for [114][220] NR_MIMO_Ph5_Part1

Document for:	Information

Introduction

This topic summary covers the contributions submitted under the following AI for first part of Rel-19 RRM for NR MIMO Phase 5:

- 7.22	NR MIMO Phase 5

- 7.22.3 RRM core requirements

  - 7.22.3.1 Enhancement for UE-initiated/event-driven beam management

Recommended issues to online discussion:

Issue 1-1

Issue 1-2

Issue 1-3

Issue 1-4

Issue 1-8

Issue 1-6

Issue 1-7

Issue 1-9

Issue 1-12

Quick check whether can agree on Issue 1-10; Issue 1-13; Issue 1-16 due to majority views

Topic #1: RRM requirements for UE-initiated/event-driven beam management

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2500054 | Qualcomm Technologies Ireland | Observation 1: In RAN1 #119 it was agreed that at least the case is supported in Rel-19 that the evaluation periodicity for event-2 is identical for the current and the new beam that the evaluation periodicity equals the periodicity of the RS of the current and the new beam. Observation 2: The case of different periodicities of the current beam RS and the new beam RS(s) is still under discussion in RAN1 but has not been agreed yet. Proposal 1: RAN4 should focus on the case of same periodicities of the current beam RS and the new beam RS(s) to define measurement reporting delay requirements. Observation 3: Tgap is the time between the occurrence of the event over the air and the availability of the first periodic CSI-RS resource for measuring. Observation 4: In the definition of the test case itself when RRM performance requirements are defined, the test can be defined in such a way that the delay uncertainty Tgap is not counted. Observation 5: For event-triggered periodic reporting of L3 measurements delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH and the case of no UL resources are excluded from the reporting delay. Proposal 2: Delay uncertainty till the first PUCCH can be sent after the event has been identified by the UE and the case of no UL resources should not be included in the measurement reporting delay for UE-initiated/event-triggered beam reporting. Observation 6: If the measurement periods TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS and the existing beam sweeping factor N = 8 are reused, no additional accuracy requirements need to be defined. Observation 7: It is desirable to reuse the existing L1-RSRP measurement accuracy requirements. Proposal 3: RAN4 should reuse the measurement periods as already defined for L1-RSRP measurements for FR2 as much as possible, i.e., TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS. Also, the beam sweeping factor in the existing requirements for L1-RSRP measurements should be reused. Observation 8: For event-2 and event-7 the existing measurement periods can be reused. It should be noted that one sample for the non-event-detecting beam would be used in the measurement process before the event happened OTA. Proposal 4: For the case M = P =1 and non-DRX, the measurement period for event-1 can be defined based on N*TSSB. For event-2 the same measurement period as for event-1 should be defined. Proposal 5: No additional accuracy requirements are needed for L1-RSRP measurements in UE-initiated / event-triggered beam management. Observation 9: In Rel-17, measurement requirements for inter-cell beam management have only been introduced for SSB based L1-RSRP but not for CSI-based L1-RSRP. Proposal 6: RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case. Proposal 7: The existing conditions for known and unknown TCI state can be reused for UE-initiated/event-triggered beam management. Proposal 8: The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management. Observation 10: in case that no filtering is applied in the UE, the measurement period for the time window approach would simply be extended by an additional scaling. No major UE implementation would be tested. Proposal 9: RAN4 should not define additional requirements for the window-based observation whether the event has occurred M times. |
| R4-2500268 | Apple | Observation 1: Time window based even triggering is not basic feature and subject to UE capability. Observation 2: It is not feasible to define core requirement for reporting delay with time window based triggering Proposal 1: Measurement reporting delay requirement for UE initiated beam report is defined without time window based evaluation. Reporting Delay Requirements Proposal 2: RAN4 only define requirements for same periodicity of new beam and current beam, until further agreement on support of different periodicity in RAN1 Proposal 3: Components of measurement reporting delay for UEIBM are measurement period, time to PUCCH. Proposal 4: Use L1-RSRP measurement period for SSB and CSI-RS as starting point for measurement period for current beam and new beam. Proposal 5: Measurement period for UEIBM is defined as max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam } In FR1/ FR2 with resources not adjacent or overlapping: max{ Mc*N*Pc*TcurrBeam , MN*N*PN*TnewBeam } In FR2 with resources adjacent or overlapping: max{ Mc*N*2*Pc*TcurrBeam , MN*N*2*PN*TnewBeam } Proposal 6: Time to PUCCH can be defined as time to first PUCCH resource or periodicity of PUCCH resource for UEIBM - Tfirst-PUCCH or TPUCCH. Proposal 7: Measurement reporting delay for UEIBM is defined as max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam } + Tfirst-PUCCH or TPUCCH. CSI-RS Based measurements Observation 3: CSI-RS based new beam measurement cannot be distinguished between inter-cell and intra-cell. Proposal 8: For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation. TCI State/ Known condition Observation 4: No impact of UE triggered beam report to known/unknown conditions for TCI state. Observation 5: The known/unknown condition would be determined by the same criteria as legacy. Proposal 9: Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM Proposal 10: The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management. Enhancements to RX beam sweeping Observation 6: Faster RX, TX beam refinement is not in scope of this WI. Proposal 11: Reuse current beam sweeping factor for UE/event triggered beam reporting. Capabilities Proposal 12: Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added. Proposal 13: Discuss later in the WI on compatibility of capabilities with LTM. Accuracy Requirements Proposal 14: Legacy accuracy requirements for L1-RSRP shall be applicable for UEIBM. No new accuracy requirements are necessary. |
| R4-2500332 | China Telecom | Proposal 1: RAN4 to specify requirements to cover both cases of no time window based and time window based. Proposal 2: Event triggered measurement reporting delay is TL1-RSRP_measure_period + Tfirst_PUCCH, TL1-RSRP_measure_period is based on SSB or CSI-RS, and Tfirst_PUCCH is the uncertainty time to UL resource for the 1st PUCCH. Proposal 3: Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM. Proposal 4: The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management. Proposal 5: Reuse current beam sweeping factor for UE/event triggered beam reporting. Proposal 6: Align the agreement on whether to define the capability for support of L1 event triggering and reporting in TS 38.133 for R19 MIMO WI and R19 mobility WI. Proposal 7: The existing accuracy requirements for L1 measurements can be applied. Not to define new accuracy requirements. |
| R4-2500622 | Xiaomi | Observation 1: For legacy L3 event triggered reporting, it is specified in 38.331 that the measurement result which triggers L3 reporting will be included in the L3 report. Proposal 1: For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting. Proposal 2: If multiple beams will be measured, total measurement time will be maximum measurement period between current beam and new beam. Proposal 3: For event triggered L1 reporting delay is defined as the time between an event that will trigger a L1 measurement report and the point when UE transmits a first PUCCH triggered by the event over the air interface. The event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on and L3 filtering. Where Tmeasure is the measurement delay. Proposal 4: Reuse the current assumption of beam sweeping for requirements for UE/ event triggered beam reporting. Proposal 5: No new accuracy requirements for L1 measurements are required. Proposal 6: Reduction of beam application delay may have impact on TCI state activation delay, RAN4 shall wait for more RAN1 progress. |
| R4-2500775 | LG Electronics Inc. | Proposal 1: RAN4 to specify reporting delay requirement as follows: - Reporting delay = Measurement time + UL uncertainty - Measurement time is longer periodicity of the current beam and new beam Proposal 2: Whether to update L1 report which is measured in L1 report needs to wait RAN1 progress Proposal 3: No need to define new accuracy requirements Proposal 4: Reuse legacy known/unknown conditions for TCI-switcing based on UE-initiated/event-driven beam reporting Proposal 5: For unified TCI switching delay requirements, RAN4 needs to discuss whether active TCI state switching delay can be reduced or not |
| R4-2500909 | ZTE Corporation, Sanechips | Proposal 1: RAN4 needs to determine the evaluation period for Event-1, Event-2 and Event-7 respectively. In details, - For Event-1, only current beam should be monitored; - For Event-2, both current beam and new beam should be monitored; - For Event-7, besides new beam, all beams associated with the active TCI state list should be monitored. Proposal 2: Based on RAN1 progress on Event-2 until now, both same periodicity and different periodicity cases should be considered to determine the evaluation period. Proposal 3: For Event-2, both the basic Case 1 and the optional Case 2 should be considered when defining the evaluation period. The maximum of current beam periodicity and new beam periodicity should be considered for both two solutions. - For the basic Case 1, which is one-shot measurement based reporting, the sample number is 1. - For the optional Case 2, the sample number M and timer X should be considered. Proposal 4: For Event-1, both the basic Case 1 and the optional Case 2 should be considered when defining the evaluation period, only the periodicity of current beam is involved in. - For the basic Case 1, which is one-shot measurement based reporting, the sample number is 1. - For the optional Case 2, the sample number M and timer X should be considered. Proposal 5: For the basic Case 1 of Event-7, the maximum between the new beam periodicity and the periodicity of all beams associated to the active TCI state list should be considered. Proposal 6: For the optional Case 2 of Event-7, some further clarification in RAN1 is needed in terms of whether the Q-th best TCI state should be consistent or not during the event detection. RAN4 could wait for more RAN1 input or send LS to trigger the clarification. Proposal 7: For event triggered UEIBM, the evaluation period is equivalent to the measurement period. Proposal 8: Based on RAN1 progress, for the TCI state switching triggered by UEIBR(may or may not receive TCI state switching command), UE is assumed to store the QCL properties of the SSB associated with new beam, the legacy TCI state switching components of TL1-RSRP and Tfirst-SSB+TSSB-proc are ignored. Proposal 9: For CSI-RS based inter-cell L1-RSRP measurement, since no specific discussion before, RAN4 needs to identify whether additional clarifications on the following points are necessary: - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC; - The prerequisite on L3 measurement on NSC. Proposal 10: RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements. Proposal 11: At least the legacy L1-RSRP accuracy requirement should be guaranteed for both basic Case 1 and optional Case 2. |
| R4-2501081 | CMCC | Proposal 1: for whether FR1 is covered for UE-initiated/event-driven beam management, it is proposed to follow previous agreements and send LS to RAN1 in RAN4#114 meeting. The draft LS is provided in the Annex. Proposal 2: for definition of delay requirements for Event 2, it is proposed to follow RAN1 agreements that the periodicity of the current beam RS is the same as that of the new beam RS(s). Proposal 3: for reporting delay for SSB based UE-initiated/event-driven beam reporting, it is proposed to take measurement period defind for Rel-18 LTM as baseline - in detail, the reporting delay for SSB bsed UE-initiated/event-driven beam reporting is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index , which are specified for L1-RSRP measurement for Rel-18 LTM . Proposal 4: for reporting delay for CSI-RS based UE-initiated/event-driven beam reporting, it is proposed to define L1 measurement reporting delay requirements, and FFS whether legacy CSI-RS based L1 measurement delay requirements can be reused. Proposal 5: it is proposed to define capabilities for support of UE-initiated/event-driven beam reporting. Proposal 6: it is not prefered to mix Rel-19 mobility and Rel-19 MIMO. The capability for supported number of events triggered L1 measurement reporting is defined seperately for Rel-19 mobility and Rel-19 MIMO. Proposal 7: for UE-initiated/event-driven beam reporting, except reporting delay requirements, it is proposed to define reporting accuracy requirements. - The reporting accuracy requirement is that the reported L1 measurement contained in event triggered beam reports shall meet the accuracy requirements - For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline. - For CSI-RS based beam reprt, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements taking neighbour cell measurement into account. |
| R4-2501222 | Huawei, HiSilicon | Observation 1: Based on RAN1 agreements, Event 1/2/7 are supported, where - Event 2 and Event 7 are event triggered compared by comparation of current beam and new beam - Event 1 are triggered by comparation between current beam and a certain threshold. Observation 2: The condition of event 2/7 triggering shall consider all following cases: 1. Bad quality of current beam 2. Good quality of new beam 3. Change of both current beam and current beam, which resulting in offset better of new beam Proposal 1: The measurement requirements are defined assume that UE use the latest results from both new beam and current beam after the condition is met over the air. Note: Simplified solution can be considered in test case that the condition of only one of beams (i.e. new beam and current beam) is changed. Observation 3: For time window-based Event triggering, there is no difference compared with non-time-window based triggering, in terms of measurement behaviour/requirements and evaluation. Observation 4: Time window-based Event triggering is important for this feature, which may help mitigate the wrong event triggering cause by instant channel fading. Proposal 2: RAN4 define requirements for both Time window-based Event triggering and no-time-window-based event triggering. Proposal 3: From RAN4 perspective, the periodicity of current beam and new beam(s) can be the same or different which is up to network configurations. Observation 5: Defining evaluation period based on periodicity of only one of the RS may cause instance wrongly counted (out-dated result of one beam will be used for evaluation for multiple times). Proposal 4: The evaluation period shall be defined as the largest periodicity of the current beam and new beam RS. Observation 6: From UE perspective, for CSI-RS based BM, there is no differentiation w.r.t. intra-cell or inter-cell. Proposal 5: CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation. Proposal 7: The existing known conditions for TCI state switching can cover the case that L1-RSRP reporting triggered by UEIBM. Observation 7: Do not combine UE capabilities across different WID, and postpone the discussion in RAN4 with more RAN1/2 progress. |
| R4-2501405 | Nokia | Proposal 1: The UE shall be able to send the first PUCCH latest at the PUCCH occasion that is available after the maximum L1 event evaluation delay (i.e. the maximum time it can take the UE to realize the condition is fulfilled). Proposal 2: For events involving serving and candidate beam, maximum L1 event evaluation delay is the maximum of L1 measurement period for serving beam and candidate beam that the event considers. Proposal 3: Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM Observation 1: RAN1 is discussing various alternatives to reduce the beam application latency, some of which may have an impact on the TCI state switch delay. Proposal 4: RAN4 to wait for RAN1 progress on discussion of beam application latency reduction after a UEI beam report is sent before defining TCI state switching delay requirements. Proposal 5: UE behaviour in the interim between sending the 1st PUCCH and waiting for UL resources to send the UEI report is up to RAN1. Observation 2: No baseline RAN4 requirements defined for CSI-RS based L1-RSRP measurements for inter-cell beam management. Proposal 6: RAN4 not to define requirements for CSI-RS based L1-RSRP measurements for inter-cell beam management within the Rel-19 WI. Proposal 7: Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting. Observation 3: RAN1 is discussing UE capability about the number of events supported for UEIBM. Proposal 8: Do not combine capabilities across different L1 measurements (MIMO and LTM). Proposal 9: The existing accuracy requirements for L1 measurements can be applied. Not to define new accuracy requirements. |
| R4-2501462 | Samsung | Observation 1: RAN1 only support Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). Proposal 1: According to latest RAN1 support, the evaluation periodicity can reuse the legacy L1-RSRP measurement period. Proposal 2: RAN4 to specify requirements for both basic feature and within a time window configured. Proposal 3: For the case of no time window based, the L1 evaluation time is defined as: FR2+SSB based+event-2 Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P*N)*TSSB). DRX cycle ≤ 320ms; max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(1.5*M*P*N)*TDRX. Note: TDRX is the DRX cycle length. TReport is configured periodicity for reporting.. TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index in indicated TCI state and SSB-index configured in [TBD] FR2+SSB based+event-1 Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P*N)*TSSB). DRX cycle ≤ 320ms; max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(1.5*M*P*N)*TDRX. Note: TDRX is the DRX cycle length. TReport is configured periodicity for reporting.. TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index in indicated TCI state. Observation 2: For event-7, no conclusion of whether the RS periodicity of RSs in activated TCI states are the same or not. Observation 3: The measurement should cover all activated TCI state and new beam (s). Proposal 4: For the case of no time window based, the L1 evaluation time is defined as: FR2+SSB based+event-7 Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P*N)*max(TSSB_active_0, …TSSB_active_i, TSSB_active_X, TSSB_new)). DRX cycle ≤ 320ms; max(TReport, ceil(1.5*M*P*N)*max(TDRX, TSSB_active_0, …TSSB_active_i, TSSB_active_X, TSSB_new)). DRX cycle > 320ms; ceil(1.5*M*P*N)*TDRX. Note: TDRX is the DRX cycle length. TReport is configured periodicity for reporting.. TSSB_active_i = ssb-periodicityServingCell is the periodicity of the SSB-Index in i-th active TCI state. Proposal 5: For within a time window configured case, the L1 evaluation time is defined as multiple times of L1 evaluation time in basic to fulfil M counter is met for the event. Proposal 6: After we achieved the agreement of L1 evaluation time, the reporting delay can be specified as: T_L1 evaluation time + T_time_first_pucch. Proposal 7: No need to add TSSB_time_index Proposal 8: Similar approach of TL1-RSRP_Measurement_Period_CSI-RS in section 9.5.4.2 can be reused as the principle in SSB based UEIBM. Proposal 9: For known conditions for TCI states for TCI states switching based on UEIBM, it can use as legacy. Proposal 10: No difference by using UE triggered L1 report or NW based L1 report. Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching if known case. No unknown case is applied if the TCI states switching is based on UEIBM. Proposal 11: The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. Proposal 12: No enhancement for fast beam sweeping. Proposal 13: The existing accuracy requirements for L1 measurements can be applied. No new accuracy requirements. Proposal 14: Define separate capabilities for MIMO and mobility. |
| R4-2501598 | vivo | Proposal 1 As the first step, RAN4 to discuss the event-triggered L1 reporting requirements based on the following assumptions: - the condition which triggers the event is remained fixed during the time UE performs L1 measurements. - the condition which triggers the event is remained fixed during the time after UE sends the first uplink transmission and before UE is scheduled with the second uplink transmission for the L1 reporting.. RAN4 may further discuss other cases if time allows. Proposal 2 As the first step, RAN4 to discuss the event-triggered L1 reporting requirements based on the following assumptions: - during the time UE performs L1 measurements, no change on the indicated TCI state of current serving cell - during the time UE performs L1 measurements, no change on the to-be-measured RS of the new beam. RAN4 may further discuss other cases if time allows. Proposal 3 As the first step, RAN4 to discuss the event-triggered L1 reporting requirements based on the following assumptions (Alt-1 in RAN1 LS R4-2500009): - The periodicity of the current beam RS should be the same as that of the new beam RS(s) RAN4 may further discuss RRM requirements for Alt-2 and the potential restrictions to the requirement applicability if time allows, based on RAN1 further agreements. |
| R4-2502062 | Ericsson | Proposal 1: SSB based L1-RSRP measurement period for the SSB configured for event triggered measurement is given by following table. Configuration; TL1-RSRP_event triggered_measurement_Period_SSB (ms). non-DRX; ceil(M*P*N)*TSSB). DRX cycle ≤ 320ms; ceil(1.5*M*P*N)*max(TDRX,TSSB). DRX cycle > 320ms; ceil(1.5*M*P*N)*TDRX. Note 1: TSSB = is the periodicity of the SSB-Index configured for L1-RSRP measurement that may trigger the measurement report. TDRX is the DRX cycle length.. Proposal 2: CSI-RS based L1-RSRP measurement period for the CSI-RS configured for event triggered measurement period is given by following table. Configuration; TL1-RSRP_ event triggered _measurement_Period_CSI-RS (ms). non-DRX; ceil(M*P*N)*TCSI-RS. DRX cycle ≤ 320ms; ceil(1.5*M*P*N)*max(TDRX,TCSI-RS). DRX cycle > 320ms; ceil(M*P*N)*TDRX. Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement that may trigger the measurement report. TDRX is the DRX cycle length. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3.. Proposal 3: RAN4 not to consider counter or timer in the event triggered reporting as L3 measurements did not consider that. That means M1=1 in RAN4 requirements. Proposal 4: Event evaluation period for a SSB or CSI-RS can be decided in RAN1. Proposal 5: RAN4 to further discuss the components and criteria for defining TTime to first UL channel. Proposal 6: Event triggered measurement reporting delay for SSB is TL1-RSRP_event triggered_measurement_Period_SSB (ms) + TTime to first UL channel and event triggered measurement delay for CSI-RS is TL1-RSRP_event triggered_measurement_Period_CSI-RS (ms) + TTime to first UL channel. Proposal 7: Event triggered measurement report requirements are applicable for intra and inter-cell scenario for SSB and CSI-RS. Proposal 8: RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2. Proposal 9: Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching. Proposal 10: RAN4 not to specify new Rx beam sweeping factor or new accuracy requirements as part of the event triggered measurement requirements. Proposal 11: UE Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added. Proposal 12: RAN4 to wait for further progress in RAN1/2 for the UE capability discussions and compatibility between MIMO and mobility. |
| R4-2502182 | MediaTek inc | Proposal 1: Reuse legacy measurement period for event 2 on UE-initiated/event-driven beam management. - SSB based L1-RSRP measurement for both intra-cell and inter-cell. - CSI-RS based L1-RSRP measurement for intra-cell. - Descope CSI-RS based L1-RSRP measurement for Inter-cell case. Proposal 2: RAN4 to consider a unified measurement delay requirement of event 2 for both case 1 (no time window based) and case 2 (with time window based): - TL1-RSRP_measure = ceil(M*P*N) * TRS - M: number of samples for L1-RSRP measurement. - M = 1 or 3 for case 1. - M = 1 for case 2. - P: sharing factor, same as legacy. - N: beam sweeping factor, same as legacy. - TRS is the periodicity of reference signal (RS). The RS could be either SSB or CSI-RS for current beam and new beam. Proposal 3: RAN4 to consider the reporting delay requirement of event 2 for both case 1 and case 2: - Tevent_reporting_delay = Tevaluate_L1_event + Tfirst UL channel - Tevaluate_L1_event: the evaluation period of event 2 for current and new beam. - For case 1: Tevaluate_L1_event = TL1-RSRP_measure + Tuncertainty - For case 2: Tevaluate_L1_event = TL1-RSRP_measure * C + Tuncertainty given that Tevaluate_L1_event ≤ Ttime_window - TL1-RSRP_measure: L1-RSRP measurement delay for either SSB or CSI-RS. - Tuncertainty: the uncertainty time to fulfill the event criteria. The value could be zero or times of TL1-RSRP_measure. - C: event instance counter is configured by NW. - Ttime_window: time window configured by NW - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH. Proposal 4: Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM. Proposal 5: The existing requirements for TCI state switch delay should be reused for UE-initiated/event-driven beam management. Proposal 6: RAN4 do not consider beam sweeping enhancement on UE event triggered reporting since it is out of scope in this WI. Proposal 7: RAN4 to reuse the legacy L1-RSRP accuracy requirement for UE-initiated/event-driven beam management. Proposal 8: Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency. Proposal 9: Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency. |

Open issues summary

Sub-topic 1-1

Issue 1-1: assumption or applicability of event triggered L1 measurement:

- Proposals

  - Option 1: (vivo)

    - Follow the assumption:

      - the condition which triggers the event is remained fixed during the time UE performs L1 measurements.

      - the condition which triggers the event is remained fixed during the time after UE sends the first uplink transmission and before UE is scheduled with the second uplink transmission for the L1 reporting

      - during the time UE performs L1 measurements, no change on the indicated TCI state of current serving cell

      - during the time UE performs L1 measurements, no change on the to-be-measured RS of the new beam.

- Recommended WF

  - To be discuss

Issue 1-2: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1)?

- Proposals

  - Option 1: (Qualcomm)

    - Focus on the case of same periodicities of the current beam RS and the new beam RS(s) to define measurement reporting delay requirements.

    - Same as legacy TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS by removing TReport, and reuse the same N as legacy; FFS on M and P

  - Option 2: (Apple)

    - Only define requirements for same periodicity of new beam and current beam until now

    - max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam }

    - In FR1 and FR2 when current beam and new beam are not adjacent or overlapping:

      - TL1-RSRP_currBeam = Mc*N*Pc*TcurrBeam

      - TL1-RSRP_newBeam = MN*N*PN*TnewBeam

    - In FR2 when current beam and new beam are adjacent or overlapping:

      - TL1-RSRP_currBeam = Mc*N*2*Pc*TcurrBeam

      - TL1-RSRP_newBeam = MN*N*2*PN*TnewBeam

TcurrBeam and TnewBeam are the same

  - Option 3: (Xiaomi, Nokia)

    - Maximum of {TL1-RSRP} between current beam and new beam if multiple beams will be measured.

  - Option 3a: (LGE, ZTE, Huawei, MediaTek)

    - Maximum periodicity of current beam and new beam

  - Option 3b: (ZTE for more details)

    - Remove Treport

    - Event-1&2:

      - one-shot measurement with 1 sample

    - Event-7:

      - maximum between the new beam periodicity and the periodicity of all beams associated to the active TCI state list should be considered

  - Option 4: (CMCC)

    - SSB based: Reuse the same TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index in Rel-18 LTM

    - CSI-RS based: FFS on whether legacy CSI-RS based L1 measurement delay can be reused.

  - Option 5: (Huawei)

    - In core requirements, UE shall use the latest results from both new beam and current beam after the condition is met over the air

  - Option 6 (Samsung)

SSB based:

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Event-2: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index in indicated TCI state and SSB-index configured in [TBD]

Event-7:

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*max(TSSB_active_0, …TSSB_active_i, TSSB_active_X, TSSB_new)) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX, TSSB_active_0, …TSSB_active_i, TSSB_active_X, TSSB_new)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

TSSB_active_i = ssb-periodicityServingCell is the periodicity of the SSB-Index in i-th active TCI state.

No need to add TSSB_time_index

    - Similar approach of TL1-RSRP_Measurement_Period_CSI-RS in section 9.5.4.2 can be reused as the principle in SSB based UEIBM.

  - Option 7: (Ericsson)

| Configuration | TL1-RSRP_event triggered_measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | ceil(1.5*M*P*N)*max(TDRX,TSSB) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note 1: TSSB = is the periodicity of the SSB-Index configured for L1-RSRP measurement that may trigger the measurement report. TDRX is the DRX cycle length. |  |

| Configuration | TL1-RSRP_ event triggered _measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | ceil(M*P*N)*TCSI-RS |
| DRX cycle ≤ 320ms | ceil(1.5*M*P*N)*max(TDRX,TCSI-RS) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement that may trigger the measurement report. TDRX is the DRX cycle length. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

Moderator’ comments: to avoid all discussion in the same one Issue, it is decoupled with Issue 1-3. No time window-based measurement will be defined in RAN4 (no observation any company against this one)

- Recommended discussion

  - First discuss the TL1-RSRP_event triggered_measurement_Period_SSB or TL1-RSRP_event triggered_measurement_Period_CSI-RS

    - Option 1 (Qualcomm, Apple, Xiaomi, Nokia, ZTE, Huawei, MediaTek, Ericsson, Samsung):

      - update based on 9.5.4.1 (SSB based) and 9.5.4.2 (CSI-RS based)

      - Option 1a (Samsung): In addition of the basic bullet, TSSB_time_index is not needed.

    - Option 2 (CMCC):

      - Reuse the same TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index in Rel-18 LTM

Moderator’ comments: need clarification from option2. What is the exact reusing in option 2 from LTM. In LTM WI, there can be several cases: no intra-frequency L1-RSRP measurement on LTM neighbor cell(s); with intra-frequency L1-RSRP measurement on LTM neighbor cell(s); inter-frequency neighbor cells, in different cases, it can be scaled by different factors such as number of layers, number of Neighbor cells

  - For the discussion on update based on 9.5.4.1-2 (SSB based) and 9.5.4.2-2 (CSI-RS based)

    - Agree on: TReport = 0 or be removed from the measurement period

    - M:

      - Option1: FFS on M (Qualcomm)

      - Option2: reuse legacy definition of 1 and 3 depends on timeRestrictionForChannelMeasurement

      - Option3: Mc and MN for current beam and new beam (Apple)

      - Option4: M = 1 (ZTE, MediaTek)

    - N:

      - Option1: Reuse the legacy definition

      - Option 2: enable same fast beam sweeping in another Issue 1-13.

    - P:

      - Option 1: FFS on P (Qualcomm)

      - Option 2: reuse legacy definition of SSB based in 9.5.4.1 and CSI-RS based in 9.5.4.2

      - Option 3: Pc and PN for current beam and new beam (Apple)

    - TSSB

      - Option 1: TSSB_current, TSSB_new are the same

      - Option 2: Max{TSSB_current, TSSB_new}

      - Option 3:

        - The definition of TSSB is the periodicity of the SSB-Index configured for L1-RSRP measurement that may trigger the measurement report.

      - Option 4:

        - Different description of different Events

          - The periodicity of the SSB-Index in indicated TCI state and SSB-index configured in [TBD]

          - Periodicity of the SSB-Index in i-th active TCI state

Issue 1-3: Same or different periodicities of current beam and new beam(s)?

- Proposals

  - Option 1: (Qualcomm, Apple, CMCC, Samsung, vivo)

    - The same periodicities of current beam and new beam(s) to define the requirements, until further RAN1 agreements on different periodicities.

    - According to RAN1 LS (R1-2410914)

  - Option 2: (Huawei)

    - From RAN4 perspective, the periodicity of current beam and new beam(s) can be the same or different which is up to network configurations. Send LS to RAN1

  - Option 3: (vivo)

    - RAN4 may further discuss RRM requirements for Alt-2 and the potential restrictions to the requirement applicability if time allows, based on RAN1 further agreements.

- Recommended WF

  - TBA

Issue 1-4: Components of Event triggered measurement reporting delay?

- Proposals

  - Option 1: (Qualcomm)

    - Tgap+Tmeas+TPUCCH

    - Tgap is the time between the occurrence of the event over the air and the availability of the first periodic CSI-RS resource for measuring. Can be 0 in the test case.

    - Tmeas is time of L1-RSRP measurement in Issue 1-1

  - Option 2: (Apple)

    - max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam }+TPUCCH

    - TL1-RSRP_currBeam or TL1-RSRP_currBeam is time of L1-RSRP measurement in Issue 1-1

    - TPUCCH can be time to first PUCCH after measurement delay or periodicity of PUCCH resouce

  - Option 3: (China Telecom, LGE, Samsung, Ericsson)

    - Tmeas+TPUCCH

    - Tmeas is the time of L1-RSRP measurement in Issue 1-1

    - TPUCCH is the uncertainty time to UL resource for the first PUCCH; (Ericsson): Further discuss the components and criteria for TTime to first UL channel

  - Option 4: (Xiaomi)

    - The event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on and L3 filtering. Where Tmeasure is in Issue 1-1.

  - Option 5: (CMCC)

    - The reporting delay for SSB based UE-initiated/event-driven beam reporting is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index , which are specified for L1-RSRP measurement for Rel-18 LTM .

  - Option 6: (MediaTek)

    - TL1-RSRP_measure + Tuncertainty + Tfirst UL channel

    - Tuncertainty: the uncertainty time to fulfill the event criteria.

- Recommended WF

  - First discuss on the description in two ways:

    - Series 1: The event triggered measurement reporting delay is TL1-RSRP_measure + Tfirst UL channel + other components (Option 1, 2, 3, 6 above. Different ones on whether other components are included or the detailed definitions are a little different in Option 1, 2, 3, 6)

    - Series 2: The event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on and L3 filtering. (Option 4, 5, Where Tmeasure are different in Option 4, 5)

Issue 1-5: Whether to specify the requirements for Event-1/Event-7?

[Background: In last RAN#119 meeting, the agreement is shown as below which confirm the working assumption]

On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed: On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. - Q is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2.

- Proposals

  - Option 1:

    - Define the reporting delay requirements for Event-2/Event-1/Event-7

  - Option 2:

    - Only define RAN4 requirements for some of Events.

- Recommended WF

  - TBA

Moderator’ comments: In previous RAN4 discussion, we focus on the discussion based on Evnet-2 because this is the only one agreed event in RAN1 at that time. In last RAN1 meeting, RAN1 confirmed to support Event-1 and Event-7 as well with lower priority compared to event 2. In this meeting, implicit proposals from companies to specify requirements to cover all. Not sure whether any proponent for option 2? Try for consensus.

Issue 1-6: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with configurable M?

- Proposals

  - Option 1: (Qualcomm, Apple, Ericsson)

    - Only define requirements for the number of events is met at once

  - Option 2: (China Telecom, ZTE, Huawei, Samsung, MediaTek)

    - Specify requirements to cover both cases of no time window based and with time window based with configurable M

  - Option 2a: (ZTE)

    - Event1&Event 2: Case 2: multiple samples M and timer X should be considered

    - Event1&Event 7: FFS

  - Option 2b (Samsung)

    - For within a time window configured case, the L1 evaluation time is defined as multiple times of L1 evaluation time in basic to fulfil M counter is met for the event.

  - Option 2c (MediaTek)

    - For multiple events is met by counter:

      - Tevaluate_L1_event = TL1-RSRP_measure * C + Tuncertainty given that Tevaluate_L1_event ≤ Ttime_window

      - TL1-RSRP_measure: L1-RSRP measurement delay for either SSB or CSI-RS

      - C: event instance counter is configured by NW.

- Recommended WF

  - TBA

Issue 1-7: Whether/how to specify the UE behaviour if new L1-RSRP during the UL resource waiting time?

- Proposals

  - Option 1: (Xiaomi)

    - For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting.

  - Option 2: (LGE, Nokia)

    - Need to wait RAN1 progress

  - Option 3: (vivo)

    - Follow the assumption: the condition which triggers the event is remained fixed during the time after UE sends the first uplink transmission and before UE is scheduled with the second uplink transmission for the L1 reporting. FFS on other cases if time allows

- Recommended WF

  - TBA

Issue 1-8: Whether to confirm to specify FR1 requirements and send LS to RAN1.

- Proposals

  - Option 1: (CMCC).

    - for whether FR1 is covered for UE-initiated/event-driven beam management, it is proposed to follow previous agreements to send LS to RAN1.

      - Background: In last meeting, it was agreed to send LS to RAN1 to ask whether FR1 is covered. However, the LS was missing in last meeting

- Recommended WF

  - TBA

Issue 1-9: CSI-RS based L1-RSRP for inter-cell BM

- Proposals

  - Option 1: (Qualcomm, Nokia, MediaTek(descoped))

    - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case.

  - Option 3: (Apple, Huawei, Ericsson)

    - For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

    - Option 3a (Apple): The measurement delay is the same as that in serving cell in legacy L1-RSRP

    - Option 3b (Ericsson): reuse the principle of sharing agreed for SSB based measurement.

  - Option 4: (ZTE)

    - RAN4 needs to identify whether additional clarifications on the following points are necessary:

      - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC;

      - The prerequisite on L3 measurement on NSC.

      - RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

  - Option 5: (Ericsson)

    - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2

    - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements

- Recommended WF

  - TBA

Issue 1-10: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM

- Proposals

  - Option 1: (Qualcomm, Apple, China Telecom, LGE, CMCC, Huawei, Nokia, Samsung, MediaTek)

    - Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM

- Recommended WF

  - Agree option 1

Issue 1-11: Whether to specify new unified TCI switching delay requirements?

- Proposals

  - Option 1: (Qualcomm, Apple, China Telecom, Samsung (For known conditions for TCI states), Ericsson, MediaTek)

    - The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management.

  - Option 2: (Xiaomi, Nokia)

    - Reduction of beam application delay may have impact on TCI state activation delay, RAN4 shall wait for more RAN1 progress.

  - Option 3: (LGE)

    - For unified TCI switching delay requirements, RAN4 needs to discuss whether active TCI state switching delay can be reduced or not

  - Option 4: (ZTE)

    - For MAC-CE based DL TCI switching delay: no need to consider TL1-RSRP and Tfirst-SSB+TSSB-proc

  - Option 5: (Samsung)

    - Not consider unknown case.

- Recommended WF

  - TBA

Issue 1-12: Other conditions for L1 measurement and TCI state switching?

- Proposals

  - Option 1: (Samsung)

    - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI.

- Recommended WF

  - TBA

Issue 1-13: Whether to enhance beam sweeping in event triggered L1 beam reporting?

- Proposals

  - Option 1: (Qualcomm, Apple, China Telecom, Xiaomi, Samsung, MediaTek)

    - Reuse current beam sweeping factor for UE/event triggered beam reporting

  - Option 2: (Nokia)

    - Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting.

- Recommended WF

  - TBA

Issue 1-14: Capabilities for Support of Event Triggering and Reporting Criteria?

- Proposals

  - Option 1: (Apple, Ericsson)

    - Defined by RAN1. To be discussed later if any RAN4 specific UE capabilities need to be added.

  - Option 2: (China Telecom)

    - Align the agreement on whether to define the capability for support of L1 event triggering and reporting in TS 38.133 for R19 MIMO WI and R19 mobility WI.

  - Option 3: (CMCC, MediaTek)

    - Define capabilities similar as legacy L3 (like legacy L3 in section 9.1.4).

Issue 1-15: Compatibility of capabilities with LTM

- Proposals

  - Option 1: (Apple, Ericsson)

    - Discuss it later.

  - Option 2: (CMCC, Huawei, Nokia, Samsung)

    - Do not mix/combine Rel-19 mobility and Rel-19 MIMO. Define the capability separately.

  - Option 4: (MediaTek)

    - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

- Recommended WF

  - TBA

Issue 1-16: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management?

- Proposals

  - Option 1: (Qualcomm, Apple, China Telecom, Xiaomi, LGE, ZTE, Nokia, Samsung, Ericsson, MediaTek)

    - The existing accuracy requirements for L1-RSRP measurements can be applied. Not to define new accuracy requirements.

  - Option 2: (CMCC)

    - For SSB based beam report, reuse Rel-18 LTM accuracy requirements as baseline.

    - For CSI-RS based beam report, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements taking neighbour cell measurement into account.

- Recommended WF

  - Agree option 1.

Moderator’ view: There is no L1-RSRP for UEIBM for neighbour cell measurement. If look into the detailed values defined in option 2, for Rel-18 LTM intra/inter-frequency L1-RSRP measurement, the side condition is the same as legacy -3dB and absolute and relative accuracy requirements are reusing legacy absolute and relative accuracy requirements. Please check whether to agree: reuse the same side condition and absolute and relative accuracy requirements for L1-RSRP in UEIBM L1-RSRP, for both SSB and CSI-RS based.