3GPP TSG-RAN WG4 Meeting #114	R4-2500548

# Athens, Greece, 17 -21 Feburary, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.908 | CR |  | rev | - | Current version: | 1.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Plane Wave Synthesizer (PWS) method inclusion for Expected EIRP test procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 17) Rel-17 (Release 18) Rel-18 (Release 19) Rel-19 (Release 20) |  |
| Reason for change: |  | To include PWS as an accepted OTA measurement system for Expected EIRP procedures in 38.908 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Mentioned PWS under Test Procedures, MU table 6.6-3 added for PWS considering n104 band, C1-1 field MU correction for IAC and CATR, Table numbers which were previously repeated across IAC and CATR are now corrected; Listed Combined Std Uncertainty as TBD for all methods. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Expected EIRP will not accoomodate PWS; MU assumptions remain challenging for T&M vendors |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2, 6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Start of Change>>

## 6.5	Method of test

### 6.5.1	General

The EEIRP requirement contains 2 averaging processes, an average over the measurement points on the measurement grid (see sub-clause 6.2) and an average of beam steering directions. Mathematically these averages can be carried out in any order, as such the method of test may also carry out the averages in any order.

As it is likely that moving the positioner so a new measurement grid point can be measured will be a slower process than changing the beam steering direction, as such the method is written assuming beam steering averaging is done first.

### 6.5.1	Initial conditions

The initial conditions section contains the test set up initial conditions including:

- the test environment,

- the channels to be tested

- the beams to be tested

- etc

Test environment

EEIRP is to be measured under normal conditions

Currently only a few parameters are measured under extreme conditions which is extremely challenging as the OTA chamber cannot be environmentally controlled. Controlling the temperature of the DUT locally is done for limited tests but this could not be applied to EEIRP measurements.

RF channels

EEIRP measurement is a very long measurement, the number of channels tested will directly multiple the test time.

The frequency variation across the EEIRP band is approximately 10%, over this range, the gain of the main beam will change. However EEIRP is dominated by the sidelobe behaviour of the antenna pattern rather than the main beam, looking at the EEIRP performance of an antenna over the frequency range there is no clear worst case, the performance is no worse at Bottom channel or Top channel than it is at the middle channel.

As no channel is worse than any other it could be argued any channel could be used, as such the middle channel is the obvious choice.

Test signal and test model

EEIRP requirement is based on PSD, testing should be carried out with the highest PSD the BS is capable of.

The performance is dependent on the radiation beam shape, non-linearities or distortions potentially generated by multi-carrier signals will not significantly alter the beam shape of the wanted signal.

As such as single carrier test signal is used with test model NR-FR1-TM1.1.

As the requirement is PSD if the BS output power is constant then this will be achieved at the smallest channel band width. Potentially in order to meet EEIRP small channel bandwidth may have a limited output power capability. As specific channel BW is hence not specified only that we test with the highest PSD and corresponding channel BW.

### 6.5.2	Test procedure

The test procedure is based on a CATR/IAC/PWS where the BS is placed on a positioner and rotated so the required measurement direction is in line with the measurement antenna. The procedure has similarities to existing EIRP and TRP measurement procedures.

The test procedure is as follows:

1. Calibrate the test range, according to calibration described in TR 37.941, subclause 8.3

2. Place the test object at the positioner, such that blocking effect in the vertical domain is minimized.

3. Align the manufacturer declared coordinate system orientation (D.2) of the test object with the coordinate system used by the test system.

4. The measurement device characteristics shall be:

-	Detection mode: True RMS.

-	[The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table 4.1.2.2-1 for FR1.]

5. Configure the test object according to applicable test configuration in TS 38.141-2, subclause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1, subclause 4.9.2.

6. Set the positioner at location for measurement point (θn,φm) for m=1..M and n=1.. N.

7. Enable test beam  k for k=1..K.

8. Measure EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denotes two orthogonal polarizations.

9. Repeat step 6 and 7 for all K test beams.

10. Repeat step 5, 6, 7 and 8 for all M and N positioner locations.

11. Calculate EEIRPi profile from measured EIRPkθn,φm patterns for i=1..7 as described in TR 38.141-2, Annex Y.

NOTE: step 9 and step 10 could be exchangeable.

## 6.6	Measurement uncertainty

The EEIRP measurement is very similar to the in-band TRP measurements, as such the in-band emission MU budgets from TR 37.941 can be used as a starting point for the EEIRP budgets.

Additional sources of uncertainty are:

Dynamic range uncertainty

[One major difference with EEIRP however is that it is necessary to measure and add power on the same frequency as the wanted signal when the wanted signal is present and steered in another direction. Existing measurements either are measuring on adjacent channels (where frequency isolation can be used) or where the wanted signal is also being measured and is dominant. As such we are relying on the chamber spatial isolation to remove the effect of the wanted signal. Reflections and refractions of the main beam will limit the minimum power level that can be accurately measured.]

EEIRP sampling Error

The difference between the test beam directions set and the

Differences exist between the test beam directions set and infinite set of random beam directions and also due to errors caused by the sampling grid.

The can be referred to as EEIRP sampling error a definition is provided below:

EEIRP sample error – Error due to the sampling of the 2 averaging processes used to define EEIRP. The sampling error covers errors due to difference in power distribution due to the test beam direction set and an infinite set of random beam directions as well as errors due to the sampling grid.

[So that we can provide an example in the tables below we have assumed a worst-case error of 1.2dB as placeholder, if we assume a gaussian distribution then this is added in as a standard deviation of 0.6dB. This is done only to demonstrate an approximate value at this stage, the exact nature of the error and the optimum way to add it in needs to be further discussed]

These 2 errors have been added to the MU budget as follows:

| Table 6.6-1: IAC MU value derivation for EEIRP measurement, FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |  |  |  |
|  |  |  |  | 4.2<f≤7.125 GHz |  |  |  |  |  |  |  | 4.2<f≤7.125 GHz |  |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A1-1 | Positioning misalignment between the BS and the reference antenna |  |  | 0.03 | Rectangular | 1.73 | 1 |  |  |  |  | 0.02 |  |
| A1-2 | Pointing misalignment between the BS and the receiving antenna |  |  | 0.3 | Rectangular | 1.73 | 1 |  |  |  |  | 0.17 |  |
| A1-3 | Quality of quiet zone |  |  | 0.1 | Gaussian | 1.00 | 1 |  |  |  |  | 0.10 |  |
| A1-4a | Polarization mismatch between the BS and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-5a | Mutual coupling between the BS and the receiving antenna |  |  | 0 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| A1-6 | Phase curvature |  |  | 0.05 | Gaussian | 1.00 | 1 |  |  |  |  | 0.05 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) |  |  | 0.50 | Gaussian | 1.00 | 1 |  |  |  |  | 0.50 |  |
| A1-7 | Impedance mismatch in the receiving chain |  |  | 0.33 | U-shaped | 1.41 | 1 |  |  |  |  | 0.23 |  |
| A1-8 | Random uncertainty |  |  | 0.1 | Rectangular | 1.73 | 1 |  |  |  |  | 0.06 |  |
| A2-21 | Measurement system dynamic range uncertainty |  |  | [TBD] | Gaussian | 1 | 1 |  |  |  |  | [TBD] |  |
| *new | [EEIRP sampling error] |  |  | [TBD] | [Gaussian] | 1 | 1 |  |  |  |  | [TBD] |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A1-9 | Impedance mismatch between the receiving antenna and the network analyzer |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.04 |  |
| A1-10 | Positioning and pointing misalignment between the reference antenna and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-11 | Impedance mismatch between the reference antenna and the network analyzer |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.04 |  |
| A1-3 | Quality of quiet zone (normal test conditions) |  |  | 0.10 | Gaussian | 1.00 | 1 |  |  |  |  | 0.10 |  |
| A1-4b | Polarization mismatch between the reference antenna and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-5b | Mutual coupling between the reference antenna and the receiving antenna |  |  | 0.00 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| A1-6 | Phase curvature |  |  | 0.05 | Gaussian | 1.00 | 1 |  |  |  |  | 0.05 |  |
| C1-3 | Uncertainty of the network analyzer |  |  | 0.20 | Gaussian | 1.00 | 1 |  |  |  |  | [0.20] |  |
| A1-12 | Influence of the reference antenna feed cable |  |  | 0.05 | Rectangular | 1.73 | 1 |  |  |  |  | 0.03 |  |
| A1-13 | Reference antenna feed cable loss measurement uncertainty |  |  | 0.06 | Gaussian | 1.00 | 1 |  |  |  |  | 0.06 |  |
| A1-14 | Influence of the receiving antenna feed cable |  |  | 0.05 | Rectangular | 1.73 | 1 |  |  |  |  | 0.03 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  |  | 0.43 | Rectangular | 1.73 | 1 |  |  |  |  | 0.25 |  |
| A1-15 | Uncertainty of the absolute gain of the receiving antenna |  |  | 0.00 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | TBD |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | TBD |
| Table 6.6-2: CATR MU value derivation for EEIRP measurement, FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |  |  |  |
|  |  |  |  | 4.2<f≤7.125 GHz |  |  |  |  |  |  |  | 4.2<f≤7.125 GHz |  |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A2-18a | Misalignment and pointing error of BS (for TRP) |  |  | 0.30 | Rectangular | 1.73 | 1 |  |  |  |  | 0.173 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) |  |  | 0.50 | Gaussian | 1.00 | 1 |  |  |  |  | 0.50 |  |
| A2-2a | Standing wave between BS and test range antenna |  |  | 0.21 | U-shaped | 1.41 | 1 |  |  |  |  | 0.148 |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  | 0.00 | Gaussian | 1.00 | 1 |  |  |  |  | 0.001 |  |
| A2-4a | QZ ripple experienced by BS |  |  | 0.09 | Gaussian | 1.00 | 1 |  |  |  |  | 0.093 |  |
| A2-12 | Frequency flatness of test system |  |  | 0.25 | Gaussian | 1.00 | 1 |  |  |  |  | 0.250 |  |
| A2-17 | Measurement system dynamic range uncertainty |  |  | [0.51] | Gaussian | 1 | 1 |  |  |  |  | [0.51] |  |
| *new | [EEIRP sampling error] |  |  | [TBD] | [Gaussian] | 1 | 1 |  |  |  |  | [TBD] |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer |  |  | 0.20 | Gaussian | 1.00 | 1 |  |  |  |  | 0.20 |  |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver |  |  | 0.33 | U-shaped | 1.41 | 1 |  |  |  |  | 0.23 |  |
| A2-6 | Insertion loss of receiver chain |  |  | 0.18 | Rectangular | 1.73 | 1 |  |  |  |  | 0.10 |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  | 0.00 | Gaussian | 1.00 | 1 |  |  |  |  | 0.00 |  |
| A2-7 | Influence of the calibration antenna feed cable |  |  | 0.02 | U-shaped | 1.41 | 1 |  |  |  |  | 0.02 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  |  | 0.43 | Rectangular | 1.73 | 1 |  |  |  |  | 0.25 |  |
| A2-8 | Misalignment positioning system |  |  | 0.00 | Exp. normal | 2.00 | 1 |  |  |  |  | 0.00 |  |
| A2-18b | Misalignment and pointing error of calibration antenna (for TRP) |  |  | 0.50 | Exp. normal | 2.00 | 1 |  |  |  |  | 0.25 |  |
| A2-9 | Rotary joints |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.03 |  |
| A2-2b | Standing wave between calibration antenna and test range antenna |  |  | 0.09 | U-shaped | 1.41 | 1 |  |  |  |  | 0.06 |  |
| A2-4b | QZ ripple experienced by calibration antenna (normal test conditions) |  |  | 0.01 | Gaussian | 1.00 | 1 |  |  |  |  | 0.01 |  |
| A2-11 | Switching uncertainty |  |  | 0.26 | Rectangular | 1.73 | 1 |  |  |  |  | 0.15 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | TBD |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | TBD |

As with the TRP cases the CATR is slightly higher error so we can concentrate on this when considering the final MU value.

| Table 6.6-3: PWS MU value derivation for EEIRP measurement, FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |  |  |  |
|  |  |  |  | 4.2<f≤7.125 GHz |  |  |  |  |  |  |  | 4.2<f≤7.125 GHz |  |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A7-1a | Misalignment and pointing error of BS |  |  | 0.10 | Rectangular | 1.73 | 1 |  |  |  |  | 0.06 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) |  |  | 0.50 | Gaussian | 1.00 | 1 |  |  |  |  | 0.50 |  |
| A7-2a | Longitudinal position uncertainty (i.e. standing wave and imperfect field synthesis) for BS antenna |  |  | 0.20 | Rectangular | 1.73 | 1 |  |  |  |  | 0.12 |  |
| A7-3 | RF leakage (calibration antenna connector terminated) |  |  | 0.09 | Gaussian | 1.00 | 1 |  |  |  |  | 0.09 |  |
| A7-4a | QZ ripple experienced by BS |  |  | 0.77 | Rectangular | 1.73 | 1 |  |  |  |  | 0.44 |  |
| A7-5 | Miscellaneous uncertainty |  |  | 0.00 | Gaussian | 1.00 | 1 |  |  |  |  | 0.00 |  |
| A7-14 | System non-linearity |  |  | 0.22 | Rectangular | 1.73 | 1 |  |  |  |  | 0.13 |  |
| A7-13 | Frequency flatness of test system |  |  | 0.13 | Rectangular | 1.73 | 1 |  |  |  |  | 0.08 |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of network analyzer |  |  | 0.20 | Gaussian | 1.00 | 1 |  |  |  |  | 0.20 |  |
| A7-6 | Mismatch (i.e. reference antenna, network analyser and reference cable) |  |  | 0.45 | U-shaped | 1.41 | 1 |  |  |  |  | 0.32 |  |
| A7-7 | Insertion loss of receiver chain |  |  | 0.18 | Rectangular | 1.73 | 1 |  |  |  |  | 0.10 |  |
| A7-3 | RF leakage (calibration antenna connector terminated) |  |  | 0.09 | Gaussian | 1.00 | 1 |  |  |  |  | 0.09 |  |
| A7-8 | Influence of the calibration antenna feed cable |  |  | 0.10 | Rectangular | 1.73 | 1 |  |  |  |  | 0.06 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  |  | 0.43 | Rectangular | 1.73 | 1 |  |  |  |  | 0.25 |  |
| A7-9 | Misalignment of positioning system |  |  | 0.00 | Exp. normal | 2.00 | 1 |  |  |  |  | 0.00 |  |
| A7-1b | Misalignment and pointing error of calibration antenna |  |  | 0.05 | Rectangular | 1.73 | 1 |  |  |  |  | 0.03 |  |
| A7-10 | Rotary joints |  |  | 0.00 | U-shaped | 1.73 | 1 |  |  |  |  | 0.00 |  |
| A7-2b | Longitudinal position uncertainty (i.e. standing wave and imperfect field synthesis) for calibration antenna |  |  | 0.18 | Rectangular | 1.73 | 1 |  |  |  |  | 0.10 |  |
| A7-4b | QZ ripple experienced by calibration antenna |  |  | 0.20 | Rectangular | 1.73 | 1 |  |  |  |  | 0.12 |  |
| A7-11 | Switching uncertainty |  |  | 0.02 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A7-12 | Field repeatability |  |  | 0.20 | Gaussian | 1.00 | 1 |  |  |  |  | 0.20 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | TBD |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | TBD |

## <<End of Change>>