# 3GPP TSG-RAN WG4 Meeting #114                               R4-2500552

# Athens, Greece, 17th-21st Feb, 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on REFSENS, ASCS, ACS of LP-WUR

# Agenda Item:   7.26.2.2

# Document for:  Approval

- Introduction

In this contribution, we provided our views and simulation results on REFSENS, ASCS and ACS for LP-WUR.

- Discussion

## 2.1 REFSENS

In last meeting, companies provided their simulation results on target SNR for OOK-based receiver and OFDM-based receiver. The value of target SNR is also under discussion. Besides, companies had different opinions on NF and IM. Therefore, NF and IM can be packaged [1].

Issue 2-2-1: General views on REFSENS for LP-WUS Agreements: - Discuss REFSENS for the two types of receivers respectively, and then check if one single value or two values are specified pending on the final REFSENS values. Issue 2-2-3: companies input on SNR, NF and IM value for FR1 OOK-based LR Agreements: - Use 2.5dB IM as a starting point, encourage companies to report the difference aspects due to OOK waveform receiver. The corresponding band should also be reported. - The total value of NF+IM can be considered. - On how to handle the NF values: - Option 1: For NF whether average value can be adopted. - Option 2: two sets NFs for different LP-WUR types with different power consumption target - Option 3: also consider coverage - Capture the table in WF as background information

REFSENS for LP-WUR can be expressed as

REFSENS(dBm) = kT0 + 10log10(NRB_WUS*12*SCS) + NF + SNR + IM

where some other related parameters are not included for LP-WUR, such as diversity gain, because compared to the main receiver, 1Rx is used in LP-WUR architecture without the corresponding diversity Rx chain so that there is no need to consider diversity gain.

For SNR, we show the SNR evaluation results for both OOK-based receiver and OFDM-based receiver under different waveform types in Figure 1 and Table 1. We can find that OFDM-based receiver has better performance than OOK-based receiver thus it is necessary to differentiate these two receivers. Besides, OOK-4 waveform with M=4 has higher SNR requirement than other waveform, so we also provide the simulation results of OOK-4, M=4 with 3rd and 5th order filters in Fig. 1.

Table 1  SNR evaluation results for LP-WUR

| Receiver Type | Waveform Type | SNR to achieve 1% MDR/BLER |
| --- | --- | --- |
| OOK-based Receiver | OOK-4, M=1 | -6 |
|  | OOK-4, M=2 | -4 |
|  | OOK-4, M=4 | -2 |
| OFDM-based Receiver | OOK-4, M=1 | -10.5 |
|  | OOK-4, M=2 | -8 |
|  | OOK-4, M=4 | -5 |

Fig. 1  SNR evaluation for OOK-based receiver and OFDM-based receiver

We find that filter order has minor impact on LP-WUR performance in REFSENS environment because there isn’t interference.

Observation 1: OFDM-based receiver outperforms OOK-based receiver. OOK-4 waveform with M=4 has higher SNR requirement than other waveform.

Based on the simulation results above, we propose that -2dB SNR is enough for OOK-based receiver and -5dB SNR is enough for OFDM-based receiver.

For NF values, OOK-based receiver and OFDM-based receiver have quite different architectures and we should differentiate NF for OOK-based receiver and OFDM-based receiver. It is the common understanding that the NF for LR is higher than that for MR. We propose +5dB and +2dB on top of MR NF (9dB) for OOK-based receiver and OFDM-based receiver, respectively.

For IM, we agree that we can use legacy value as a start point.

We summarize our results on SNR, IM and NF for REFSENS of OOK-based receiver and OFDM-based receiver in Table 2 based on our simulation results and analysis.

Proposal 1: Consider the REFSENS results including SNR, IM and NF for OOK-based LP-WUR and OFDM-based LP-WUR in Table 2.

Table 2  SNR, IM and NF to derive REFSENS of LP-WUR

| Receiver Type | SCS (kHz) | WUS RB | SNR (dB) | IM (dB) | NF (dB) | REFSENS (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| OOK-based Receiver | 15 | 11 | -2 | 2.5 | 14 | -96.5 |
|  | 30 | 11 | -2 | 2.5 | 14 | -93.5 |
| OFDM-based Receiver | 15 | 11 | -5 | 2.5 | 11 | -102.5 |
|  | 30 | 11 | -5 | 2.5 | 11 | -99.5 |

## 2.2 ASCS

In last meeting, the simulation results provided by companies showed that guard RB has minor improvement on the required SNR to achieve 1% BLER/MDR. Therefore, there is no need to allocate guard RB for ASCS. With this conclusion, our proposal on the test parameters for ASCS is shown in Table 3. To perform ASCS test, no guard RB is needed and NR PDSCH should occupy all the RBs that are not used for LP-WUS with the same PSD as LP-WUS.

Proposal 2: Define the test parameters for ASCS of LP-WUR in Table 3. For these test parameters, the missed detection rate shall be ≤1%.

Table 3  Test parameters for ASCS of LP-WUR

| RX parameter | LP-WUS transmission bandwidth configuration (RB) |
| --- | --- |
|  | 11RB |
| LP-WUS Power (dBm) | LR REFSENS |
| BWinterferer | NR PDSCH is mapped on RBs not used for LP-WUS |
| In-band Pinterferer | Same PSD as LP-WUS |

## 2.3 ACS

The agreements on ACS for LP-WUR are listed as follows [1].

Issue 2-4-1: Phase noise for simulation assumption Agreements: - Companies can use different phase noise assumptions, the phase noise assumption should be reported Issue 2-4-2: General ACS requirements value and guard RB Agreements: - For ACS evaluation, companies are encouraged to provide the simulation results by assuming - Baseline: the same interference experienced by LP-WUR and main radio. - Optional: the different interference level for LP-WUR and main radio Issue 2-4-4: Test parameters for LP-WUR ACS case WF: - RAN4 further discuss detailed test parameters table and how to modify spec.

We use the phase noise model shown in R4-2415780 [2]. In legacy spec, to perform ACS test, the wanted signal power is set under a good condition with MR REFSENS + 14dB. Thus, we also provide the simulation results under LR REFSENS + 14dB condition for LP-WUR as shown in Fig. 2.

Fig. 2  LP-WUR performance in ACS case under different interference level and guard RB numbers for

OOK-based receiver and OFDM-based receiver

It can be found that OFDM-based receiver outperforms OOK-based receiver a lot. OFDM-based receiver can address up to 7dB higher interference level than OOK-based receiver. Therefore, we prefer to define different ACS requirements and test parameters for these two type of receivers.

Observation 2: OFDM-based receiver can address up to 7dB higher interference level than OOK-based receiver.

Moreover, with more guard RB numbers, LP-WUR can address higher interference level. To achieve better receiving performance, guard RBs can be allocated. But this depends on the network configuration. Besides, it should be noticed that for 5MHz CBW with 30kHz SCS, all the RBs are occupied by LP-WUS and no additional RBs are left for guard RBs. In this case, we suggest not to limit the number of guard RBs as a core requirement and it can be left flexible.

Considering that RAN4 is to define the minimum requirements, usually based on the worst case assumption, we prefer not to allocate guard RB number in ACS test parameters. According to our simulation results, 22dB ACS for OOK-based receiver and 29dB ACS for OFDM-based receiver is feasible.

Proposal 3: Not to define the number of guard RBs for ACS as a core requirement as well as in test parameters.

Proposal 4: ACS requirement for OOK-based receiver and OFDM-based receiver could be 22dB and 29dB, respectively.

Then, we propose the test parameters in Table 4-7 as follows, where 1.5dB implementation margin is considered.

Proposal 5: Define the test parameters for ACS of LP-WUR in Table 4-7. For these test parameters, the missed detection rate shall be ≤1%.

Table 4  Test parameters for ACS of OOK-based receiver, Case 1

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | LR REFSENS + 14 dB |
| Pinterferer | dBm | LR REFSENS + [34.5] dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + 2.5 / -(minimum guard band3 + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

Table 5  Test parameters for ACS of OOK-based receiver, Case 2

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | -56.5 |
| Pinterferer | dBm | [-36] |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + 2.5 / -(minimum guard band3 + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

Table 6  Test parameters for ACS of OFDM-based receiver, Case 1

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | LR REFSENS + 14 dB |
| Pinterferer | dBm | LR REFSENS + [41.5] dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + 2.5 / -(minimum guard band3 + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

Table 7  Test parameters for ACS of OFDM-based receiver, Case 2

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | -56.5 |
| Pinterferer | dBm | [-29] |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + 2.5 / -(minimum guard band3 + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

- Conclusion

Observations:

Observation 1: OFDM-based receiver outperforms OOK-based receiver. OOK-4 waveform with M=4 has higher SNR requirement than other waveform.

Observation 2: OFDM-based receiver can address up to 7dB higher interference level than OOK-based receiver.

Proposals:

Proposal 1: Consider the REFSENS results including SNR, IM and NF for OOK-based LP-WUR and OFDM-based LP-WUR in Table 2.

Table 2  SNR, IM and NF to derive REFSENS of LP-WUR

| Receiver Type | SCS (kHz) | WUS RB | SNR (dB) | IM (dB) | NF (dB) | REFSENS (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| OOK-based Receiver | 15 | 11 | -2 | 2.5 | 14 | -96.5 |
|  | 30 | 11 | -2 | 2.5 | 14 | -93.5 |
| OFDM-based Receiver | 15 | 11 | -5 | 2.5 | 11 | -102.5 |
|  | 30 | 11 | -5 | 2.5 | 11 | -99.5 |

Proposal 2: Define the test parameters for ASCS of LP-WUR in Table 3. For these test parameters, the missed detection rate shall be ≤1%.

Table 3  Test parameters for ASCS of LP-WUR

| RX parameter | LP-WUS transmission bandwidth configuration (RB) |
| --- | --- |
|  | 11RB |
| LP-WUS Power (dBm) | LR REFSENS |
| BWinterferer | NR PDSCH is mapped on RBs not used for LP-WUS |
| In-band Pinterferer | Same PSD as LP-WUS |

Proposal 3: Not to define the number of guard RBs for ACS as a core requirement as well as in test parameters.

Proposal 4: ACS requirement for OOK-based receiver and OFDM-based receiver could be 22dB and 29dB, respectively.

Proposal 5: Define the test parameters for ACS of LP-WUR in Table 4-7. For these test parameters, the missed detection rate shall be ≤1%.

Table 4  Test parameters for ACS of OOK-based receiver, Case 1

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | LR REFSENS + 14 dB |
| Pinterferer | dBm | LR REFSENS + [34.5] dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + 2.5 / -(minimum guard band3 + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

Table 5  Test parameters for ACS of OOK-based receiver, Case 2

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | -56.5 |
| Pinterferer | dBm | [-36] |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + 2.5 / -(minimum guard band3 + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

Table 6  Test parameters for ACS of OFDM-based receiver, Case 1

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | LR REFSENS + 14 dB |
| Pinterferer | dBm | LR REFSENS + [41.5] dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + 2.5 / -(minimum guard band3 + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

Table 7  Test parameters for ACS of OFDM-based receiver, Case 2

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | -56.5 |
| Pinterferer | dBm | [-29] |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + 2.5 / -(minimum guard band3 + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

- Reference

- R4-2420394 WF on UE RF requirements for LP-WUS, vivo, RAN4#113, Orlando, USA, Dec 2024.

- R4-2415780 Further discussions on LP-WUS REFSENS, ASCS and ACS, vivo, RAN4#112bis, Hefei, China, Oct 2024.