# 3GPP TSG-RAN WG4 Meeting #114	R4-2500631

# Athens, GR, 17th – 21st February, 2025

Source: 	Xiaomi

Title: 	Discussion on (e)RedCap RRM requirements in NTN for NR Phase 3

Agenda Item:	7.29.4.1

Document for:	Discussion

# 1Introduction

In the last RAN4#112bis meeting, further discussion on RRM requirements for Rel-19 NR NTN phase3 took place and a WF was approved as detailed in [1]. In this contribution, we would like to provide our further discussion on (e)RedCap RRM requirements in NTN for NR Phase 3 and provide our proposals.

- Discussion

General consideration on RRM requirements

Issue 1-1-1: Whether to define measurement requirement for inter-RAT (NR NTN to E-UTRAN TN) in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN? <Way Forward>: - Proposal 1 (CATT, Xiaomi, LG, Samsung CMCC, Apple, Ericsson, HW, ZTE): - RAN4 to define measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN. - Proposal 2 (vivo): - RAN4 not to define the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

Cell re-selection requirement for (e)RedCap UE with FR1-NTN is still under discussion. The open issue is whether to define the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement. In R18 NTN, inter-RAT cell resection requirements from NTN in FR1-NTN to LTE TN was introduced. In R17 RedCap, inter-RAT E-UTRAN measurement requirements was also introduced. So, from our perspective, we think it is reasonable to define the measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement.

Proposal 1: RAN4 to define measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement for (e)RedCap UE with FR1-NTN.

Issue 1-2-1: Relaxed measurement Online agreement < Agreement >: - For supported types of relaxations for (e)RedCap UE with FR1-NTN, - Only to support R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for cell re-selection. - RAN4 to further discuss the thresholds and specific relaxation factor.

Based on the current TS 38.133, NTN has already supported Rel-16 relaxation functionality introduced for TN, i.e. relaxed measurement criterion for UE with low mobility and relaxed measurement criterion for UE not at cell edge. RAN4 has defined the same relaxed measurement requirements for NTN as TN. The related clauses are captured below:

#### 4.2C.2.7 Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion The requirements in clause 4.2.2.9 apply provided that target cell’s satellite is GEO. #### 4.2C.2.8 Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion The requirements in clause 4.2.2.10 apply provided that target cell’s satellite is GEO.

Based on last meeting agreements that (e)RedCap UE with FR1-NTN would only support R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for cell re-selection, it is straightforward to reuse the above captured requirements for (e)RedCap UE with FR1-NTN. Which is to say, the specific relaxation factor would reuse the values introduced for TN R16 relaxation.

As for the thresholds for relaxation evaluation, we think the thresholds discussion falls within the scope of RAN2. RAN2 has defined independent signaling for the relaxation criterion introduced in different release. RAN4 has already agreed to support the R16 NR TN relaxation for (e)RedCap UE with FR1-NTN, we think it is clear that the R16 relaxation thresholds configuration can be reused.

Proposal 2: RAN4 to reuse the thresholds and specific relaxation factors specified for R17 NR NTN relaxation, i.e. R16 NR TN relaxation, for (e)RedCap with FR1-NTN.

Issue 1-2-2: eDRX configuration Online agreement < Agreement >: - For earth-moving LEO scenario: - All eDRX cycle cannot be applied for RedCap/eRedCap UEs in NTN and not to define requirements for eDRX in earth-moving beam LEO scenario. - For quasi-earth fixed LEO scenario: - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN. - Requirements are applicable for 2.56s and 5.12s eDRX cycle - FFS for 10.24s eDRX cycle and larger than 10.24s eDRX cycle

The Rel-17 RedCap WI introduced extended DRX cycles for RRC Idle state (up to 10485.76 seconds) and RRC Inactive state (up to 10.24 seconds). Furthermore, the Rel-18 eRedCap WI introduced enhanced eDRX cycle length in RRC Inactive state beyond 10.24 seconds, up to the same eDRX cycle length as for RRC Idle state (up to 10485.76 seconds). During discussion in the (e)RedCap WI, the eDRX feature is introduced as an optional feature for both RedCap and non-RedCap UEs. As the eDRX feature is not specific for (e)RedCap UE, we think RAN4 should discuss whether to support eDRX feature for NR NTN.

Based on last meeting discussion, the remaining open issue is whether to support eDRX cycle=10.24s and eDRX cycle larger than 10.24s for quasi-earth fixed LEO scenario. We understand the serving time can be up to several minutes for quasi-earth fixed LEO deployment. Considering that the number of samples needed for Tdetect/Tmeasure/Tevaluate should be completed within the serving time, we think the upper limit for the applicable eDRX cycle can be 10.24s.

As for eDRX cycle larger than 10.24s, we prefer not to introduce for NTN. In case of eDRX cycle larger than 10.24s, UE in TN is allowed to detect neighbor cell(s) across multiple eDRX cycles. It would be impractical in quasi-earth fixed LEO scenario. In this case, we understand that applicability condition need to be further discussed. As the power saving demand is not that critical for NTN UE, we prefer not to consider the eDRX cycle larger than 10.24s case.

Proposal 3: RAN4 to define measurement requirements for eDRX cycle length = 10.24s for quasi-earth fixed LEO scenario.

Issue 1-2-3: Concurrent gap Online agreement < Agreement >: - Deprioritize parallel measurement gap for (e)RedCap UEs in NTN. - Come back to this issue at Apr 2025 meeting. Issue 1-2-4: Multiple SMTCs <Agreements>: - RAN4 to define RRM requirements for multiple SMTC-s for a single frequency carrier for (e)RedCap UEs. - For whether to support parallel measurements on multiple NGSO satellites within a SMTC (Feature 25-5) for (e)RedCap UEs? - Option 1 (MTK, Xiaomi, Samsung, ZTE, Ericsson, LG, QC): Not support - Option 2 (HW, Nokia): FFS - Further discuss the supported number of SMTCs, the number of cells and SSB, the number of supported satellites, the proximity distance between two SMTCs. - Further discuss the specific measurement requirements, i.e, whether and how to extend measurement period. - Further discuss other R17 NTN UE capabilities (i.e, Feature 25-2/25-3) related to 'in parallel'.

In NTN Rel-17, a NTN UE shall at the least be capable of 2 SMTCs , and up to 4 SMTCs based on UE capability, with same periodicity but different offsets to measure neighbour satellites and the serving satellites. Additionally, UE capability of supporting parallel measurements on multiple NGSO satellites within a SMTC (Feature 25-5) was introduced. Considering the reduced complexity of (e)RedCap, it is challenging for RedCap UE to support parallel measurements. So we think it is more reasonable not to define requirements for the case of parallel measurements for (e)RedCap UE with FR1-NTN.

- Proposal 4: (e)RedCap UE with FR1-NTN do not support parallel measurements on multiple NGSO satellites within a SMTC (Feature 25-5).

- Furthermore, considering the supported number of SMTCs, the number of cells and SSB and the number of supported satellites, we think the number for normal NTN UE could be reused for NTN RedCap. As in TN development, RedCap UE reused the the number of cells and number of SSB defined for normal UE.

- Proposal 5: The supported number of SMTCs, the number of cells and SSB and the number of supported satellites defined for NTN normal UE can be reused for NTN RedCap.

- In current cell re-selection requirements for NTN, the factor NSMTC,overlap only counts the number of SMTCs that partially overlap with each other. NTN UE is not required to conduct measurements in multiple SMTCs in parallel. The measurements in multiple SMTCs can target for different frequency layers or different satellites. There may exist RF adjustment for the measurement in different SMTCs. We understand this is the reason for introducing collision between multiple SMTCs on a SAN carrier in RRC_CONNECTED mode. In order to reduce UE complexity, we are fine to define the proximity distance between two SMTCs.

- Proposal 6: RAN4 to define the proximity distance between two SMTCs in RRC_IDLE/INACTIVE mode for (e)RedCap UEs in NTN.

Reduced UE complexity

Based on Rel-17 and Rel-18 progress, the main characteristic for RedCap UE is the reduced UE complexity. RedCap UE supports 1 or 2 UE Rx branches and HD-FDD or FD-FDD operation. Based on last meeting progress, it was agreed to follow the same principle for (e)RedCap UE in TN network for (e)RedCap UEs with FR1-NTN.

Besides some additional clarification on available samples for HD-FDD operation, the same RRM requirements as for normal NR UE are reused for Rel-17 RedCap with 2Rx, while the relaxed measurement requirements are defined for Rel-17 RedCap with 1Rx. The relaxation method including extending the number of measurement samples and relaxing the measurement accuracy. RAN4 has also agreed to reuse the existing requirements for NTN as a baseline for 2Rx (e)RedCap UEs with FR1-NTN.

In last meeting discussion, great progress was achieved. The only remaining issue is on requirements of measurement procedure for 1Rx/2Rx (e)RedCap UEs with FR1-NTN. From our perspective, when defining RRM requirements for 1RX (e)RedCap UEs with FR1-NTN, similar principle as 1RX RedCap in TN network could be taken.

- Proposal 7: RAN4 to define RRM requirements for 1 RX (e)RedCap with FR1-NTN by relaxing requirements of 2 RX RedCap with FR1-NTN:

- Measurement sample extension for time index detection requirements;

- Measurement sample extension for PSS/SSS detection requirements;

- Measurement accuracy relaxation for L3 and L1 measurement accuracy requirements.

- Conclusion

Proposal 1: RAN4 to define measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement for (e)RedCap UE with FR1-NTN.

Proposal 2: RAN4 to reuse the thresholds and specific relaxation factors specified for R17 NR NTN relaxation, i.e. R16 NR TN relaxation, for (e)RedCap with FR1-NTN.

Proposal 3: RAN4 to define measurement requirements for eDRX cycle length = 10.24s for quasi-earth fixed LEO scenario.

- Proposal 4: (e)RedCap UE with FR1-NTN do not support parallel measurements on multiple NGSO satellites within a SMTC (Feature 25-5).

- Proposal 5: The supported number of SMTCs, the number of cells and SSB and the number of supported satellites defined for NTN normal UE can be reused for NTN RedCap.

- Proposal 6: RAN4 to define the proximity distance between two SMTCs in RRC_IDLE/INACTIVE mode for (e)RedCap UEs in NTN.

- Proposal 7: RAN4 to define RRM requirements for 1 RX (e)RedCap with FR1-NTN by relaxing requirements of 2 RX RedCap with FR1-NTN:

- Measurement sample extension for time index detection requirements;

- Measurement sample extension for PSS/SSS detection requirements;

- Measurement accuracy relaxation for L3 and L1 measurement accuracy requirements.

- Reference

- R4-2420104, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT