3GPP TSG-RAN WG4 Meeting # 114                                    R4-2500703

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.27.2.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on CSI-RS based L1-RSRP measurement

Document for:	Approval

Introduction

In Rel-18, LTM was introduced to reduce handover delay and interruption time compared to layer3 based mobility. As mentioned in WID [1], LTM as introduced in Rel-18 has a number of limitations compared to L3 mobility. In Rel-19, these limitations should be removed as much as possible.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution, we will propose our views on RRM requirement for CSI-RS based L1-RSRP measurement.

Discussion

In previous meeting, we have discussed the RRM scope of CSI-RS based L1-RSRP measurement and reached consensus that RAN4 should discuss at least measurement delay, measurement reporting requirements, measurement restrictions and scheduling availability for CSI-RS based L1-RSRP measurement on LTM candidate cells, which is considered in high level and same as the procedure of study on SSB based L1-RSRP measurement on candidate cells.

Similar as SSB based L1-RSRP measurement on neighbor cells, RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells. And we discussed the known cell condition in CSI-RS based L1 measurement last meeting.

For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met: - Agreement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - Further check whether to consider SSB-based L3 measurement only - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2. - L1 measurement in the 2nd bullet is not needed for FR1. - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet. - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable]. - Further discuss whether to use detectable or measurable. - Further discuss the side condition

For the first bullet, we have reached consensus that consider SSB based L3 measurement as the pre-requisite condition only. E.g. no need to consider CSI-RS based L3 measurement as condition. For the last two bullets, further discussion is needed. In this discussion paper, we will to express our views on these issues

For the second bullet, whether to keep L1 measurement depends on whether it is necessary to perform SSB based L1 measurement before performing CSI-RS based L1 measurement may involve the procedure of CSI-RS based L1 measurement. It is associated with another issue which is under discussion.. In our understanding, for LTM,  CSI-RS based L1 measurement and SSB based L1 measurement are seperated features and each can work seperately. Therefore, prefer to remove L1 measurement in the second bullet.

Proposal 1: Remove Rel-18 LTM SSB-based L1-RSRP measurements.

For the third bullet, whether to use detectable or measurable on the description of CSI-RS from the target cell configured for L1 measurement. Prior to this, we believe that a reasonable approach is to distinguish the difference between these two descriptions.

In the current specification, detectable is used to describe SSB. According to information provided by other companies, it is considered inaccurate to use detectable here for CSI-RS because there are clear identification requirements for SSB in clause 9.2, and it is possible to perform cell search and detect SSB without prior knowledge. If no condition is specified for CSI-RS, maybe use detectable is inaccurate. To avoid confusion with SSB, we agree with this view, but further discussion may be needed on how to understand measurable.

The cell is considered as known if the following conditions are met in this requirement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.2 remains detectable according to the cell identification requirements specified in clause 9.2. Otherwise, the cell is unknown.

Proposal 2: Use measureble to avoid confusion with SSB, and further discussion may be needed on how to understand measurable

- Issue 3-2: Measurement procedure <Agreement> - Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements and reports at least before LTM configuration. - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions, or based on UE capability or based on network RRC configuration. FFS the details. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Depending on whether step 2 is skipped, the requirements in step 3 may be different. - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.]

The controversial aspect of this issue lies in step 2, whether SSB-based L1 measurements and reports on the same candidate cell are needed to be performed before executing CSI-RS based L1 measurements. Consistent with the previous issue, we think that SSB-based L1 measurements do not need to be conducted before CSI-RS based L1 measurements, and they should be able to be conducted independently. Furthermore, if step 2 is completed first, there seems to be no need for the existence of CSI-RS L1 measurement. So we prefer to skip step 2. Of course, some conditions can be given, such as the configuration of repetition, etc

Proposal 3: Step 2 about SSB-based L1 measurements and reports on the same candidate cell for FR2-1 can be skipped under certain conditions, or based on UE capability or based on network RRC configuration.

In the last meeting, we have discussed the definiation of intra-f and inter-f measurement for CSI-RS based L1 RSRP measurement. And there is an agreement with some bracket.

- Issue 3-3: Definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement <Agreement> - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of [active DL BWP], and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of [active DL BWP], and - It is applied for SCS = 60KHz - [the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement (if configured).]

Observe the current conclusion, it is samilar as the definition of intra-frequency/inter-frequency CSI-RS based L3 measurement, which is aglin with the previous discussion for measurement. Of course, there are some differences with CSI-RS based L3 measurement. The main reason is the different configuration methods of CSI-RS, one is to follow CSI-RS configuration for MO (MeasObjectNR), the other one is to follow NZP-CSI-RS-Resource structure.

In order to address the issue that different CSI-RS resources on the serving cell may have different SCS and CP types, active DL BWP is used instead of "serving cell" in legacy L3 measurement. Meanwhile, regarding center frequency, any of the CSI-RS resources of the serving cell indicated for L1 measurement is used for judgment. In our opinion, such modifications can solve the problem of L1 and L3 mismatch based on the definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement. So, we are fine with the wording and prefer to remove brackets.

Proposal 4: Prefer to remove brackets and a measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

  - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

  - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

    - It is applied for SCS = 60KHz

  - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement (if configured).

Meanwhile, for CSI-RS based L1-RSRP measurement, we discussed the measurement type, mainly study whether to cover with and without gap.

- Issue 3-8: supported measurement type <Agreement> - CSI-RS based L1 measurement without gap is supported. - FFS on CSI-RS based L1 measurement with gap

In our views, both with and without gap should be covered due to this is for neighbor cell measurement and measurement with gap is a typical scenario. Some companies proposed the concern. For serving cell, there is no specific restriction in time domain on configurations of CSI-RS resources for L1-RSRP measurement. So, it’s difficult to ensure all the CSI-RS of the same frequency layer are within a single gap window.

For this issue, we think it can be solved by using the same principle for CSI-RS based L3 measurement that all CSI-RS resources in the same MO are confined in the same 5ms window. However, if we consider perform SSB based L3 measurement and CSI-RS based L1 measurement in FR2, it is difficult to use one gap to cover both SSB based L3 or L1 measurement and CSI-RS based L1 measurement, so concurrent gap shall be used.

Proposal 5: Support CSI-RS based L1 measurement with gap.

Issue 2-12: RTD assumption - Further discuss based on the following assumption: - RTD is the received time difference among CSI-RS and SSB from different cells. - Alt 1: define requirements for both RTD<CP and RTD>CP. Introduce an optional UE capability for RTD>CP. - Alt 2: only define requirements for RTD<CP. - NW shall only enable this when RTD<CP is guaranteed.

As to RTD assumption, RTD>CP is a typical case for neighbor cell measurement. It’s reasonable to consider this case in CSI-RS based L1-RSRP measurement. We prefer to define requirements for both RTD<CP and RTD>CP. Meanwhile, the same method as SSB based L1-RSRP measurement can be used, i.e., introduce an optional UE capability for RTD>CP. However, as mentioned by UE vendor, it’s complicated to use different FFT for CSI-RS based measurement. Consider UE implementation and RTD>CP has been supported in SSB based measurement, it’s ok for us to deporitize the discussion on RTD>CP.

Proposal 6: Prefer to define requirements for both RTD<CP and RTD>CP.

For the issues about CSI acquisition on candidate cell(s) before or during LTM cell switch and CSI-RS based L1-SINR measurement, they are remain open in RAN1. Therefore, we should wait for more progress from other group before defining requirements.

Proposal 7: Wait for more RAN1 progress to start the discussion on CSI acquisition in RAN4.

Proposal 8: Focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement.

As to the accuracy and side condition for CSI-RS based L1-RSRP measurement, we prefer to reuse CSI-RS based L1-RSRP accuracy of the serving cell. In our understanding, one CSI-RS resource is used for both serving and neighboring cell measurements, e.g., CSI-RS resources for L1-RSRP measurement on neighbor cell for UE#1 are in fact the resources for L1 measurement of serving cell for UE#2. Meanwhile, measurement accuracy is performance requirement, we can discuss it in performance part.

Proposal 9: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline.

Conclusions

In this contribution, we put forward the following proposals on R19 L1/L2 triggered mobility.

Proposal 1: Remove Rel-18 LTM SSB-based L1-RSRP measurements.

Proposal 2: Use measureble to avoid confusion with SSB, and further discussion may be needed on how to understand measurable.

Proposal 3: Step 2 about SSB-based L1 measurements and reports on the same candidate cell for FR2-1 can be skipped under certain conditions, or based on UE capability or based on network RRC configuration.

Proposal 4: Prefer to remove brackets and a measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

  - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

  - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

    - It is applied for SCS = 60KHz

  - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement (if configured).

Proposal 5: Support CSI-RS based L1 measurement with gap.

Proposal 6: Prefer to define requirements for both RTD<CP and RTD>CP.

Proposal 7: Wait for more RAN1 progress to start the discussion on CSI acquisition in RAN4.

Proposal 8: Focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement.

Proposal 9: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline.

References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2420101, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple