3GPP TSG-RAN4 Meeting #114	R4-2500726

Athens, GR, February 17 – 21, 2025

Agenda item:			7.6.2

Source:	vivo

Title:	Discussion on methods for reducing the number of UE RX chains

Document for:	Discussion

- Introduction

In RAN4#113 meeting, the WF[1] has been agreed as follows, and this contribution will discuss about the remaining open issues.

#### Issue 2-1: Discussion on baseband assumptions Agreement: - Consider only same SCS for both CCs for one Rx RF chain architecture in this SI - 15KHz SCS is used as the starting point - Single FFT or separate FFT can be left to UE implementation, and however the condition of single FFT compared to separate FFT can be captured in the TR. Issue 2-5: Triggering condition to enable ”One RF Rx chain” RAN4 to identify the mechanisms that surround the UE implementation, i.e. whether one Rx RF chain operation can be enabled. - Further study whether the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled - Other determining factors are not precluded to be studied - Study feasibility to measure total aggregated power of all interferences in the gap between CC1 and CC2 Issue 3-3: Discussion of impacts on DL performance for UE enabling fragmented carrier mode - Companies are encouraged to bring analysis with their own assumptions following the table format - A separate table would be needed for Tx leakage + blocker and individual CA channel bandwidth combination in Table 7.3A.2.2-1 of TS 38.101-1 #### Issue 3-4: Discussion on in-gap requirements - RAN4 evaluate performance impact on the cases: - image of in-gap interference overlaps with wanted carrier - For the cases, image of ACS/IBB1/IBB2/NBB overlaps with wanted CC - image of in-gap blocking interference does not overlap with wanted carrier #### Issue 2-4: Signaling related Companies are encouraged to provide analysis with pros and cons on preferred signalling solutions so that they can be discussed in next meeting. #### Issue 3-1: Discussion on evaluation priority of example bands It would be up to companies’ preference and contribution driven on the example bands evaluation. The study needs to cover evaluation on both FDD and TDD bands and focus on general requirements. Band specific requirements can be left for WI phase. Open issues (for information) #### Issue 2-7: Others RAN4 to clarify whether either of the two fragmented carriers shall be a PCELL or if fragmented carriers can be defined purely as DL SCELL’s #### Issue 3-2: Discussion on maximum power level of in-gap interference For one Rx RF chain, the maximum acceptable absolute power of in-gap interferer should be less than or equal to -25dBm, specific value is FFS #### Issue 3-5: Discussion on DL fragmented carriers in TDD bands #### Background: To consider transition time if there’s one LO for the TDD band and study the feasibility Discuss following options for the number LO’s needed for fragmented carrier operation in TDD with one of the carriers having UL allocation:  Option 1: One LO will be enough for both UL and DL.  Option 2: Separate LOs will be needed for UL and DL.

- Discussion

## Triggering condition to enable “One RF Rx chain”

As has been specified in the spec for fully separate and partially shared architecture, the absolute power level of the in-gap interferer and the relative power imbalance between it and the wanted signal is the test parameter and condition of ACS and IBB applying. With the corresponding test parameters, when the throughput is ≥ 95 % of the maximum throughput of the reference measurement channels then the ACS/IBB requirements are considered to be fulfilled. With RX sharing these requirements should be relaxed to different levels according to the frequency range and bandwidth of the in-gap interferer.

However, there should be some trigger conditions on whether RX sharing is allowed. When the condition is met then RX sharing could be performed and then the receive performance and demodulation effect could be verified further. If there is no trigger condition the UE would not know when to assess the receiving performance based on the relaxed set of requirements. RX chain sharing should be an optional capability and not mandatory, nor is it the default state for the UE.

Therefore, it is illogical for the UE defaults to rely on the fully shared architecture for signal reception with the relaxed DL requirements. Instead, the UE should first determine some basic information, such as when the interference signal’s absolute power and/or relative power density difference satisfy the corresponding requirement then the RX chain could switch to shared state and the received signal could be evaluated by a different set of requirements.

The conditions for switching to a shared RX chain can include: the PSD difference between the interference signal and the target signal does not exceed a certain value, the frequency separation between two DL CCs does not exceed a certain value, and the time difference of the received signals between the two DL CCs does not exceed a certain value, etc.

Proposal 1: RX chain sharing should be an optional capability and not mandatory, nor is it the default state for the UE.

Proposal 2: There should be some trigger conditions on whether RX sharing is allowed:

- The PSD difference between the in-gap interferer and target CC does not exceed a certain value.

- The frequency separation between two DL CCs does not exceed a certain value.

- Other triggering conditions are not precluded.

Detecting & reporting related issues

How to know the PSD difference or the absolute power of the interference should depend on the UE detecting the environment and judging the blocker signal level.

As the blocker is from another operator and its power could not be restricted by the deployment, the NW could not directly know the power strength either. Hence the corresponding information could only be detected by the UE. The NW should first indicate the UE to measure the power or PSD of a certain frequency range, and then the UE would report the corresponding measurement information of the in-gap blocker to the NW.

At the same time, the mechanism of detection needs to be discussed, including detection duration and detection period. For example, the process of detection is continuous or only carried out in a certain period of time, which determines the frequency and speed of switching. Theoretically, not all UEs can support continuous detection, some UEs need to interrupt signal transmission during detection.

Proposal 3: The NW should indicate the frequency information of the gap, and the UE should detect the power difference (or PSD difference) between the in-gap blocker and the target DL CCs and report it back to the NW.

Decision maker of switching

From one side, when the sharing condition is satisfied, the UE with sharing capability could decide to switch to the shared RX chain by itself and the relaxed set of requirements would take place of the legacy set. The switching response is faster and the time for the UE to report to the NW is omitted. When the state of the blocker changes, the switching response will be relatively quick, but the UE still needs to report the link status for the NW to determine whether to indicate a new CA configuration.

From the other side, the indication of switching could also be from the NW, i.e., with the capability of sharing the RX chain reported by the UE and the information that the blocker fulfills the switching condition, the NW could decide to switch the state of the RX chain and indicate higher CA combinations with more CCs. If the NW determines and indicates when to perform the switch, it can simultaneously indicate the new CA configuration, allowing the UE to more efficiently transmit data directly using the new CA combination or even the freed MIMO layers. From the perspective of improving the efficiency of the overall RX chain switching and CA configuration switching, the scheme that NW indicates the switching seems more reasonable.

Proposal 4: It is up to the NW to decide when to perform the RX chain switching.

## CA configuration indication scheme

With the RX chains switching to shared architecture, the RF requirements should be adjusted and some RF chains could be released, potentially allowing for the aggregation of more CCs or MIMO layers. This would result in a wider variety of CA configurations supported by the UE. There are two kinds of methods for a UE to inform the network of new CA configurations it supports only when shared RX chain and adjusted RF requirements are used:

- The new CA configuration, could be reported to the NW together with other default supported CA configurations, as they are all related to the stable hardware capabilities. And it could be reported through a relative static mechanism, such as RRC signaling, as the current technology does. The RX chain state and CA configuration indication could be dynamically switched according to the environment.

- The new CA configuration could be separately reported to the NW when or after the RX chains switch to be shared, which is more dynamic but complex. When the RX chains could be switched to or after switching to the shared architecture, the UE reports to the NW that the new CA configuration it supports with the sharing architecture, and then the NW could send a new CA configuration to indicate to the UE that there are more CCs to be aggregated.

Proposal 5: The new CA configuration supported only by fully shared RX chains could be reported to the NW together with other default supported CA configuration.

## Fall back issue

When the prerequisite of sharing is not met, the UE should fall back to partially-shared or fully-separated architecture, and maybe a non-CA state from [2]. The trigger factor for falling back may be that the output power (or PSD) of the in-gap interferer is too high to maintain a sharing RX chain with good receiving performance, or the frequency separation between the two target CCs are too wide to be covered in one RX RF chain.

With the RX chains switching to being shared, the RF requirements should be adjusted and some RF chains could be released, potentially allowing for the aggregation of more CCs. This would result in a wider variety of CA configurations supported by the UE. However, these new CA configurations could only be indicated with shared RX chains. Also, when the RX chains fall back to separate architecture, the CA configuration should also switch to the default group with less CCs combined. The abandoned CC could be one of the non-contiguous intra-band CCs or the other inter-band CC. For example, the CA config in original separate architecture is CA_nX(2A), after RX chain sharing it may change to CA_nX(2A)-nY as the two CCs in band nX could be received by only one RX RF chain. However, when the fall back mechanism is triggered then the shared architecture should switch to separate state and then the CA config would also fall back to CA_nX(2A) or CA_nX-nY, the decision on which CC to disconnect is up to the network.

Observation 1: The CA configurations supported by shared and separate RX architecture are not always the same, some CA configurations are supported only in the shared state.

Proposal 6: The fallback of the Rx chain may also require corresponding changes to the CA configuration, the decision on which CC to disconnect is up to the network.

How to deal with multiple in-gap blockers

Considering the analysis in [3,4], the in-gap interference may not only come from one single operator, and there could also be quite a few different combinations of blocker power and gap width. Therefore, how to evaluate all the in-gap signals’ power or PSD differences should be discussed. For the gap that is no wider than the interference’s bandwidth of ACS, it is rather easy to just refer to the corresponding PSD difference requirement of ACS. The complex situation is the gap’s width is even larger than the interference range of the IBB case2. In such a wide gap, many blockers may fall in different frequencies and originate from different operators. In a rough analysis, there are three possible options to evaluate the PSD:

Option1. Comparing all the in-gap requirements including ACS\IBB1\IBB2 and choosing the most stringent PSD difference requirement of them e.g., ACS, as the final requirement for all the in-gap blockers, and no matter which frequency range the blocker falls in, only when the PSD difference between it and either of the target CCs is no higher than the most stringent requirement then the RX chain sharing could be indicated or performed. It will guarantee the DL performance to maintain as a relatively stable level but in that case, there are fewer cases available for RX chain sharing.

Option2. Choosing the most lenient one among the in-gap requirements as the final PSD difference indicator for all the blockers, so that the sharing condition will be relaxed and more cases could enjoy the sharing architecture. However, the degradation of DL performance will be much severe, especially for the blockers falling close to the DL CCs.

Option3. Re-unify a compromising metric for the gap's entire bandwidth, which is more difficult to assess.

Observation 2: For wide Wgap cases, the formulation of PSD difference should be a trade-off between ensuring receive link performance and enhancing the potential for RX chain sharing.

- Conclusion

Proposal 1: RX chain sharing should be an optional capability and not mandatory, nor is it the default state for the UE.

Proposal 2: There should be some trigger conditions on whether RX sharing is allowed:

- The PSD difference between the in-gap interferer and target CC does not exceed a certain value.

- The frequency separation between two DL CCs does not exceed a certain value.

- Other triggering conditions are not precluded.

Proposal 3: The NW should indicate the frequency information of the gap, and the UE should detect the power difference (or PSD difference) between the in-gap blocker and the target DL CCs and report it back to the NW.

Proposal 4: It is up to the NW to decide when to perform the RX chain switching.

Proposal 5: The new CA configuration supported only by fully shared RX chains could be reported to the NW together with other default supported CA configuration.

Observation 1: The CA configurations supported by shared and separate RX architecture are not always the same, some CA configurations are supported only in the shared state.

Proposal 6: The fallback of the Rx chain may also require corresponding changes to the CA configuration, the decision on which CC to disconnect is up to the network.

Observation 2: For wide Wgap cases, the formulation of PSD difference should be a trade-off between ensuring receive link performance and enhancing the potential for RX chain sharing.

- References

- R4-2420337, WF on NR FR1 DL fragmented carriers study, MediaTek Inc. RAN4#113.

- R4-2415804, On methods for reducing the number of UE Rx chain, Huawei, HiSilicon, RAN4#112bis.

- R4-2418532, On UE RF requirements for fragmented carriers, Apple, RAN4#113.

- R4-2419455, Discussion on Fragmented Carrier impacts on UE RF requirements, Nokia, RAN4#113.