# 3GPP TSG-RAN WG4 Meeting #114	R4-2500735

# Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Discussion on system parameters for less than 5MHz in FR1-NTN bands

Agenda Item:	7.10.3.1

Document for:	Approval

Introduction

In the last meeting, we further discussed system parameters for less than 5MHz in FR1-NTN bands. For most of the requirements, they were defined the same way with 3MHz in FR1 TN bands. In this contribution, we further discuss the remaining issues of system parameters for less than 5MHz for NTN.

Discussion

According to the WF [1], the remaining issues for less than 5MHz for NTN are captured as follows:

Issue 1-1: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional?

- Option 1: Optional

- Option 2: Mandatory

In FR1 NR TN, 3MHz was defined as an optional feature and release independent from Rel-18. We can still follow the same logic for FR1-NTN.

Table 5.1-3: NR channel bandwidths

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the channel bandwidth was introduced) |
| --- | --- | --- | --- |
| NR channel bandwidths, all unless otherwise stated | FDD, TDD, SDL, SUL | Rel-15 | Table B.4.1-1, Table B.4.3-1 |
| Channel bandwidth of 3 MHz | FDD | Rel-18 | Table B.4.1-1 |

Usually, a new bandwidth is introduced in later releases and it is optional from the release firstly introduced. In FR1-NTN, we suggest to introduce 3MHz as an optional feature and release independent from Rel-19. Support of 3MHz requires additional baseband capability, which may need modification on modem and RF to support this feature. In this sense, we prefer 3MHz as an optional feature. However, if there is strong request from operators, we are OK to mandate this feature.

Proposal 1: Channel bandwidth 3MHz for FR1-NTN can be an optional feature in Rel-19 unless there is strong request from Operator to make this feature mandatory.

Issue 1-2: UE capability Signalling

- FFS on re-use of existing rel-18 signalling parameters

Issue 1-3: If 3MHz is a mandatory feature, the UE behaviour to support 3MHz Channel bandwidth

- WF:

  - FFS on UE behaviours if 3MHz CHBW is mandatory for certain band.

    - Reception of 12 PRB PBCH based on RB-level puncturing;

    - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS;

    - Reception of 15 PRB CORESET0;

Issue 1-4: Clarification on asymmetric band combination with 3MHz UL and X MHz DL

- Agreement

  - Asymmetric band combinations with the 3MHz UL channel include the 5MHz DL and larger (applicable) DL channel bandwidths.

In TN 3MHz, three UE capabilities were introduced, that is support12PRB-CORESET0-r18, support3MHz-ChannelBW-Symmetric-r18, support3MHz-ChannelBW-Asymmetric-r18. We can expand the applicability of these TN capabilities to NTN.

| support12PRB-CORESET0-r18 Indicates whether the UE supports reception of 12 PRB CORESET0 with an associated SS/PBCH block that is located according to Table 5.4.3.1-2 in TS 38.101-1 [2]. A UE supporting this feature shall also indicate support of support3MHz-ChannelBW-Symmetric-r18. This feature is supported for 15kHz SCS only. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 12 PRB BWP operation. | Band | No | FDD only | FR1 only |
| --- | --- | --- | --- | --- |
| support3MHz-ChannelBW-Asymmetric-r18 Indicates whether the UE supports 3 MHz channel bandwidth in uplink with larger than 3 MHz channel BW in DL, including short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS. This feature is supported for 15kHz SCS only. It applies to bands where the UE indicates support for asymmetricBandwidthCombinationSet with 3 MHz UL according to clause 5.3.6 of TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE 1: The UE supporting this feature supports configuration of 15 PRB UL BWP operation. NOTE 2: If the UE indicates support in asymmetricBandwidthCombinationSet for a 3MHz UL in a band according to clause 5.3.6 of 38.101-1 [2], this feature shall be indicated for the band. | Band | No | FDD only | FR1 only |
| support3MHz-ChannelBW-Symmetric-r18 Indicates whether the UE supports 3 MHz symmetric channel bandwidth in DL and UL, including the following functional components: - Reception of 12 PRB PBCH based on RB-level puncturing; - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; - Reception of 15 PRB CORESET0. This feature is supported for 15kHz SCS only. It is applicable when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 15 PRB BWP operation in DL and UL. | Band | No | FDD only | FR1 only |

For symmetric channel bandwidth with 3MHz, it can be supported for FR1-NTN. For asymmetric channel bandwidth, there was clear request for 3MHz UL with 5MHz, 10MHz, 15MHz DL for band n254. These asymmetric bandwidths need to be captured in the specification. From our perspective, the UE capabilities support3MHz-ChannelBW-Symmetric-r18, support3MHz-ChannelBW-Asymmetric-r18 can be expanded to FR1-NTN.

Proposal 2: Expand the applicability of TN capabilities support12PRB-CORESET0-r18, support3MHz-ChannelBW-Symmetric-r18, support3MHz-ChannelBW-Asymmetric-r18 for 3MHz to FR1-NTN.

Conclusion

This contribution gives our views on system parameters for FR1-NTN supporting less than 5MHz channel bandwidth. The following proposals are made:

Proposal 1: Channel bandwidth 3MHz for FR1-NTN can be an optional feature in Rel-19 unless there is strong request from Operator to make this feature mandatory.

Proposal 2: Expand the applicability of TN capabilities support12PRB-CORESET0-r18, support3MHz-ChannelBW-Symmetric-r18, support3MHz-ChannelBW-Asymmetric-r18 for 3MHz to FR1-NTN.

- References

[1] R4-2413528, WF on for [112][312] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi, RAN4#112.

[2] R4-2416587, Way Forward for [112bis][311] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi, RAN4#112bis.

[3] R4-2419789, Way Forward for [113][311] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi, RAN4#113.