# 3GPP TSG-RAN WG4 Meeting #114	R4-2500736

# Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Discussion on UE RF requirements for less than 5MHz in FR1-NTN bands

Agenda Item:	7.10.3.2

Document for:	Approval

Introduction

In the last meeting, we further discussed UE RF requirements for less than 5MHz in FR1-NTN bands. All the open issues for RF requirements are captured in [1]. In this contribution, we will discuss the UE RF requirements for less than 5MHz in FR1-NTN bands.

Discussion

The remaining issues for less than 5MHz for NTN are captured in WF [1].

Rx requirements

For NR-NTN UE Rx requirements, one work is to define 3MHz REFSENS requirements for bands n256, n255, n254. In the last meeting, we had the following WF:

Issue 2-2-1: Reference sensitivity

- Option 1:

  - 2.2dB improvement compared with 5 MHz CBW;

- Option 2:

  - 1.7dB improvement compared with 5 MHz CBW;

- Encourage companies to consider Rx non-linearity in the reference sensitivity evaluations

For 3MHz REFSENS, the transmission bandwidth is 15RB with 15kHz if we follow the NRB number from TN. If we only consider the BW scaling, the REFSENS for 3MHz is 2.2dB improvement compared with 5MHz. However, more factor needs to be considered other than BW. Rx non-linearity can be a factor influencing REFSENS for 3MHz. Observed from LTE bands, both 2.2 dB and 1.7dB improvement are adopted for different bands. For band n256, n255 n254, we can follow LTE band 65 or 66, since they shared similar frequency range. 1.7dB improvement was adopted for 3MHz for these bands. We can adopt the same value for 3MHz in FR1-NTN bands. Based on these, we calculate the REFSENS for 3MHz as follows:

| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
|  | 15 | -101.2 | -99.5 | -96.3 | -94.5 | -93.8 |
| n256 | 30 |  |  | -96.6 | -94.6 | -94.0 |
|  | 60 |  |  | -97.0 | -94.9 | -94.2 |
|  | 15 | -101.7 | -100.0 | -96.8 | -95.0 | -93.8 |
| n255 | 30 |  |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -101.2 | -99.5 | -96.3 | -94.5 |  |
| n254 | 30 |  |  | -96.6 | -94.6 |  |
|  | 60 |  |  | -97.0 | -94.9 |  |

For the Uplink configuration for reference sensitivity, we need to add 3MHz as follows:

Table 7.3.2-2: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | Duplex Mode |
|  | 15 | 15 | 25 | 50 | 75 | 100 |  |
| n256 | 30 |  |  | 24 | 36 | 50 | FDD |
|  | 60 |  |  | 10 | 18 | 24 |  |
|  | 15 | 15 | 25 | 50 | 75 | [75] |  |
| n255 | 30 |  |  | 24 | 36 | [36] | FDD |
|  | 60 |  |  | 10 | 18 | [18] |  |
|  | 15 | 15 | 25 | 50 | 75 |  |  |
| n254 | 30 |  |  | 24 | 36 |  | FDD |
|  | 60 |  |  | 10 | 18 |  |  |

For other Rx requirements (Maximum input level, ACS, blocking…), we suggest to extend them to support 3MHz since there were no big issues.

Conclusion

This contribution discusses NR-NTN UE RF requirements for less than 5MHz. The following proposal is made:

Proposal 1: For UE Rx requirements for 3MHz,

- Define 3MHz REFSENS requirements for bands n256, n255, n254

| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
|  | 15 | -101.2 | -99.5 | -96.3 | -94.5 | -93.8 |
| n256 | 30 |  |  | -96.6 | -94.6 | -94.0 |
|  | 60 |  |  | -97.0 | -94.9 | -94.2 |
|  | 15 | -101.7 | -100.0 | -96.8 | -95.0 | -93.8 |
| n255 | 30 |  |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -101.2 | -99.5 | -96.3 | -94.5 |  |
| n254 | 30 |  |  | -96.6 | -94.6 |  |
|  | 60 |  |  | -97.0 | -94.9 |  |

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | Duplex Mode |
|  | 15 | 15 | 25 | 50 | 75 | 100 |  |
| n256 | 30 |  |  | 24 | 36 | 50 | FDD |
|  | 60 |  |  | 10 | 18 | 24 |  |
|  | 15 | 15 | 25 | 50 | 75 | [75] |  |
| n255 | 30 |  |  | 24 | 36 | [36] | FDD |
|  | 60 |  |  | 10 | 18 | [18] |  |
|  | 15 | 15 | 25 | 50 | 75 |  |  |
| n254 | 30 |  |  | 24 | 36 |  | FDD |
|  | 60 |  |  | 10 | 18 |  |  |

- For other Rx requirements (Maximum input level, ACS, blocking…), extend them to support 3MHz

- References

[1] R4-2416587, Way Forward for [112bis][311] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi, RAN4#112bis.

[2] R4-2419789, Way Forward for [113][311] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi, RAN4#113.