# 3GPP TSG-RAN WG4 Meeting #114	R4-2500739

Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Discussion on UE capabilities for FR2-NTN

Agenda Item:	8.2

Document for:	Approval

Introduction

An incoming LS [1] was received for Rel-18 NR_NTN_enh-Core. RAN2 asked RAN4 if UE capabilities for 200MHz channel bandwidth and 2 MIMO layers are mandatory for FR2-NTN bands. In this contribution, we tried to give our views on this issue and provided reply LS to RAN2.

Discussion

The content of the incoming LS [1] from RAN2 was captured as follows:

# 1. Overall Description: At RAN2#128 it has been discussed UE capabilities for FR2-NTN bands. RAN2 would like to check with RAN4 whether UE capabilities for 200Mhz channel bandwidth and 2 MIMO layers are mandatory for FR2-NTN bands. # 2. Actions: To RAN4 group: RAN2 respectfully asks for RAN4 feedback on whether UE capabilities for 200Mhz channel bandwidth and 2 MIMO layers are mandatory for FR2-NTN bands.

In TN FR2, if a channel bandwidth is optional, it is clearly stated in the specification. In TS 38.101-2, the channel bandwidth for 400 MHz is optional which can be seen from the note 1 in Table 5.3.5-1. For other channel bandwidths 50M, 100M, 200M, they are mandatory for FR2-1 bands.

### 5.3.5 Channel bandwidth per operating band The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1. The transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths. The channel bandwidths are specified for both the Tx and Rx path. Table 5.3.5-1: Channel bandwidths for each NR band Operating band; SCS (kHz); UE channel bandwidth (MHz). 50; 100; 200; 400; 800; 1600; 2000. n257; 60; 50; 100; 200. 120; 50; 100; 200; 4001. n258; 60; 50; 100; 200. 120; 50; 100; 200; 4001. n259; 60; 50; 100; 200. 120; 50; 100; 200; 4001. n260; 60; 50; 100; 200. 120; 50; 100; 200; 4001. n261; 60; 50; 100; 200. 120; 50; 100; 200; 4001. n262; 60; 50; 100; 200. 120; 50; 100; 200; 4001. n263; 120; 100; 400. 4802; 400; 8001; 16001. 9602; 400; 8001; 16001; 20001. NOTE 1: This UE channel bandwidth is optional in this release of the specification. NOTE 2: This SCS is optional in this release of the specification..

For FR2-NTN bands, 200MHz is mandatory interpreted from Table 5.3.5 in TS 38.101-5. There is no clear note to say 200MHz is optional then it is mandatory. All the transmission bandwidth configuration in Table 5.3.5-2 shall be supported for each specified channel bandwidths. We suggest to reply RAN2 that 200MHz in mandatory for FR2-NTN.

### 5.3.5 UE channel bandwidth per operating band The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1 for FR1-NTN and table 5.3.5-2 for FR2-NTN. The transmission bandwidth configuration in Table 5.3.2-1 and Table 5.3.2-2 shall be supported for each of the specified channel bandwidths. The channel bandwidths are specified for both the Tx and Rx path. <Table 5.3.5-1 omitted> Table 5.3.5-2: Channel bandwidths for each NTN satellite band in FR2-NTN SAN Operating Band; SCS (kHz); SAN channel bandwidth (MHz). 50; 100; 200; 400. n512; 60; 50; 100; 200. 120; 50; 100; 200; 400. n511; 60; 50; 100; 200. 120; 50; 100; 200; 400. n510; 60; 50; 100; 200. 120; 50; 100; 200; 400.

Proposal 1: For FR2-NTN, 200MHz is mandatory from current specification TS 38.101-5.

RAN2 also asked whether UE capability for 2MIMO layers is mandatory for FR2-NTN. We need to discuss both DL and UL 2 MIMO layer for FR2-NTN.

In TN FR2-1, UL MIMO is supported under Suffix D in this specification. The RF requirements for UL MIMO are complete. 2 MIMO layer is supported for FR2-1.

- 6.2D Transmitter power for UL MIMO - 6.2D.1 UE maximum output power for UL MIMO #### 6.2D.1.0 General The requirements in the following clauses define the maximum output power radiated by the UE with nrofSRS-Ports set to 2, for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. MPR shall be applied as specified in clause 6.2D.2 For the maximum output power requirement for 2-layer UL MIMO operation, a UE shall be configured for 2-layer UL MIMO transmission as specified in Table 6.2D.1.0-1. Table 6.2D.1.0-1: UL MIMO configuration Transmission scheme; DCI format; Number of layers; TPMI index. Codebook based uplink; DCI format 0_1; 2; 0. The maximum output power requirement for single layer transmission shall apply to a UE that supports ULFPTx feature and is configured for single layer transmission in its declared full power mode [10, TS 38.213] as specified in Table 6.2D.1.0-2. Table 6.2D.1.0-2: PUSCH Configuration for uplink full power transmission (ULFPTx) ULFPTx Mode; Transmission scheme; DCI format; Modulation; Number of layers; TPMI index. Mode-1; Codebook based uplink; DCI format 0_1; DFT-s-OFDM, CP-OFDM 1; 1; 2. Mode-2; Codebook based uplink; DCI format 0_1; DFT-s-OFDM, CP-OFDM; 1; 0 or 12. Mode-full power; Codebook based uplink; DCI format 0_1; DFT-s-OFDM, CP-OFDM; 1; 0,1. NOTE 1: For PUSCH configured with ULFPTxModes set to Mode-1, all requirements for 1-layer CP-OFDM based modulation in subsection 6.2D are assumed to be met if the requirement for 2-layer UL MIMO has been validated. NOTE 2: TPMI index selected shall be based upon the full power TPMI reported by the UE [10, TS 38.213]..

However, for FR2-NTN, there was not any RF requirements for UL MIMO in TS 38.101-5, not to say 2 MIMO layers. From our perspective, 2 UL MIMO layers is not supported for FR2-NTN. There will be standard work to be done if 2 MIMO layer is to be supported. If it is not supported, we don’t need to further discuss whether it is mandatory or optional.

For 2 DL MIMO layers, it is mandatory supported for TN FR2 according to the description of maxNumberMIMO-layersPDSCH. However, we think it is not supported for FR2-NTN. In current specification 38.101-5, reference measurement channels for PDSCH, only 1 MIMO layer is supported for FR2-NTN. In RAN4, we didn’t discuss the support of 2 MIMO layer PDSCH. Thus, we suggest to reply RAN2 that 2 DL MIMO layer is not supported.

Table A.3.2.1.3-4: PDSCH Reference Channel for FDD (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.3-4.1 FDD |  |  |  |  |
| Channel bandwidth | MHz | 200 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Number of allocated resource blocks | PRBs | 132 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 13 |  |  |  |  |
| Allocated slots per 2 frames | Slots | 159 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  | 12 |  |  |  |  |
| Other parts omitted |  |  |  |  |  |  |

Proposal 2: For FR2-NTN, 2 UL/DL MIMO layers is not supported from current specification TS 38.101-5.

Conclusion

This contribution discusses incoming LS [1] for FR2-NTN capabilities of 200MHz and 2 MIMO layers support. The following proposals are made:

Proposal 1: For FR2-NTN, 200MHz is mandatory from current specification TS 38.101-5.

Proposal 2: For FR2-NTN, 2 UL/DL MIMO layers is not supported from current specification TS 38.101-5.

- References

[1] R2-2411195, LS on UE capabilities for FR2-NTN, RAN2#128.