3GPP TSG-RAN WG4 Meeting # 114 	R4-2500742

Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Discussion on co-existence of NTN HPUE

Agenda Item:	7.10.2.1

Document for:	Approval

- Introduction

In the past meetings, the co-existence between NR and NTN HPUE was fully evaluated by companies and in this contribution, we provide our analysis on the final ACLR value for NTN HPUE.

- Discussion

  - IoT NTN

Issue 1-7: Relationship between ACLR requirements and co-existence study for IoT-NTN

- Proposals

  - Option 1: to clarify the relationship between the ACLR requirements for NB-IoT NTN UE and IoT NTN coexistence study since these two parts are not related during the Rel-18. (P1/R4-2419071)

  - Option 2: For co-ex study conclusion, to clarify and align understanding that derived ACLR from agreed ACIR is an equivalent ACLR in victim bandwidth. (P9/R4-2419642)

Figure 4-1: The ACLR models and its reference bandwidth

In [1], the issue about conversion between ACIR and ACLR was arisen.

Recall that in the simulation assumption [2], the aggressor (NTN) bandwidth is 15kHz/3.75kHz for each UE and victim (TN) bandwidth is 20MHz with 3 UEs. Based on the assumption, we can easily calculate the difference between the modelling methods above.

For 15kHz:

Pinf = (Pagg-X) dBm +(Pagg-(X+13) +10*log10(TN UE BW/15kHz -1)) dBm

= (Pagg-X) dBm +(Pagg-X+2.3) dBm

=> ACLR = ACIR + 4.3 dB

For 3.75kHz:

Pinf = (Pagg-X) dBm +(Pagg-(X+13) +10*log10(TN UE BW/3.75kHz -1)) dBm

= (Pagg-X) dBm +(Pagg-X+8.5) dBm

=> ACLR = ACIR + 9.1 dB

Observation 1: The maximum difference between ACLR and ACIR in IoT NTN is 4.3 dB for 15kHz and 9.1dB for 3.75kHz.

In [3], the observation for the required ACLR/ACIR is recorded, and the maximum ACLR/ACIR is 20.3 dB for PC2 and 24.2 dB for PC1 under the worst case without isolation. However, current ACLR for PC3 NB-IoT is already defined as 37dBc, which means although we take the difference of model above into consideration, there is still sufficient margin for legacy requirement, and the conversion seems to be unnecessary.

Observation 2: For IoT NTN co-existence study, the conversion between ACLR and ACIR is not necessary if current 37dBc ACLR is reused.

Another related issue here is the definition of ACLR. In TS 36.102, the ACLR of NB-IoT is defined as follows；

|  | GSMACLR | UTRAACLR |
| --- | --- | --- |
| ACLR | 20 dB | 37 dB |
| Adjacent channel center frequency offset from category NB1 or NB2 Channel edge | ±200 kHz | ±2.5 MHz |
| Adjacent channel measurement bandwidth | 180 kHz | 3.84 MHz |
| Measurement filter | Rectangular | RRC-filter α=0.22 |
| Category NB1 and NB2 channel measurement bandwidth | 180 kHz | 180 kHz |
| Category NB1 and NB2 channel Measurement filter | Rectangular | Rectangular |

The adjacent channel bandwidth is 5MHz which is not align with the current simulation assumption, but smaller bandwidth has no impact on the ACIR based model since the interference is exactly same in the simulation. For ACLR based model, the interference become lower since the integral interval is smaller. In all, current evaluation is sufficient for ACLR under same definition as current NB-IoT.

Observation 3: Current evaluation assumption is worse than the NB-IoT ACLR definition, which can provide sufficient insurance for co-existence by using same definition.

Based on the analysis above, we propose:

Proposal 1: For PC1/PC2 IoT NTN, using same ACLR definition as current PC3, i.e., adjacent channel center frequency offset is ±2.5MHz and adjacent channel measurement bandwidth is 3.84MHz.

Proposal 2: For PC1/PC2 IoT NTN, reusing same ACLR value as current PC3, i.e., 37 dBc

  - NR NTN

For NR NTN, the observation for the evaluation are as follows:

For NR-NTN PC2:

- Less than 22 dB with 1.5km isolation distance assumed (baseline assumption)

- [26.5 dB] with 0km isolation distance assumed.

  - Interested companies are encouraged to provide results on this case

For NR-NTN PC1.5:

- Less than 22.9 dB with 1.5km isolation distance assumed (baseline assumption)

- [30.2] dB with 0km isolation distance assumed.

  - Interested companies are encouraged to provide results on this case

For NR-NTN PC1:

- Less than 23.7 dB with 1.5km isolation distance assumed (baseline assumption)

- [28.3/32.7] dB with 0km isolation distance assumed.

  - Interested companies are encouraged to provide results on this case

The most controversial discussion here is whether the ACLR should be derived with isolation distance. In our understanding, the isolation distance is a baseline assumption for the co-existence feasibility in legacy PC3 and it quite meaningless to remove this in HPUE, since without isolation distance, it will be doubtful whether the legacy NTN system can co-exist with TN.

Observation 4: The feasibility of co-existence for legacy PC3 NTN is based on assuming that isolation exists, and it is meaningless to remove such assumption for NTN HPUE.

Proposal 3: The ACLR of NTN HPUE should be derived based on the assumption that isolation distance exists.

With the 1.5km isolation, current 30dBc PC3 ACLR is already sufficient.

Proposal 4: For PC1/PC1.5/PC2 NTN HPUE, reuse PC3 ACLR, i.e., 30 dBc.

- Conclusion

In this contribution, we provide our view on the finalization of ACLR discussion.

Observation 1: The maximum difference between ACLR and ACIR in IoT NTN is 4.3 dB for 15kHz and 9.1dB for 3.75kHz.

Observation 2: For IoT NTN co-existence study, the conversion between ACLR and ACIR is not necessary if current 37dBc ACLR is reused.

Observation 3: Current evaluation assumption is worse than the NB-IoT ACLR definition, which can provide sufficient insurance for co-existence by using same definition.

Proposal 1: For PC1/PC2 IoT NTN, using same ACLR definition as current PC3, i.e., adjacent channel center frequency offset is ±2.5MHz and adjacent channel measurement bandwidth is 3.84MHz.

Proposal 2: For PC1/PC2 IoT NTN, reusing same ACLR value as current PC3, i.e., 37 dBc.

Observation 4: The feasibility of co-existence for legacy PC3 NTN is based on assuming that isolation exists, and it is meaningless to remove such assumption for NTN HPUE.

Proposal 3: The ACLR of NTN HPUE should be derived based on the assumption that isolation distance exists.

Proposal 4: For PC1/PC1.5/PC2 NTN HPUE, reuse PC3 ACLR, i.e., 30 dBc.

- References

- R4-2418148, Topic summary for [113][126] NR_IoT_NTN_HPUE_part1, Moderator (Samsung)

- R4-2414276, Simulation assumption for co-existence study, Samsung

- R4-2420403, WF on NR NTN HPUE, Samsung