3GPP TSG-RAN WG4 Meeting #114		R4-2500751
Athens, GR, 17th – 21st February, 2025

Agenda item:	7.14.2.2

Source:	vivo

Title:	Discussions on NTN OTA test method

Document for:	Approval

1	Introduction

This contribution provides our further views on NTN OTA test method and performance metric.

2	Discussion

RAN4 made good progress on usage scenarios and performance metric of NR-NTN last meeting. There are still some open issues for UE support only scenario 2 and non-handheld UEs [1]:

Issue 3-1-3: Handheld IoT-NTN and NR-NTN performance metric

Agreements:

  - For IoT-NTN and NR-NTN handheld UEs supporting Scenario 1 the performance metrics are full sphere TRP and TRS

  - For IoT-NTN and NR-NTN handheld UEs supporting Scenario 2 only, further study performance metric

Issue 3-1-4: non-Handheld IoT-NTN performance metric

Agreements:

  - FFS on partial sphere metrics, such as the radiated power and receive sensitivity within a certain angular range, should be performance metric for non-handheld IoT NTN devices.

  - This can be considered as a package with issue 3-1-3.

For NTN handheld UEs supporting only scenario 2, the typical usage scenario would be application-assisted beam tracking to satellite, the UE is only working under a specific small range of angel. In that sense it is reasonable to define a partial sphere performance metric to better reflect the performance of this UE type. Upper Hemisphere metric with/without peak EIRP value can be considered.

Proposal 1: For NTN handheld only support Scenario 2, the performance metric can be specified as Upper Hemisphere metric with/without peak EIRP value.

In addition, another key issue is how to design the test system to support accurate NTN testing [1].

Issue 3-2-4: How to resolve UE antenna circular polarization issue

Proposals

  - Proposal 1: Use circular polarisation test antennas to assess the performance of the UE. This enables us to characterise the exact loss seen by the UE when operating in a system that utilises circular polarisation antennas, as all satcom systems do.

    - If we ignore circular polarisation testing at this stage, it may preclude the ability of an operator to leverage XPD to increase the capacity of their system.

  - Proposal 2: The NR FR1 NTN conformance testing should focus on LP rather than CP assessments.

- WF

  - FFS

To support CP based OTA testing, current test system needs hardware update and there might be some challenges, e.g.,

- The existing multi-probe system may not be upgraded. if we focus on single antenna chamber for NTN testing, then it would result in relatively long testing time

- The lack of a circularly polarized reference antenna with a perfect axial ratio for test system calibration may lead to large measurement errors, which gain outweighs the loss

- Manual switching between linear and circularly polarized measurement antennas is required to accommodate different AUT characteristics, and the system must be recalibrated after switching antennas, this would increase much higher testing burden

- The bandwidth of circularly polarized antennas is typically narrow bandwidth, then it would require multiple antennas in the chamber to cover a wide frequency range NTN testing

- The gain of low-frequency circularly polarized antennas is too low, which would make them unsuitable for use as OTA measurement antennas

Consider above challenges to support CP-based NTN testing, we suggest focus on reusing current LP system as a baseline and further study the statistical UE performance impacts as a MU contributor of NTN OTA system.

Proposal 2: Consider current LP-based OTA test system as a baseline for NTN OTA testing.

Proposal 3: Further study the statistical analysis of UE performance difference between LP vs CP testing, this aspect can be considered as MU contributor of the NTN OTA system.

3 Conclusion

In this paper, we provide our further views on NTN OTA.

Proposal 1: For NTN handheld only support Scenario 2, the performance metric can be specified as Upper Hemisphere metric with/without peak EIRP value.

Proposal 2: Consider current LP-based OTA test system as a baseline for NTN OTA testing.

Proposal 3: Further study the statistical analysis of UE performance difference between LP vs CP testing, this aspect can be considered as MU contributor of the NTN OTA system.

4	References

- R4-2419808, WF for [113][325] TRP_TRS_Ph3, vivo, RAN4#113