3GPP TSG-RAN WG4 Meeting #114	R4-2500756

Athens, GR, 17th – 21st February, 2025

Agenda item:	 7.26.2.1

Source:	 vivo

Title:	Discussions on LP-WUS system parameters

Document for:	 Approval

- Introduction

Issue 1-1-4: views on which requirements should be updated in core spec

Agreements:

  - The part should be updated for LP-WUS/WUR in spec.

    - The additional requirements are not precluded, if needed.

| Sections or requirements | Needed or not? |
| --- | --- |
| Operation bands | FFS |
| UE channel bandwidth | FFS |
| Channel raster | FFS |
| REFSENS | YES |
| Maximum input level | YES |
| ACS | YES |
| ASCS | YES |
| In-band blocking | YES |
| Out-of-band blocking | YES |
| Narrow band blocking | YES |
| Spurious response | FFS |
| Wide band Intermodulation | FFS |
| Spurious emissions | FFS |
| Annex A Measurement channels | YES |

In RAN4#113 meeting, the following agreements were reached [1]:

For operating bands and UE channel bandwidth, it was not concluded whether spec update is needed or not. This contribution shares our views on how to edit LP-WUS general part.

- LP-WUS general parameters

2.1 Operating bands

It was agreed that LP-WUS is a general feature which could apply to all FR1 bands, in WF [2]:

Issue 1-1-1: Operation bands for LP-WUS feature

Agreements:

RAN4 confirm LP-WUS is a general feature not limited to specific example band(s)

Issue 1-1-2: FR1 example bands for requirements as phase 1

Agreements:

No need to list specific example band(s)

Besides, RAN1 is developing the UE feature of LP-WUS [3], and potentially OOK-based or OFDM-based receiver is a per-band capability for LP-WUS reception. In that sense there would be no need to limit operating bands in RAN4.

Proposal 1: Conclude that update of operation band clause for LP-WUS is not needed.

2.2 Channel bandwidth

RAN1 has specified 11 RBs for LP-WUS with 15kHz or 30kHz SCS, then the bandwidth of LP-WUS signal is a fixed value under a specific SCS configuration. To guide LP-WUR design and RF implementation, it is valuable to clearly state in RAN4 spec that the LP-WUR is 11RBs.

Therefore, we suggest stating clearly the number of RBs for LP-WUR operation in RAN4 RF spec.

Proposal 2: Add 5.3M clause to describe the number of RBs for LP-WUS/WUR operation.

## 5.3M	Channel bandwidth for LP-WUS/WUR

The number of RBs for LP-WUS/WUR operation is fixed as 11RBs with 15kHz/30kHz SCS.

The draft CR for LP-WUS RF is prepared in [4].

- Conclusion

In this contribution, we share our views on LP-WUR system parameters:

Proposal 1: Conclude that update of operation band clause for LP-WUS is not needed.

Proposal 2: Add 5.3M clause to describe the number of RBs for LP-WUS/WUR operation.

# References

- R4-2420528, WF on UE RF requirements for LP-WUS, vivo, RAN4#113

- R4-2410569, WF for [111][136] NR_LPWUS_UERF, vivo, RAN4#111

- R1-2500375, Rapporteur input on UE features for Rel-19 LP-WUS/WUR, vivo, DOCOMO, RAN1 #120

- R4-2500758, Draft CR to 38.101-1 on LP-WUS RF, vivo, RAN4#114