3GPP TSG-RAN4 Meeting #114 R4-2500758 Athens, GR, 17th – 21st February, 2025 CR-Form-v12.3. CHANGE REQUEST. 38.101-1; CR; draft; rev; -; Current version:; 19.0.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; Core Network. Title:; Draft CR to 38.101-1 on LP-WUS RF. Source to WG:; vivo. Source to TSG:; R4. Work item code:; NR_LPWUS-Core; Date:; 2025-02-05. Category:; B; Release:; Rel-19. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20). Reason for change:; This CR provides content to accommodate LP-WUS/WUR RF requirements.. Summary of change:; The following changes: - Abbreviations - Suffix - Channel bandwidth - Rx general - REFSENS - Max input power - ACS/ASCS - Blocking - Spurious emissions. Consequences if not approved:; The spec is not appicable for LP-WUS feature.. Clauses affected:; 3.3, 4.3, 5.3, 7.1, 7.3, 7.4, 7.5, 7.6, 7.9. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:.

<<< Skip Unchanged Sections >>>

## <<< START OF CHANGES >>>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

ASCS	Adjacent Subcarrier selectivity

ATG	Air-To-Ground

BS	Base Station

BW	Bandwidth

BWP	Bandwidth Part

CA	Carrier Aggregation

CA_nX-nY	Inter-band CA of component carrier(s) in one sub-block within Band nX and component carrier(s) in one sub-block within Band nY where nX and nY are the applicable NR operating bands

CC	Component Carriers

CG	Carrier Group

CP-OFDM	Cyclic Prefix-OFDM

CW	Continuous Wave

DC	Dual Connectivity

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DM-RS	Demodulation Reference Signal

DTX	Discontinuous Transmission

E-UTRA	Evolved UTRA

EIRP	Equivalent Isotropically Radiated Power

(e)RedCap	Redcap or eRedCap

eRedCap	enhanced Reduced Capability

EVM	Error Vector Magnitude

FAR	False Alarm Rate

FR	Frequency Range

FRC	Fixed Reference Channel

FRMCS	Future Railway Mobile Communication System

FWA	Fixed Wireless Access

GSCN	Global Synchronization Channel Number

HD	Half Duplex

IBB	In-band Blocking

IDFT	Inverse Discrete Fourier Transformation

ITS	Intelligent Transportation System

ITU-R	Radiocommunication Sector of the International Telecommunication Union

LP-WUS	Low Power-Wake Up Signal

LP-WUR	Low Power-Wake Up Receiver

LP-SS	Low Power- Synchronization Signal

LR	LP-WUR

MBW	Measurement bandwidth

MCG	Master Cell Group

MDR	Miss Detection Rate

MOP	Maximum Output Power

MPR	Allowed maximum power reduction

MR	Main Radio

MSD	Maximum Sensitivity Degradation

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

NS	Network Signalling

OCNG	OFDMA Channel Noise Generator

OOB	Out-of-band

OOK	On-Off keying

P-MPR	Power Management Maximum Power Reduction

PRB	Physical Resource Block

PS	Public Safety

PSBCH	Physical Sidelink Broadcast CHannel

PSCCH	Physical Sidelink Control CHannel

PSFCH	Physical Sidelink Feedback CHannel

PSSCH	Physical Sidelink Shared CHannel

QAM	Quadrature Amplitude Modulation

RE	Resource Element

REFSENS	Reference Sensitivity

RedCap	Reduced Capability

RF	Radio Frequency

RMR	Railway Mobile Radio

RMS	Root Mean Square (value)

RSRP	Reference Signal Receiving PowerRx	Receiver

Rx	Receiver

SC	Single Carrier

SCG	Secondary Cell Group

SCS	Subcarrier spacing

SDL	Supplementary Downlink

SEM	Spectrum Emission Mask

SL	Sidelink

SL-MIMO	Sidelink-Multiple Antenna transmission

SL-U	Sidelink at unlicensed band

SNR	Signal-to-Noise Ratio

SRS	Sounding Reference Symbol

SS	Synchronization Symbol

S-SSB	Sidelink Synchronization Signal Block

SUL	Supplementary uplink

TAB	Transceiver Array Boundary

TAE	Time Alignment Error

TAG	Timing Advance Group

Tx	Transmitter

TxD	Tx Diversity

UAS	Uncrewed Aircraft Systems

UAV	Uncrewed Aerial Vehicle

UL MIMO	Uplink Multiple Antenna transmission

ULFPTx	Uplink Full Power Transmission

USS	UAS Service Supplier

V2X	Vehicle to Everything

XR	eXtended Reality

## <<< END OF CHANGES >>>

<<< Skip Unchanged Sections >>>

## <<< START OF CHANGES >>>

4.3	Specification suffix information

Unless stated otherwise, the suffixes shown in Table 4.3-1 are used for indicating at 2nd level clause. For shared spectrum channel access, suffixes A, B, and D are used for indicating at 3rd level clause. For V2X, suffixes A and F are used for indicating at 3rd level clause.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplementary Uplink (SUL) |
| D | UL MIMO |
| E | V2X |
| F | Shared spectrum channel access |
| G | Tx Diversity (TxD) |
| H | Carrier Aggregation (CA) with UL MIMO |
| I | (e)RedCap |
| J | ATG |
| K | Aerial UE (UAV) |
| L | Carrier Aggregation (CA) with Tx Diversity |
| M | LP-WUS/WUR |

## <<< END OF CHANGES >>>

<<< Skip Unchanged Sections >>>

## <<< START OF CHANGES >>>

5.3M	Channel bandwidth for LP-WUS/WUR

The number of RBs for LP-WUS/WUR operation is fixed as 11RBs with 15kHz/30kHz SCS.

## <<< END OF CHANGES >>>

<<< Skip Unchanged Sections >>>

## <<< START OF CHANGES >>>

7.1M	General

Unless otherwise stated the LP-WUS/WUR receiver characteristics are specified at the antenna connector of the UE. UE with a reference antenna with a gain of 0 dBi is assumed as baseline.

## <<< END OF CHANGES >>>

<<< Skip Unchanged Sections >>>

## <<< START OF CHANGES >>>

7.3M	Reference sensitivity for LP-WUS/WUR

7.3M.1	General

For UE support LP-WUS/WUR, the reference sensitivity power level REFSENS for LP-WUR is the minimum mean power applied to single UE antenna port, at which the Miss Detection Rate (MDR) shall meet the requirements for the specified reference measurement channel.

7.3M.2	Reference sensitivity power level

For a UE support LP-WUS, the Miss Detection Rate (MDR) shall be ≤ 1 % with the reference measurement channels as specified in Annexes TBD.

For an OOK-based LP-WUR UE equipped with 1 Rx antenna port, 10MHz reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔR1R in Table 7.3M.2-1 for LP-WUR at the applicable operating bands.

Table 7.3M.2-1: OOK-based LP-WUR Single antenna port reference sensitivity allowance ΔR1R

| Operating band | LP-WUS bandwidth (RB) | ΔR1R (dB) of 10MHz REFSENS |
| --- | --- | --- |
| TDD band | 11 | TBD |
| FDD band | 11 | TBD |

For an OFDM-based LP-WUR UE equipped with 1 Rx antenna port, 10MHz reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔR1R in Table 7.3M.2-2 for LP-WUR at the applicable operating bands.

Table 7.3M.2-2: OFDM-based LP-WUR Single antenna port reference sensitivity allowance ΔR1R

| Operating band | LP-WUS bandwidth (RB) | ΔR1R (dB) of 10MHz REFSENS |
| --- | --- | --- |
| TDD band | 11 | TBD |
| FDD band | 11 | TBD |

## <<< END OF CHANGES >>>

<<< Skip Unchanged Sections >>>

## <<< START OF CHANGES >>>

7.4M	Maximum input level for LP-WUS/WUR

LP-WUS/WUR maximum input level is defined as the maximum mean power received at single UE antenna port. The Miss Detection Rate (MDR) for LP-WUS shall be ≤ 1 % with the reference measurement channels as specified in Annexes TBD., with parameters as specified in Table 7.4M-1.

Table 7.4M-1: Maximum input level

| Rx Parameter | Units | LP-WUS bandwidth (RB) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth configuration | dBm | [-25] |

## <<< END OF CHANGES >>>

<<< Skip Unchanged Sections >>>

## <<< START OF CHANGES >>>

7.5M	Adjacent channel and subcarrier selectivity for LP-WUS/WUR

7.5M.1	General

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the configured channel frequency to the receive filter attenuation on the adjacent channel(s).

Adjacent subcarrier selectivity (ASCS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured channel frequency in the presence of adjacent in-band NR signal(s) at a given frequency offset (guard RB) between LP-WUS and NR.

7.5M.2	Adjacent channel selectivity for LP-WUS/WUR

The UE shall fulfil the minimum requirements specified in Table 7.5M.2-1 for LP-WUR. These requirements apply for all values of an adjacent channel interferer up to -25 dBm and for any SCS specified for the channel bandwidth of the wanted signal. However, it is not possible to directly measure the ACS; instead the lower and upper range of test parameters are chosen as in Table 7.5M.2-3 for verification of the requirements specified in Table 7.5M.2-1 for LP-WUR. For these test parameters, the Miss Detection Rate (MDR) for LP-WUS shall be ≤ 1 % with the reference measurement channels as specified in Annexes TBD.

Table 7.5M.2-1: ACS for LP-WUR type 1 UE

Table 7.5M.2-2: Test parameters for LP-WUR, case 1

Table 7.5M.2-3: Test parameters for LP-WUR, case 1

7.5M.3	Adjacent subcarrier selectivity for LP-WUR

The UE shall fulfil the minimum requirements specified in Table 7.5M.3-1 for LP-WUR. For these test parameters, the Miss Detection Rate (MDR) for LP-WUS shall be ≤ 1 % with the reference measurement channels as specified in Annexes TBD.

Table 7.5M.3-1: ASCS parameters for LP-WUR

| RX parameter | Units | LP-WUS bandwidth (RB) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth configuration | dBm | REFSENS+TBD |
| Pinterferer,NR | dBm | Same PSD as LP-WUS |
| Guard RB on each side of LP-WUS signal |  | 0 |
| NOTE: The NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration, excluding the LP-WUS RBs. |  |  |

## <<< END OF CHANGES >>>

<<< Skip Unchanged Sections >>>

## <<< START OF CHANGES >>>

7.6M	Blocking characteristics for LP-WUS/WUR

7.6M.1	General

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

7.6M.2	In-band blocking for LP-WUS/WUR

For LP-WUS UE, the in-band blocking requirement is specified as following:

TBD

7.6M.3	Out-of-band blocking for LP-WUS/WUR

For LP-WUS UE, the ou-of-band blocking requirement is specified as following:

TBD

## <<< END OF CHANGES >>>

<<< Skip Unchanged Sections >>>

## <<< START OF CHANGES >>>

7.9M	Spurious emissions for LP-WUS/WUR

For LP-WUS UE, the spurious emissions as specified in clause 7.9 applies.

## <<< END OF CHANGES >>>