3GPP TSG-RAN WG4 Meeting # 114	R4-2500762

Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	NR NTN HPUE Tx Requirements and feasibility analysis

Agenda Item:	7.10.2.2.1

Document for:	Approval

Introduction

In RAN4#113, the summary and WF in NR NTN HPUE requirements is in [1][2]. A comprehensive discussion paper for Tx is in [3]. The are some progresses, but still issues to be resolved.

In this paper, further discussion was made for NR-NTN. It is noted that the duty cycle related discussion is not included here but put to another paper for Iot-NTN Tx.

HPUE Impact for Tx Requirements

Currently the co-existence study is closing to end, that a number of ACLR tentative ranges have been collected, but no final ACLR requirements set yet. This would be treated in the co-existence paper.

MOP

The MOP discussion is mainly focus on the tolerance and following Agreement has been reached in [2]:

Agreement:

Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance for PC2 for NR-NTN Non-Redcap HPUE

Theoretically, the nominal MOP for the same power class can be different for NR NTN and NR, since they are defined in separate specs. However, all the current discussion has assumed the same nominal MOP in NTN as TN, i.e. PC1/1.5/2 is 31/29/26 dBm, and there were not any different views on this point. This might be because no benefit foreseen to have different values, and reusing those basic parameters are also natural from the reusing the TN components.

Observation 1a: No need to consider different nominal MOP for the same power class in NR NTN with TN.

For the tolerance value, currently all the HPUE cases in NR NTN is +2/-3. Considering the target to reuse TN components as much as possible, this tolerance range is also proposed to be re-used.

Observation 1b: Reusing tolerance for different power class in TN for NTN is needed for basic reuse of the components.

All those requirements are not applicable NR-NTN redcap since HPUE is not yet supported in REl-19.

Based these conditions, the following proposal is provided:

Proposal 1: Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance for PC1 and PC1.5 for NR-NTN Non-Redcap HPUE.

SEM

There has been some agreements already for SEM, and the latest one in [2] is as following:

Way forward:

- Follow previous agreement that re-use existing SEM from PC3 to other PCs as starting point.

- Further discuss if SEM can be relaxed for the best power gains for NTN with co-ex study outcomes.

There has not been too much progress in last meeting since the meeting ahead, PC3 SEM is already confirmed as starting point. There are more factors were considered currently.

First point is whether SEM could be relaxed or not. This is similar to the ACLR which is deemed has some room for relaxation. However, we think SEM is not that suitable for relaxation, as it mainly consider regulatory requirements.

The second point is whether to consider the NTN specific regulatory requirements in the general SEM in NR-NTN. Theoretically, this could be a way forward considering NTN has separate specs. However, re-discuss the SEM in TN which had already be used for over a decade since LTE may involves considerable work. In addition, there are already quite complete and effective A-MPR solution to account for various regulatory requirements with even more flexibility compared to SEM, making the discussion of a new SEM not that attractive.

Observation 2: Reconsider NTN regulatory requirement in SEM may involve considerable work, and the current A-MPR solution is already effective and flexible to consider the various regional regulatory requirements.

Based on the condition, the following proposal is provided:

Proposal 2: Re-use existing SEM from PC3 to other PCs for NR-NTN.

ACLR

Previous way is as following:

Way forward:

- Further discuss the ACLR requirements for NR-NTN HPUE

  - Take into account the co-ex study outcomes in this #113 meeting with previous agreements.

This is being discussed under the co-existence agenda and there are dedicated contributions. Apart from co-existence study, there are also other factors would impact ACLR requirement. If further relaxing ACLR compared to PC3 can have further power gain, then it might be worthy to be considered. If not, then the need may not be large.

Generally, this topic can be discussed in the co-existence agenda.

MPR/A-MPR

The previous agreement is:

Agreement:

MPR for NTN HPUE should be further evaluated according to the agreed ACLR value, PA model and other limiting factor for MPR evaluation.

This is a quite general agreement. The MPR/A-MPR is pretty much based on SEM/ACLR, and it is difficult to have concrete progress without defining the preconditions.

Proposal 3: Discuss the MPR/A-MPR after SEM/ACLR is confirmed.

Feasibility and Requirements applicability

Current condition:

In last meeting, no more agreements were made for feasibility or requirements applicability for NR-NTN. The WF [3] has include some table summary in the open issues part.

Currently there are some agreements in previous meetings.

For PC2:

  - For PC2 (NR NTN), 1 Tx and 2 Tx, are to be considered for requirements.

For PC1：

- Confirm feasibility of PC1 1Tx for non-handheld UE.

- For PC1, at least 1Tx to be considered for requirements.

For PC1.5:

- Both 1Tx and 2Tx for PC1.5 non-handheld UE can be considered as starting point for requirements discussions:

For PC2, it is clear that 1Tx and 2Tx requirements would be defined. In addition, though not officially confirmed, those requirements would be applicable for both handheld and non-handheld which is already common in TN.

For PC1.5 and PC1, the condition is somewhat complicated, since multiple possibilities of implementation (1/2/3Tx) and different types are considered.

The following part has re-used some from [3] with update, especially on the summary tables, which did not have much discussion in last meeting.

Now it is suggested that the discussion can focus on what requirements should be considered and defined, considering that there is already common understanding in what the typical implementation for NR NTN could be, and handheld and non-handheld UE basically share many requirements except MPR.

Based on the previous analysis, we have updated the feasibility summary as following for handheld UE, and it can be seen duplexer poses more restriction compared to PA:

Observation 3: A feasibility analysis for handheld UE:

Table 1. Current restriction summary for handheld UE for NR NTN (With Update)

|  | PA | Duplexer | Conclusion based on PA&Duplexer |
| --- | --- | --- | --- |
| PC2 1Tx, PC1.5 2Tx | Feasible with sufficient margin | (Almost) feasible | [Feasible] |
| PC1.5 1Tx, PC1 2Tx | May feasible but insufficient margin | Not that feasible | [Not feasible] |
| PC1 1Tx | May not feasible | Not feasible | Not feasible. |
| PC1 3Tx (New) | May feasible | May feasible | FFS |

It should be noted that although PC1 3Tx was not considered before, it actually can be implemented by using three separate PC2 RF chain (30.77dBm), this implementation has the potential to using PC2 duplexer, thus may have some advantage in size and power consumption for a PC1 UE.

Based on the feasibility analysis and the already agreed WF and UE architecture, the following requirements applicability status proposal can be as following:

Observation 4:

Table 2: Current requirements applicability and feasibility (Uncertain part in [])

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | Non-handheld |
|  | [2Tx] | [Non-handheld] |
|  | [3Tx] | [TBD] |
| PC1.5 | [1Tx] | [Non-handheld] |
|  | [2Tx] | [Both] |
| PC2 | 1Tx | Both |
|  | 2Tx | Both |

Analysis and proposals

For us, PC1 2Tx is not that useful, since it can not use PC2 duplexer, and still need multiple large form factor duplexers. This implementation seems do not have enough advantages compared to 1Tx and 3Tx. Considering that there are already many cases, some simplifications are preferred.

For PC1.5, it is interesting to discuss whether 1Tx is needed or not. Since PC2 duplexer cannot be reused for PC1.5 1Tx, the need for this requirement seems would focus on non-handheld. Then, whether PC1 is better choice compared to PC1.5 may be arguable. We do not have strong views on this part. For 2Tx, we tend to define requirements, although there are still discussion on the feasibility. Considering the transmit power would be almost always welcome, and this seems the only option to use current components for TN.

In all, the following proposal is provided for requirements definition

Proposal 4: The following suggestions for requirements definition were made:

For PC1:

- 2Tx requirements need not be defined.

- 3Tx requirements can be further discussed, especially for handheld.

For PC1.5,

- 1Tx requirements can be further discussed for non-handheld; can be deprioritized

- 2Tx requirements can be defined.

Table 3: Proposed cases for requirements definition

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | Non-handheld |
|  | [2Tx](discuss) | [Non-handheld] |
|  | [3Tx] (discuss) | [TBD] (discuss) |
| PC1.5 | [1Tx] (discuss) | [Non-handheld] |
|  | [2Tx] (discuss) | [Both] (discuss) |
| PC2 | 1Tx | Both |
|  | 2Tx | Both |

Conclusion

In this paper, the following observations and proposals are provided:

RF Requirements

Observation 1a: No need to consider different nominal MOP for the same power class in NR NTN with TN.

Observation 1b: Reusing tolerance for different power class in TN for NTN is needed for basic reuse of the components.

Proposal 1: Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance for PC1 and PC1.5 for NR-NTN Non-Redcap HPUE.

Observation 2: Reconsider NTN regulatory requirement in SEM may involve considerable work, and the current A-MPR solution is already effective and flexible to consider the various regional regulatory requirements.

Proposal 2: Re-use existing SEM from PC3 to other PCs for NR-NTN.

Proposal 3: Discuss the MPR/A-MPR after SEM/ACLR is confirmed.

Feasibility and Requirements applicability

Observation 3: A feasibility analysis for handheld UE:

Table 1. Current restriction summary for handheld UE for NR NTN (With Update)

|  | PA | Duplexer | Conclusion based on PA&Duplexer |
| --- | --- | --- | --- |
| PC2 1Tx, PC1.5 2Tx | Feasible with sufficient margin | (Almost) feasible | [Feasible] |
| PC1.5 1Tx, PC1 2Tx | May feasible but insufficient margin | Not that feasible | [Not feasible] |
| PC1 1Tx | May not feasible | Not feasible | Not feasible. |
| PC1 3Tx (New) | May feasible | May feasible | FFS |

Observation 4:

Table 2: Current requirements applicability and feasibility (Uncertain part in [])

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | Non-handheld |
|  | [2Tx] | [Non-handheld] |
|  | [3Tx] | [TBD] |
| PC1.5 | [1Tx] | [Non-handheld] |
|  | [2Tx] | [Both] |
| PC2 | 1Tx | Both |
|  | 2Tx | Both |

Proposal 4: The following suggestions for requirements definition were made:

For PC1:

- 2Tx requirements need not be defined.

- 3Tx requirements can be further discussed, especially for handheld.

For PC1.5,

- 1Tx requirements can be further discussed for non-handheld; can be deprioritized

- 2Tx requirements can be defined.

Table 3: Proposed cases for requirements definition

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | Non-handheld |
|  | [2Tx](discuss) | [Non-handheld] |
|  | [3Tx] (discuss) | [TBD] (discuss) |
| PC1.5 | [1Tx] (discuss) | [Non-handheld] |
|  | [2Tx] (discuss) | [Both] (discuss) |
| PC2 | 1Tx | Both |
|  | 2Tx | Both |

- References

[1] R4-2418148, Topic summary for [113][126] NR_IoT_NTN_HPUE_part1, Moderator (Samsung), RAN4#113

[2] R4-2420403, WF on NR NTN HPUE, Samsung, RAN4#113

[3] R4-2418200, NR NTN HPUE Tx Requirements and feasibility analysis, vivo, RAN4#113