3GPP TSG-RAN WG4 Meeting # 114	R4-2500763

Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	NR NTN HPUE Rx Requirements

Agenda Item:	7.10.2.2.2

Document for:	Approval

Introduction

In the previous meeting, there are some discussions for NR-NTN RX and some progress as in [2]:

For PC2 on n254: [0] dB RSD for PC2 NR-NTN for 1Tx and 2Tx

For PC2 on n255: Further discuss the follow assumptions

- Option 1: Duplex isolation as 50~52; PA interference as -107 dBm/Hz (P2/R4-2419073)

- Option 2: Duplex isolation as 51; PA interference as -117 dBm/Hz (O1/R4-2419540)

For PC2 on n256: [0] dB RSD for PC2 NR-NTN for 1Tx and 2Tx as starting point

- To be confirmed by further discuss the following assumptions:

  - Option 1: Duplex isolation as 50~52; PA interference as -117.8 dBm/Hz

  - Option 2: Duplex isolation as 51; PA interference as -124 dBm/Hz

- Encourage companies to investigate and discuss the impact of Rx non-linearity in RSD evaluations.

For PC1.5 and PC1, there are not too much progress, since there are still a lot of open issues on the reference architecture and requirements applicability.

In this paper, a continual discussion of HPUE to RSD analysis for NR NTN are provided. Among them, some of the proposals and analysis in [3] were re-submitted with certain update.

HPUE Impact for NR-NTN RSD

General

Similar to [3] which submitted in last meeting, it should be noted that the scenarios needed to be defined for RSD is related to what Tx requirements would be defined, and here is a table from [4] in Tx part:

Table 1: Current requirements applicability and feasibility (Uncertain part in [])

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | Non-handheld |
|  | [2Tx] | [Non-handheld] |
|  | [3Tx] | [TBD] |
| PC1.5 | [1Tx] | [Non-handheld] |
|  | [2Tx] | [Both] |
| PC2 | 1Tx | Both |
|  | 2Tx | Both |

Observation 1: RSD definition would depend on Tx scenarios including power class, antenna numbers and UE type (handheld and non-handheld).

It can be seen that there are still a lot of uncertain scenarios.

Currently PC2 is under discussing with priority and the current WF is focusing on this power class, while others are not agreed yet.

PC2

As already been agreed in previous WF, the RSD is [0] for n254 and [0] as starting point for n256.

For n256, there were already detailed analysis in [5], that the frequency separation for n256 is the same to n1, and the gap between UL/DL is even larger. The condition of n256 is not worse than n1 for duplexer implementation and the frequency range is also quite close. Based on this condition, the RSD for n256 can be [0]dB as n1.

For n255, the range is same to n24, with less restriction. Currently the n24 do not have a MSD definition. The data sheet in [6] also show a more than 50dB Tx-Rx isolation in n255. Although this component may targeted for PC3, we believe it would not be difficult to adapt to PC2 need. Further MSD can be discussed for n255.

Proposal 2a: For NR-NTN power class 2,

- For n256, RSD = 0dB, for both 2Tx and 1Tx cases.

- For n255, RSD is FFS

As already been suggested in [3][5], power class 2 is confirmed to be feasible for both handheld and non-handheld UE. Currently there is no differentiation of handheld / non-handheld UE for the RSD requirements, and the understanding is these requirements are based on common components and architectures.

Proposal 2b: For power class 2, there is no differentiation of handheld and non-handheld for RSD.

PC1.5

As discussed, the PC1.5 definition are still under discussion. However, some preliminary thoughts can also be discussed. The following observation and proposals are re-submitted with update:

If PC1.5 for 2Tx could be agreed to be defined, then it should not be too difficult for PC1.5 2Tx RSD derivation, since its similarity to PC2 2Tx in the derivation process.

Observation 2: Derivation of PC1.5 2Tx RSD can be similar to PC2 2Tx, if defined.

Furthermore, it should be noted the feasibility study of PC1.5 2Tx for NR NTN is based on handheld UE, thus the requirements should also be based on handheld UE. According to tradition and typical implementation, even the non-handheld type can share the same requirements with handheld ones as long as handheld is feasible.

Proposal 3: For PC1.5, 2Tx RSD are the same for handheld and non-handheld, while 1Tx RSD is likely to consider non-handheld only if defined.

PC1

It is noted that there are already a number of PC1 requirements defined in NR and LTE, and all of them are 1Tx architecture and for non-handheld type. This is easy to be understand since the duplexer that could be working in the PC1 is beyond what a handheld UE could use.

According to the discussion in [3], the following observation is resubmitted:

Observation 3: Currently PC1 is only for 1Tx and non-handheld case in LTE and NR TN, and typically share PC3 Rx requirements without any RSD.

According to the current agreement, PC1 1Tx for NR-NTN is also proposed to be non-handheld only.

Observation 4: For PC1 1Tx, according to de-sense analysis in 37.880, it appears that the current NTN band n254/n255/n256 are easier compared to legacy PC1 bands.

Proposal 4a: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB.

Proposal 4b: FFS for PC1 with 2 or 3 Tx cases.

Conclusion

In this paper, the RSD requirements have been discussed for NR NTN HPUE.

General

Observation 1: RSD definition would depend on Tx scenarios including power class, antenna numbers and UE type (handheld and non-handheld).

Table 1: Current requirements applicability and feasibility (Uncertain part in [])

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | Non-handheld |
|  | [2Tx] | [Non-handheld] |
|  | [3Tx] | [TBD] |
| PC1.5 | [1Tx] | [Non-handheld] |
|  | [2Tx] | [Both] |
| PC2 | 1Tx | Both |
|  | 2Tx | Both |

PC2

Proposal 2a: For NR-NTN power class 2,

- For n256, RSD = 0dB, for both 2Tx and 1Tx cases.

- For n255, RSD is FFS

Proposal 2b: For power class 2, there is no differentiation of handheld and non-handheld UE.

PC1.5

Observation 2: Derivation of PC1.5 2Tx RSD can be similar to PC2 2Tx, if defined

Proposal 3: For PC1.5, 2Tx RSD are the same for handheld and non-handheld, while 1Tx RSD is likely to consider non-handheld only if defined.

PC1

Observation 3: Currently PC1 is only for non-handheld case in LTE and NR TN, and typically share PC3 Rx requirements without any RSD.

Observation 4: For PC1, according to de-sense analysis in 37.880, it appears that the current NTN band n254/n255/n256 are easier compared to legacy PC1 bands.

Proposal 4a: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB.

Proposal 4b: FFS for PC1 with 2 or 3 Tx cases.

- References

[1] R4-2418148, Topic summary for [113][126] NR_IoT_NTN_HPUE_part1, Moderator (Samsung), RAN4#113

[2] R4-2420403, WF on NR NTN HPUE, Samsung, RAN4#113

[3] R4-2418201, NR NTN HPUE Rx Requirements, vivo, RAN4#113

[4] R4-2500762, NR NTN HPUE Tx Requirements and feasibility analysis, vivo, RAN4#114

[5] R4-2415789, Analysis for NR NTN HPUE Rx Requirements, vivo, RAN4#112bis

[6] https://www.murata.com/products/productdata/8820841873438/DS-SAYRZ1G54BA0C0A.pdf