3GPP TSG-RAN WG4 Meeting # 114	R4-2500767

Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Further discussion of UE RF impact for NR MIMO phase 5

Agenda Item:	7.22.2

Document for:	Approval

Introduction

In RAN#102, a new WID [1] was approved on NR MIMO Phase 5. There were already three RAN1 meetings and significant progress have been made, and this is the first meeting for RAN4 to discuss this.

In RAN4#112, an initial analysis and study [2] were made for the WI, and a WF [3] has been agreed. In RAN#105, the NR MIMO phase 5 WID has been revised to [4] to add some new scope.

In RAN4#112bis, the 3Tx issues and mTRP issues were discussed and document summary is in [6], a WF is agreed in [7] and an LS was sent out in [8].

In RAN4#113, further discussion was made. A discussion paper was submitted in [10]. The overall summary was in [11] and an WF was reached in [12]. In addition, an reply LS was received after the meeting in [13], in which the answers from RAN1 were included.

In this paper, further discussion was provided based on last meeting’s WF and reply LS.

Discussion

3Tx RF Impact

Power classes / UE types

In the agreed WF [12], the following agreements have been made.

<Agreement>: PC for 3Tx requirement

- 3Tx PC1.5 cannot be supported for full power mode 0 in Rel-19

<Way forward>: FFS on PC1.5 for power scaling mode (3-layer) with its applicable UE type.

This is pretty much considered the reply LS [13] in advance, in which the following contents replied:

Question 2: RAN4 would like to check with RAN1 whether a 3Tx UE supporting UL full power transmission mode 0 needs to achieve the same maximum output power with both 1Layer and 2Layers.

Reply for Question 2: Yes, a 3Tx UE supporting UL full power transmission mode 0, delivers the same maximum output power with both 1- and 2-layer precoders.

Question 3: RAN4 would like to check with RAN1 whether 3Tx UE can support UL full power transmission mode 0 with 2Layer 3-port codebooks but not support 1Layer 3-port codebooks.

Reply for Question 3: No, a 3Tx UE supporting UL full power transmission mode 0, shall support full power with both 1- and 2-layer 3-port codebook.

Based on the reply, it is further confirmed that if a 3Tx UE declare a certain power class, it has to support 1/2/3 layer 3-port codebook, and has to achieve full power in all cases. It is aligned with original understanding that it is mandatory for all the RF chains to support the full power indicated by the power class.

In this sense, the agreement in last meeting that PC1.5 can not be applied 3Tx full power mode 0 is reasonable. However, there is another key limiting factor is that transmit diversity is not considered. According to previous discussion and what is documented in [7]:

- RAN4 understanding is that TxD for 3Tx is not in the scope of this WI

However, there is some constant questions and concerns on this point. In addition, there is also questions on whether 2Tx TxD can be considered in 3Tx, if 3Tx TxD is not in the scope. The answer to this question would have an impact on the the following FFS bullet:

FFS on PC1.5 for power scaling mode (3-layer) with its applicable UE type.

If 2Tx TxD is allowed, then PC1.5 is still possible to be achieved based on the current understanding of the RF chain capability. If 2Tx TxD is not allowed, then PC1.5 would be difficult to be achieved for 3Tx UE. In our understanding, this kind of restriction may be a bit too stringent. If a 3Tx can not have PC1.5, then this might be a big restriction, and there may have very little, if any, interest for this type of device for interested parties.

However, it is admitted that with this allowance of 2Tx TxD, it might be complicated to do verification. E.g.

- UE has to declare which of the two antennas among the 3 RF chains.

- If multiple RF chains can be applied to 2Tx TxD, should all of them be verified.

Based on this condition, the following proposal is provided:

Proposal 1: Consider to allow 2Tx TxD to enable PC1.5 for 3Tx UE, if the verification complexity can be controlled.

Actually, there are other FFS issues in the MPR part that is also related to previous discussion:

FFS on whether PC2 MPR with 1Tx or 2Tx will be applied

FFS on PC1.5 MPR to use the same MPR as PC1.5 MPR with 2Tx. Subject to Tx Diversity support

Whether 2Tx TxD can be considered or not clearly have a direct impact on these two issues. Thus make it further necessary to discuss this issue.

In addition, in [10], there was an argument that even if PC1.5 is supported, we tend to let PC1.5 only support FWA UE type unless clear needs can be clarified otherwise, since multiple high power UE RF chains is challenging.

Similarly, for 3Tx PC2 UE with full power mode 0 supported, since 26dBm *3 RF chains are assumed. It is also suggested to be restricted to FWA type. Although 3*26dBm is also being considered as an implementation in NR-NTN, it should be noted that that case is for even higher power class (PC1) and NTN is much more eager for higher power compared to TN.

Proposal 2: Consider to restrict the 3Tx UE type to FWA only for

- PC2 full power mode 0

- PC1.5 if defined

Asymmetric DL sTRP/UL mTRP

In last meeting, this issue was further discussed. A draft CR in [14] was provided for information, with the following BC requirements added:

6.6K 	Beam correspondence for [UEs with different TCI states for UL and DL]

For a UE that supports [capability_indication] and is scheduled with different TCI states for DL and UL respectively, the requirements of clause 6.6 apply. The side conditions specified in clause 6.6 apply to SSB transmissions associated with each of the two TCL states, UL and DL, as well as all signals that maintain QCL type D relation with those SSBs, where applicable.

In the reply LS [13], the following reply was provided by RAN1:

Question 1:  RAN4 would like to inquire if the UL TRP(s) can transmit SSBs per the scope of the WID.

Reply for Question 1: From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s).

It can be seen that Rel-19 include the case that there exists that UE do transmission and reception in different TRPs. However, it should be also noted that the tentative requirements currently discussed, is actually a basic m-TRP issue which is not started from and not restricted to Rel-19. The BC requirements can be a starting point, but more discussions may still be needed, and more scenarios and testing method need to be discussed. This seems more suitable to be in a new WI rather than Rel-19 MIMO.

Proposal 3: Consider the m-TRP verification in next release.

Conclusion

In this paper, more study on UE RF impact of MIMO phase 5 is provided:

3Tx RF Impact

Proposal 1: Consider to allow 2Tx TxD to enable PC1.5 for 3Tx UE, if the verification complexity can be controlled.

Proposal 2: Consider to restrict the 3Tx UE type to FWA only for

- PC2 full power mode 0

- PC1.5 if defined

Asymmetric DL sTRP/UL mTRP

Proposal 3: Consider the m-TRP verification in next release.

- References

[1] RP-234007, New WID: NR MIMO Phase 5, Samsung (Moderator), RAN#102

[2] R4-2412095, Discussion of UE RF impact for NR MIMO phase 5, vivo, RAN4#112

[3] R4-2414310, WF on UE RF requirements for Rel-19 MIMO enhancement, Samsung, RAN4#112

[4] RP-242394, WID revision: NR MIMO Phase 5, RAN#105

[5] R4-2413224, On asymmetric connection scenarios for NR MIMO Phase 5, Qualcomm, RAN4#112

[6] R4-2415232, Topic summary for [112bis][128] NR_MIMO_Ph5_UE, Samsung, RAN4#112bis

[7]R4-2417116, WF on UE RF requirements for MIMO enhancement, Samsung, RAN4#112bis

[8] R4-2417117, LS on UE RF issues for Rel-19 MIMO enhancement, Huawei, RAN4#112bis

[9] R4-2415792, Further discussion of UE RF impact for NR MIMO phase 5, vivo, RAN4#112bis

[10] R4-2418204, Further discussion of UE RF impact for NR MIMO phase 5, vivo, RAN4#113

[11] R4-2418153, Topic summary for [113][131] NR_MIMO_Ph5_UE, Moderator (Samsung), RAN4#113

[12] R4-2420400, WF on UE RF requirements for MIMO enhancement, Samsung, RAN4#113

[13] R4-2500005, Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement, RAN1#119

[14] R4-2417632, dCR on RF requirements for a UE with different TCI states for UL and DL, Qualcomm, RAN4#113