3GPP TSG-RAN WG4 Meeting # 114	R4- 2500775

Athens, GR, 17th – 21st February, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements of UE-initiated/event-driven beam management

Agenda item:	7.22.3.1

Document for:	Discussion

Introduction

In agreed WID for Rel-19 MIMO operation [1], following objective was captured:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In this contribution, we provide our views on RRM core requirements for UE-initiated/event-driven beam reproting

Discussion

In the last RAN4#113, following has been agreed:

Issue 1-1: How to define Event triggered measurement reporting delay? < Agreement> - Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air. - Ending point is the point when the UE transmits a first PUCCH triggered by the event. Issue 1-3: Whether RAN4 to specify requirements for case 1 or cover both case1 and case2? Case 1: no time window based Case 2: with time window based, M is configurable Agreement: It is RAN4 common understanding that no additional filtering applies for case 2 compared to case 1, if RAN4 will define requirement for case 2. Issue 1-2: How to define measurement period (evaluation time / measurement time) in Event triggered measurement reporting delay? < Agreement> Agreement: For specifying the RAN4 delay requirement for event 2 - Measure both the current beam and new beam after the starting point, FFS the details. - Note: this does not imply to define the delay requirement based on the sum of the current beam measurement period and new beam measurement period, which is pending on further RAN1 discussion. - Start the RAN4 RRM discussion for FR2. - Send LS to RAN1 to ask whether FR1 is covered as well. FFS the exact wording in the LS. Agreement(RAN1#119) Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC). - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s) - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS. - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS. - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s). - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}. - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s). Note: There is the same periodicity for the new beam RS(s). FFS: Down-selection among above Alt(s) for cross-CC beam reporting.

Reporting delay requirements

The one of main open issue is specifing reporting delay requirement. For the delay requirement RAN4 defined the start point as reusing L3 event triggered reporting delay requirement, which is “an event that will trigger a measurement report”. If Event-2 is applied, the event means comparing L1-RSRP of current and new beams. In pervious meeting, RAN4 agreed to measure both the current beam and new beam after the starting point. When comparing current beam and new beam, periodicity of the RSs configured for the current beam and new beam can be different. Although RAN1 defiend that the periodicity of current beam and new beam are same in the same CC, still different periodicity can be configured, if current beam and new beam are in different CC. In order to compare the RSs with different periodicity, measurement periodicity needs to be the longer periodicity as baseline.

Once the event is triggered, the UE reports the result of L1-RSRSP at the available UL resource, which depends on the configuration. Thus, we think UL uncertainty can be included in the delay requirement.

Proposal 1: RAN4 to specify reporting delay requirement as follows:

- Reporting delay = Measurement time + UL uncertainty

  - Measurement time is longer periodicity of the current beam and new beam

Figure 1 Reporting delay requirement

Whether to update L1 report which is measured in L1 report?

When the event is triggered, UE is required to report at the available UL resource. Given the fact that there can be one or more measurement samples can be acquired before reporting, it is not clear whether L1 reporting can be updated based on the further measurements. We think after the event is triggered, further measurements are not required, and no need to update the reporting. In our understanding, it is discussed in RAN1, so it can be further discussed based on RAN1 progress.

Proposal 2: Whether to update L1 report which is measured in L1 report needs to wait RAN1 progress

Whether to define new accuracy requirements

<Way forward > - Option 1: (Qualcomm, Nokia, Xiaomi, Apple, MediaTek) - The existing accuracy requirements for L1 measurements can be applied. Not to define new accuracy requirements. - Option 2: (Samsung) - Focus on core requirements firstly, FFS on accuracy requirements. - Option 3: (CMCC) - For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline. - For CSI-RS based beam report, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements taking neighbour cell measurement into account. - Option 4: (ZTE) - Discuss the accuracy for the two trigger-event solutions respectively. - For the basic trigger-event detection, tight the existing L1-RSRP accuracy requirement; - For the optional trigger-event detection, resue the existing L1-RSRP accuracy requirement.

For UE-initiated/event-driven beam reporting, at least L1-RSRP is used for measurement and reporting. Reusing existing accuracy requriements for L1 measurement seems enough. No need to define new accuracy requirements.

Proposal 3: No need to define new accuracy requirements

known/unknown conditions for TCI states for TCI states switching

Issue 1-8: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM <Way forward > - Option 1: (Qualcomm, Nokia, Samsung, Apple, Huawei) - Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM - Option 2: (vivo) - Regarding the impact to know conditions of TCI state activation from event-triggered L1 reporting, RAN4 waits further RAN1/2 conclusions. - Option 3: (ZTE) - Need to define the condition of known cell for UEIBM. The definition in ICBM can be starting point.

Known conditions for TCI state depends on reporting and measurement before TCI state switch command, which is not related to NW or UE-initiated beam reporting. We think legacy known conditions can be applied for UE-initiated/event-driven beam reporting.

Proposal 4: Reuse legacy known/unknown conditions for TCI-switcing based on UE-initiated/event-driven beam reporting

Whether to specify new unified TCI switching delay requirements?

<Way forward > - Option 1: (Qualcomm, Nokia, Samsung (For known conditions for TCI states), Apple, ZTE, Ericsson) - The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management. - Option 2: (Samsung) - No unknown case is applied if the TCI states switching is based on UEIBM. - Option 3: (ZTE) - Need to decide whether to define L1-RSRP requirements for unknown cell Agreement Study the following to reduce beam application latency after a UEI/ED beam report is sent - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report - update TCI state(s) with the reported new beam(s) - activate new beam(s) without additional SSB reception - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized) - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception - Other alternatives are not precluded

In the last RAN1#119 meeting, reducing beam application latency after a UE-initiated/event-driven beam reporting was discussed. The issue is to whether to reduce activated TCI state switching delay by removing additional SSB reception (Tfirst-SSB) for reducing the unified TCI state beam application time. In our understanding, Tfirst-SSB is for AGC tunning before receiving PDSCH/PDCCH, if target TCI state is not in the active TCI state list. However, we are not sure whether RAN1 consider those perspective. We think RAN4 needs to discuss whether active TCI state swiching delay can be reduced or not.

Proposal 5: For unified TCI switching delay requirements, RAN4 needs to discuss whether active TCI state switching delay can be reduced or not

Conclusion

In this contribution, we provide views on RRM requireiments for UE-initiated/event-driven beam reproting, and we propose

Proposal 1: RAN4 to specify reporting delay requirement as follows:

- Reporting delay = Measurement time + UL uncertainty

  - Measurement time is longer periodicity of the current beam and new beam

Proposal 2: Whether to update L1 report which is measured in L1 report needs to wait RAN1 progress

Proposal 3: No need to define new accuracy requirements

Proposal 4: Reuse legacy known/unknown conditions for TCI-switcing based on UE-initiated/event-driven beam reporting

Proposal 5: For unified TCI switching delay requirements, RAN4 needs to discuss whether active TCI state switching delay can be reduced or not

Reference

[1] RP-242394, “WID revision: NR MIMO Phase 5,” Samsung